Greenhouse Gas Reporting Program

XML Reporting Instructions

for Subpart W -

Petroleum and Natural Gas Systems

United States Environmental Protection Agency

Climate Change Division

Washington, DC

February 25, 2015

These instructions explain how to report the required data for the applicable regulations. Owners and operators of units should refer to the applicable regulations for information about what data are required to be reported.

On October 24, 2014, EPA published a rule that finalizes amendments to reporting and recordkeeping requirements and an alternative electronic verification approach for the Greenhouse Gas Reporting Program. (See http://www.epa.gov/climatechange/emissions/notices.html for more information). These changes address concerns that public disclosure of certain data elements that are inputs to the equations used to calculate emissions would reveal sensitive business information, and maintain EPA’s ability to verify emissions and ensure compliance.

[This page intentionally left blank]

# Table of Contents # Page

I. Introduction 1 II. Summary of Changes 6 III. Subpart W Overview 15 1.0 Subpart W Total Emissions 22 2.0 Subpart W Summary Details 26 2.1 Offshore petroleum and natural gas production 36 2.2 Onshore petroleum and natural gas production 36 2.3 Onshore natural gas processing 37 2.4 Onshore natural gas transmission compression 37 2.5 Underground natural gas storage 38 2.6 Liquefied natural gas (LNG) storage 38 2.7 LNG import and export equipment 38 2.8 Natural gas distribution 39 3.0 Sub-Basin Selection / Onshore Requirements Under 98.236(e) 40 3.1 Basin Identification 40 3.2 Sub-Basin Identification 42 4.0 Natural Gas Pneumatic Device Venting 46 5.0 Natural Gas Driven Pneumatic Pumps 52 6.0 Acid Gas Removal Units 57 7.0 Dehydrators 64 8.0 Well Venting for Liquids Unloading 79 9.0 Gas Well Completions and Workovers 94 10.0 Blowdown Vent Stacks 109 11.0 Gas from Produced Oil Sent to Atmospheric Tanks 117 12.0 Transmission Tanks 139 13.0 Well Testing Venting and Flaring 146 14.0 Associated Gas Venting and Flaring 153 15.0 Flare Stacks 160 16.0 Centrifugal Compressors 167 17.0 Reciprocating Compressors 186 18.0 Other Emissions from Equipment Leaks Estimated Using Emission Factors 207 19.0 Local Distribution Companies 222 20.0 Enhanced Oil Recovery (EOR) Injection Pump Blowdown 232 21.0 Enhanced Oil Recovery (EOR) Hydrocarbon Liquids Dissolved CO2 237 22.0 Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions 241 23.0 Offshore Sources [98.236(b)] 260 24.0 Facility-Level Roll-up Emissions 267 IV. Appendix A - Sample XML Document for Subpart W 269

# List of Tables # Page

Table 1 Reporting Numbers 2 Table 2 Summary of Changes to the Schema for Subpart W 6 Table 3 Greenhouse Gas Information Details Data Element Definitions 25 Table 4 Subpart W Summary Details Data Element Definitions 28 Table 5 Subpart W Source Reporting Details Data Element Definitions 32 Table 6 Required Sources to Report by Industry Segment 35 Table 7 Basin Identification Data Element Definitions 41 Table 8 Sub-Basin Identification / Onshore Requirements Details Data Element Definitions 44 Table 9 Pneumatic Device Venting Details Data Element Definitions 47 Table 10 Pneumatic Device Types Details Data Element Definitions 50 Table 11 Natural Gas Driven Pneumatic Pumps Details Data Element Definitions 53 Table 12 Pneumatic Pump Types Details Data Element Definitions 55 Table 13 Acid Gas Removal Units Details Data Element Definitions 58 Table 14 Acid Gas Removal Number of Units Details Data Element Definitions 61 Table 15 Dehydrators Details Data Element Definitions 66 Table 16 Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details Data Element Definitions 70 Table 17 Absorbent Desiccant Dehydrators Details Data Element Definitions 73 Table 18 Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Data Element Definitions 75 Table 19 Well Venting Details Data Element Definitions 81 Table 20 Well Venting Calculation Methodology 1 Details Data Element Definitions 86 Table 21 Well Venting Calculation Methodology 2 Details Data Element Definitions 90 Table 22 Well Venting Calculation Methodology 3 Details Data Element Definitions 92 Table 23 Gas Well Completions and Workovers Details Data Element Definitions 96 Table 24 Gas Well Completions and Workovers with Hydraulic Fracturing Data Element Definitions 101 Table 25 Gas Well Completions and Workovers without Hydraulic Fracturing Data Element Definitions 107 Table 26 Blowdown Vent Stacks Details Data Element Definitions 111 Table 27 Blowdown Vent Stacks for Each Physical Volume Details Data Element Definitions 115 Table 28 Atmospheric Tanks Details Data Element Definitions 119 Table 29 Atmospheric Tanks Calculation Methodology 1 or 2 Details Data Element Definitions 126 Table 30 Atmospheric Tanks Calculation Methodology 3 or 4 Details Data Element Definitions 131 Table 31 Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details Data Element Definitions 135 Table 32 Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details Data Element Definitions 137 Table 33 Transmission Tanks Details Data Element Definitions 140 Table 34 Transmission Storage Tanks Details Data Element Definitions 144 Table 35 Well Testing Details Data Element Definitions 147 Table 36 Well Testing Basin Details Data Element Definitions 151 Table 37 Associated Gas Venting and Flaring Details Data Element Definitions 154 Table 38 Associated Gas Venting and Flaring Basins Details Data Element Definitions 158 Table 39 Flare Stack Details Data Element Definitions 161 Table 40 Unique Flare Stacks Details Data Element Definitions 164 Table 41 Centrifugal Compressors Details Data Element Definitions 169 Table 42 Centrifugal Compressors Modes Details Data Element Definitions 178 Table 43 Reciprocating Compressors Details Data Element Definitions 188 Table 44 Reciprocating Compressors Modes Details Data Element Definitions 197 Table 45 Other Emissions From Equipment Leaks Details Data Element Definitions 209 Table 46 Estimating Emissions Found in Leak Survey Details Data Element Definitions 213 Table 47 Estimating Emissions Using Population Counts Details Data Element Definitions 217 Table 48 Component Count Methodology One Details Data Element Definitions 220 Table 49 Local Distribution Companies Details (Part 1 of 3) Data Element Definitions 223 Table 50 Local Distribution Companies Details (Part 2 of 3) Data Element Definitions 226 Table 51 Local Distribution Companies Details (Part 3 of 3) Data Element Definitions 230 Table 52 EOR Injection Pump Blowdown Details Data Element Definitions 233 Table 53 Unique Injection Pump Blowdowns Details Data Element Definitions 235 Table 54 EOR Hydrocarbon Liquids Dissolved CO2 Details Data Element Definitions 238 Table 55 EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details Data Element Definitions 240 Table 56 Combustion Emissions Details Data Element Definitions 243 Table 57 External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details Data Element Definitions 246 Table 58 External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details Data Element Definitions 249 Table 59 Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details Data Element Definitions 251 Table 60 Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details Data Element Definitions 252 Table 61 External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details Data Element Definitions 254 Table 62 Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details Data Element Definitions 257 Table 63 Offshore Emissions Details Data Element Definitions 261 Table 64 Offshore Emissions Sources Details Data Element Definitions 263 Table 65 Facility-Level Roll-up Emissions Data Element Definitions 268

# List of Figures # Page

Figure 1 Example of an XML Tree 3 Figure 2 “Unbounded” Symbol in Schema Diagram 5 Figure 3 Subpart W Reporting Diagram 16 Figure 4 Subpart W Schema Diagram 21 Figure 5 Greenhouse Gas Information Details Schema Diagram 22 Figure 6 Subpart W Reporting Details Schema Diagram 26 Figure 7 Subpart W Summary Details Schema Diagram 27 Figure 8 Subpart W Source Reporting Details Schema Diagram 30 Figure 9 Basin Identification Schema Diagram 40 Figure 10 Sub-Basin Identification / Onshore Requirements Details Schema Diagram 43 Figure 11 Pneumatic Device Venting Details Schema Diagram 46 Figure 12 Pneumatic Device Types Details Schema Diagram 49 Figure 13 Natural Gas Driven Pneumatic Pumps Details Schema Diagram 52 Figure 14 Pneumatic Device Types Details Schema Diagram 55 Figure 15 Acid Gas Removal Units Details Schema Diagram 57 Figure 16 Acid Gas Removal Number of Units Details Schema Diagram 60 Figure 17 Dehydrators Details Schema Diagram 64 Figure 18 Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details Schema Diagram 69 Figure 19 Absorbent Desiccant Dehydrators Details Schema Diagram 72 Figure 20 Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Schema Diagram 74 Figure 21 Well Venting Details Schema Diagram 79 Figure 22 Well Venting Calculation Methodology 1 Details Schema Diagram 84 Figure 23 Well Venting Calculation Methodology 2 Details Schema Diagram 89 Figure 24 Well Venting Calculation Methodology 3 Details Schema Diagram 91 Figure 25 Gas Well Completions and Workovers Details Schema Diagram 94 Figure 26 Gas Well Completions and Workovers with Hydraulic Fracturing Schema Diagram (Part 1 of 2) 98 Figure 27 Gas Well Completions and Workovers with Hydraulic Fracturing Schema Diagram (Part 2 of 2) 99 Figure 28 Gas Well Completions and Workovers without Hydraulic Fracturing Schema Diagram 106 Figure 29 Blowdown Vent Stacks Details Schema Diagram 109 Figure 30 Blowdown Vent Stacks for Each Physical Volume Details Schema Diagram 114 Figure 31 Atmospheric Tanks Details Schema Diagram 117 Figure 32 Atmospheric Tanks Calculation Methodology 1 or 2 Details Schema Diagram (Part 1 of 2) 123 Figure 33 Atmospheric Tanks Calculation Methodology 1 or 2 Details Schema Diagram (Part 2 of 2) 124 Figure 34 Atmospheric Tanks Calculation Methodology 3 or 4 Details Schema Diagram 129 Figure 35 Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details Schema Diagram 134 Figure 36 Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details 137 Figure 37 Transmission Tanks Details Schema Diagram 139 Figure 38 Transmission Storage Tanks Details Schema Diagram 143 Figure 39 Well Testing Details Schema Diagram 146 Figure 40 Well Testing Basin Details Schema Diagram 150 Figure 41 Associated Gas Venting and Flaring Details Schema Diagram 153 Figure 42 Associated Gas Venting and Flaring Basins Details Schema Diagram 157 Figure 43 Flare Stack Details Schema Diagram 160 Figure 44 Unique Flare Stacks Details Schema Diagram 163 Figure 45 Centrifugal Compressors Details Schema Diagram 167 Figure 46 Centrifugal Compressors Modes Details (Part 1 of 3) Schema Diagram 172 Figure 47 Centrifugal Compressors Modes Details (Part 2 of 3) Schema Diagram 174 Figure 48 Centrifugal Compressors Modes Details (Part 3 of 3) Schema Diagram 176 Figure 49 Reciprocating Compressors Details Schema Diagram 186 Figure 50 Reciprocating Compressors Modes Details (Part 1 of 3) Schema Diagram 191 Figure 51 Reciprocating Compressors Modes Details (Part 2 of 3) Schema Diagram 193 Figure 52 Reciprocating Compressors Modes Details (Part 3 of 3) Schema Diagram 195 Figure 53 Other Emissions From Equipment Leaks Details Schema Diagram 207 Figure 54 Estimating Emissions Found in Leak Survey Details Schema Diagram 212 Figure 55 Estimating Emissions Using Population Counts Details Schema Diagram 217 Figure 56 Component Count Methodology One Details Schema Diagram 220 Figure 57 Local Distribution Companies Details (Part 1 of 3) Schema Diagram 222 Figure 58 Local Distribution Companies Details (Part 2 of 3) Schema Diagram 225 Figure 59 Local Distribution Companies Details (Part 3 of 3) Schema Diagram 229 Figure 60 EOR Injection Pump Blowdown Details Schema Diagram 232 Figure 61 Unique Injection Pump Blowdowns Details Schema Diagram 234 Figure 62 EOR Hydrocarbon Liquids Dissolved CO2 Details Schema Diagram 237 Figure 63 EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details Schema Diagram 239 Figure 64 Combustion Emissions Details Schema Diagram 241 Figure 65 External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details Schema Diagram 246 Figure 66 External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details Schema Diagram 248 Figure 67 Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details Schema Diagram 250 Figure 68 Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details Schema Diagram 252 Figure 69 External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details Schema Diagram 254 Figure 70 Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details Schema Diagram 257 Figure 71 Offshore Emissions Details Schema Diagram 260 Figure 72 Offshore Emissions Sources Details Schema Diagram 262 Figure 73 Facility-Level Roll-up Emissions Schema Diagram 267

# List of XML Excerpts # Page

XML Excerpt 1 Example for “Unbounded” Parent Element 5 XML Excerpt 2 Example for Greenhouse Gas Information Details 25 XML Excerpt 3 Example for Subpart W Summary Details 30 XML Excerpt 4 Example for Subpart W Source Reporting Details 34 XML Excerpt 5 Example for Basin Identification 41 XML Excerpt 6 Example for Sub-Basin Identification / Onshore Requirements Details 45 XML Excerpt 7 Example for Pneumatic Device Venting Details 49 XML Excerpt 8 Example for Pneumatic Device Types Details 51 XML Excerpt 9 Example for Natural Gas Pneumatic Pumps Types Details 54 XML Excerpt 10 Example for Pneumatic Pumps Types Details 56 XML Excerpt 11 Example for Acid Gas Removal Units Details 59 XML Excerpt 12 Example for Acid Gas Removal Number of Units Details 63 XML Excerpt 13 Example for Dehydrators Details 67 XML Excerpt 14 Example for Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details 71 XML Excerpt 15 Example for Absorbent Desiccant Dehydrators Details 73 XML Excerpt 16 Example for Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Data 78 XML Excerpt 17 Example for Well Venting Details 83 XML Excerpt 18 Example for Well Venting Calculation Methodology 1 Details 88 XML Excerpt 19 Example for Well Venting Calculation Methodology 2 Details 91 XML Excerpt 20 Example for Well Venting Calculation Methodology 3 Details 93 XML Excerpt 21 Example for Gas Well Completions and Workovers Details 97 XML Excerpt 22 Example for Gas Well Completions and Workovers with Hydraulic Fracturing Details 105 XML Excerpt 23 Example for Gas Well Completions and Workovers without Hydraulic Fracturing 108 XML Excerpt 24 Example for Blowdown Vent Stacks Details 112 XML Excerpt 25 Example for Blowdown Vent Stacks for Each Physical Volume Details 115 XML Excerpt 26 Example for Atmospheric Tanks Details 122 XML Excerpt 27 Example for Atmospheric Tanks Calculation Methodology 1 or 2 Details 128 XML Excerpt 28 Example for Atmospheric Tanks Calculation Methodology 3 or 4 Details 133 XML Excerpt 29 Example for Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details 136 XML Excerpt 30 Example for Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details 138 XML Excerpt 31 Example for Transmission Tanks Details 142 XML Excerpt 32 Example for Transmission Storage Tanks Details 145 XML Excerpt 33 Example for Well Testing Details 149 XML Excerpt 34 Example for Well Basin Testing Details 152 XML Excerpt 35 Example for Associated Gas Venting and Flaring Details 156 XML Excerpt 36 Example for Associated Gas Venting and Flaring Basins Details 159 XML Excerpt 37 Example for Flare Stack Details 162 XML Excerpt 38 Example for Unique Flare Stacks Details 166 XML Excerpt 39 Example for Centrifugal Compressors Details 171 XML Excerpt 40 Example for Centrifugal Compressors Details for Onshore Petroleum and Natural Gas Production 171 XML Excerpt 41 Example for a Centrifugal Compressor With Wet Seals in Operating Mode 184 XML Excerpt 42 Example for a Centrifugal Compressors with Dry Seals in Operating Mode 185 XML Excerpt 43 Example for a Centrifugal Compressor which did not Operate in the Reporting Year 185 XML Excerpt 44 Example for Reciprocating Compressors Details 190 XML Excerpt 45 Example for Reciprocating Compressors Details for Onshore Petroleum and Natural Gas Production 190 XML Excerpt 46 Example for a Reciprocating Compressor in Operating Mode 205 XML Excerpt 47 Example for a Reciprocating Compressor which did not Operate in the Reporting Year 206 XML Excerpt 48 Example for Other Emissions From Equipment Leaks Details 210 XML Excerpt 49 Example for Estimating Emissions Found in Leak Survey Details 216 XML Excerpt 50 Example for Estimating Emissions Using Population Counts Details 219 XML Excerpt 51 Example for Component Count Methodology One Details 221 XML Excerpt 52 Example for Local Distribution Companies Details (Part 1 of 3) 224 XML Excerpt 53 Example for Local Distribution Companies Details (Part 2 of 3) 228 XML Excerpt 54 Example for Local Distribution Companies Details (Part 3 of 3) 231 XML Excerpt 55 Example for EOR Injection Pump Blowdown Details 234 XML Excerpt 56 Example for Unique Injection Pump Blowdowns Details 236 XML Excerpt 57 Example for EOR Hydrocarbon Liquids Dissolved CO2 Details 239 XML Excerpt 58 Example for EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details 240 XML Excerpt 59 Example for Combustion Emissions Details 245 XML Excerpt 60 Example for External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details 247 XML Excerpt 61 Example for External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details 250 XML Excerpt 62 Example for Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details 251 XML Excerpt 63 Example for Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details 253 XML Excerpt 64 Example for External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details 256 XML Excerpt 65 Example for Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details 259 XML Excerpt 66 Example for Subpart W Source Reporting Details 262 XML Excerpt 67 Example for Offshore Emissions Sources Details 266 XML Excerpt 68 Example for Facility-Level Roll-up Emissions 268

# Introduction

The U.S. Environmental Protection Agency’s (EPA’s) electronic greenhouse gas reporting tool (e-GGRT) extensible markup language (XML) Reporting Schema contains all of the data elements needed to comply with the Greenhouse Gas Reporting Program (GHGRP) beginning with the 2010 data collection year. The XML schema defines expected data elements and attributes, allowable data formats for each data element and the hierarchical structure and sequence in which data elements must appear in the XML file. Similar to an architectural blueprint that describes the structural design of a house, an XML schema describes the structural design of an XML file. In some cases, it also defines which elements are optional and which are required and the maximum number of occurrences allowed for each element.

The e-GGRT XML schema is made up of a root data element (GHG) and complex and simple data elements. A simple element is a single piece of data. A complex element is a group of simple elements which are logically grouped together. The root data element is the base of the XML schema.

The schema’s structure can be thought of as a family tree. The elements are related to each other in parent-child relationships. The root element is the parent element of the entire schema. Complex elements are children of the root element and complex elements can also be children of other complex elements.

The XML upload method may be used for reporting a facility or supplier’s annual greenhouse gas (GHG) data; however, the following actions can only be performed using the e-GGRT web forms:

- User, facility and supplier registration

- Certificate of Representation and Notice of Delegation signing

- Facility representative and agent changes

- Facility and supplier address changes

- Notice of intent to not submit an annual GHG report

All XML files submitted to e-GGRT must be well formed and will be accepted only if they conform to the current version of the e-GGRT XML schema.

An XML submission must only contain GHG data for a single facility or supplier. All data for a facility or supplier must be submitted in a single file as a complete report and must include all of the relevant Subparts. It is not possible to submit a subset of any portion of a facility's data to add, delete, correct or update. The entire report must be resubmitted to make any modification at all. Each subsequent submission for the same facility replaces all of the previously submitted data.

The e-GGRT XML schema contains enumerated lists of the units of measures for some data elements and allowable values for some data elements.  For rules regarding the unit of measure or allowable values for a specific data element, please refer to the appropriate Data Element Definitions table.

The e-GGRT XML Reporting Schema is available for download at the e-GGRT help website: http://www.ccdsupport.com/confluence/display/help/XML+Reporting+Instructions. The zip file contains:

- GHG_Final.xsd and Included Files

- SchemaChanges.xlsx

Reporting Numbers

| Number Format | Description |
| --- | --- |
| Rounding | - CO2e and CO2 emissions data expressed in metric tons should be rounded to one decimal place. This should be done regardless of the level of data collection (e.g., unit-level, facility-level). Quantities less than 0.05 metric tons would round to 0.0 and be reported as such. Quantities greater than or equal to 0.05 metric tons would round up to 0.1 and be reported as such. - CH4 emissions data expressed in metric tons should be rounded to two decimal places. - N2O emissions data expressed in metric tons should be rounded to three decimal places. - Emissions data for all GHGs other than CO2, N2O and CH4 expressed in metric tons should be rounded to the fourth digit to the right of the decimal (one tenth of a kilogram, or 1 ten thousandth of a metric ton). This rounding should be applied regardless of the level of data collection (unit, facility, etc.). - Other (non-emissions) quantitative data reported by the user (e.g., a monthly HHV sample result, an annual production quantity) will not need to be rounded. - In the case of aggregation/roll-ups, those calculations should be performed on the rounded values. |
| Percentages | If a value must be reported as a percentage, then the number should be within the range of 0 to 100 (percent), e.g. 85.5% should be reported as 85.5. |
| Fractions | If a value must be reported as a decimal fraction, then the number should be within the range of 0 and 1, e.g., 1/4 should be reported as 0.25. Leading zeroes are optional. |

Key XML Terms

- XML:  A markup language for documents containing structured information. The XML specification defines a standard way to add markup to documents. Its primary purpose is to facilitate the sharing of structured data across different information systems, particularly via the internet.

- XML Schema:  An XML schema describes the structure of an XML document.  The schema also defines the set of rules to which the XML document must conform in order to be considered "valid".

- XML file:  A file containing data organized into a structured document using XML markup.

- Data Element:  An XML data element is used for storing and classifying data in an XML file.  Opening and closing tags represent the start and end of a data element. An opening tag looks like <ElementName>, while a closing tag has a slash that is placed before the element’s name </ElementName>.  The following example shows how to report the facility’s identification number:  <FacilitySiteIdentifier>23222</FacilitySiteIdentifier>.  The information shaded in blue represents the data element’s value. 

If a data element does not contain a value, then a single empty tag name may be used.  An empty tag has a slash placed after the element’s name <FacilitySiteIdentifier/>.  Note:  If you do not intend to report a value for a particular data element, then it is recommended that you do not include the data element in the XML file.

- Attribute: An XML attribute contains additional information about a specific data element. An attribute for a data element is placed within the opening tag.  The syntax for including an attribute in an element is <ElementName AttributeName="value">. For example, <TotalCH4CombustionEmissionsmassUOM="Metric Tons">.

- Root/Parent/Child Element: The schema’s structure can be thought of as a family tree.  At the top of the tree is some early ancestor and at the bottom of the tree are the latest children. With a tree structure you can see which children belong to which parents and many other relationships.  

XML data elements are sometimes referenced in terms of how they relate to each other, e.g., parent-child relationships, within the schema’s tree structure, also known as hierarchy.  The top of the XML tree is considered the root - it is the parent to all data elements within the schema. In the example below, “GHG_Unit_Details” is the root, and just like in many other family trees, there is more than one item with the same name (e.g., “Unit_ID”).  The easiest way to distinguish these items is by referencing them in terms of their parent-child relationships, e.g., NoCEMS /Unit_ID vs. CEMS/Unit_ID.

Example of an XML Tree

This document provides a step-by-step description of how to report emissions data using the XML schema.  Please note the following:

- Non-applicable data elements should not be included in the facility’s XML file.  The schema contains many data elements, some of which may not be applicable to XML reporters in general or to a particular situation. If a data element is not referenced in the instructions (definition tables), then do not report or include it in the facility’s XML file.

- Data elements must be reported in a specific order.  The figures and tables in this document depict the specific sequence in which data elements must be arranged in the facility’s XML file in order to produce a well-formed XML report.

- Enumerations are case sensitive.  Many data elements have a defined set of allowable values, also known as enumerations.  Values for enumerations must be entered exactly as they are defined within the schema (including punctuation marks) in order to be accepted by schema validation.  See the definition tables for a complete list of enumerations.

- Schema diagrams depict the hierarchy (or tree structure).  The primary purpose of the schema diagrams is to indicate the sequence in which data elements must appear within the facility’s XML file and to identify the data elements that are required (must be reported) and conditionally required (see last bullet).  Required data elements are boxed in red and conditionally required data elements are noted.

- Definition tables provide details for required and conditionally required data elements.  The tables are designed to provide unique instructions for reporting a given data element, including the list of enumerations and required units of measure, if defined.  As noted above, there are some data elements in the schema that are not applicable to XML reporters or to a particular situation. For example, the “OverrideIndicator” data element is used solely by e-GGRT to indicate that the web form reporter chose to override the system’s calculated value with their own.  These non-applicable data elements are not included in the definition tables.  If a data element is not referenced in a definition table, then do not report or include it in the facility’s XML file.

- Some data elements are conditionally required.  Data elements which are conditionally required are noted in the schema diagrams and the data element definitions tables.  If your facility meets the condition specified for the data element, then the data element is required and you must report it in the facility’s XML file.  If your facility does not meet the condition specified for the data element, then do not include the data element in the facility’s XML file.  If a parent element is not required, then do not include any of its child data elements in the facility’s XML file.

The XML symbol “1..∞” shown in the figure below means that the parent element is “unbounded” so that multiple instances of the parent element can be reported.  The XML excerpt below shows an example of reporting multiple instances of a parent element.

“Unbounded” Symbol in Schema Diagram

Example for “Unbounded” Parent Element

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Biogenic Carbon dioxide</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>600.1</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Methane</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>280.23</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Nitrous Oxide</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>19.456</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Carbon Dioxide</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>10800.7</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

Note: In the above XML excerpt, there are four instances of the data element GHGGasInfoDetails.

# Summary of Changes

The current version of the GHG XML schema (v5.2 xsd) includes the following modifications to the previous version (v4.2.xsd) in relation to Subpart W.  The vast majority of the changes result from the October 24, 2014 publication of the rule, Revisions to Reporting and Recordkeeping Requirements, and Confidentiality Determinations under the Greenhouse Gas Reporting Program.  This final rulemaking specifies amendments to reporting and recordkeeping requirements and an alternative electronic verification approach for the Greenhouse Gas Reporting Program.  These changes address concerns that public disclosure of certain data elements that are inputs to the equations used to calculate emissions would reveal business sensitive information and maintain EPA’s ability to verify emissions, and ensure compliance. The XML schema that is described in this document, including the changes described below, should be used for all reporting year 2014 annual reporting, and for any first time reporting for reporting years 2011, 2012, and 2013.

If the annual report for reporting years 2011, 2012, or 2013 has already been filed, you must report deferred data elements for Subpart W separately. Reporting deferred data elements for Subpart W for reporting years 2011, 2012, and 2013 are discussed in the document, Subpart W Deferred XML Reporting Instructions.

Summary of Changes to the Schema for Subpart W

| Source Category | Change Description |
| --- | --- |
| Pneumatic Device Venting | New Data Element: Applicable irrespective of reporting year. - Added element "ActualCount" under “PneumaticDeviceVentingDetails”. (XPath = SubPartW/SubPartWReportingFormsDetails/PneumaticDeviceVentingDetails/PneumaticDeviceTypesDetails/PneumaticDeviceTypesRowDetails/ActualCount) - Added element "EstimatedCount" under “PneumaticDeviceVentingDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/PneumaticDeviceVentingDetails/PneumaticDeviceTypesDetails/PneumaticDeviceTypesRowDetails/EstimatedCount) |
| Natural Gas Driven Pneumatic Pumps | New Data Element: Applicable irrespective of reporting year. - Added element "TotalPneumaticPumpCount" under “NaturalGasDrivenPneumaticPumpsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/NaturalGasDrivenPneumaticPumpsDetails/PneumaticPumpTypesDetails/PneumaticPumpTypesRowDetails/TotalPneumaticPumpCount) |
| Acid Gas Removal Units | New Data Element: Applicable irrespective of reporting year. - Added element " AverageVentCarbonDioxideFractionMethod2" under “AcidGasRemovalUnitsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/AcidGasRemovalUnitsDetails/AcidGasRemovalNumberOfUnitsDetails/AcidGasRemovalNumberOfUnitsRowDetails/AverageVentCarbonDioxideFractionMethod2) - Added element " AverageNaturalGasCarbonDioxideFractionIn " under “AcidGasRemovalUnitsDetails”. (XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AcidGasRemovalUnitsDetails/AcidGasRemovalNumberOfUnitsDetails/AcidGasRemovalNumberOfUnitsRowDetails/AverageNaturalGasCarbonDioxideFractionIn) - Added element " AverageNaturalGasCarbonDioxideFractionOut " under “AcidGasRemovalUnitsDetails”. (XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AcidGasRemovalUnitsDetails/AcidGasRemovalNumberOfUnitsDetails/AcidGasRemovalNumberOfUnitsRowDetails/AverageNaturalGasCarbonDioxideFractionOut) - Added element "TotalThroughput" under “AcidGasRemovalUnitsDetails”. (XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AcidGasRemovalUnitsDetails/AcidGasRemovalNumberOfUnitsDetails/AcidGasRemovalNumberOfUnitsRowDetails/TotalThroughput) - Added element "AnnualCarbonDioxideRecovered" under “AcidGasRemovalUnitsDetails”. (XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AcidGasRemovalUnitsDetails/AcidGasRemovalNumberOfUnitsDetails/AcidGasRemovalNumberOfUnitsRowDetails/AnnualCarbonDioxideRecovered) |
| Dehydrators | New Data Element: Applicable irrespective of reporting year. - Added element "GlycolDehydratorCount" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/GlycolDehydratorCount) - Added element "NaturalGasFlowRate" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsMoreThanSpecifiedValueDetails/GlycolDehydratorsMoreThanSpecifiedValueRowDetails/NaturalGasFlowRate) - Added element "AbsorbentCirculatorPumpType" under “DehydratorsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/AbsorbentCirculatorPumpType) - Added element "IsStripperGasUsed" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/IsStripperGasUsed) - Added element "IsFlashTankSeparatorUsed" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/IsFlashTankSeparatorUsed) - Added element "AbsorbentType" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/AbsorbentType) - Added element "GlycolDehydratorOperatingHours" under “DehydratorsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/GlycolDehydratorOperatingHours) - Added element "WetNaturalGasTemperature" under “DehydratorsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/WetNaturalGasTemperature) - Added element "WetNaturalGasPressure" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/WetNaturalGasPressure) - Added element "WetNaturalGasCarbonDioxideConcentration" under “DehydratorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/WetNaturalGasCarbonDioxideConcentration) - Added element "WetNaturalGasMethaneConcentration" under “DehydratorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/WetNaturalGasMethaneConcentration) Enumeration additions: Applicable irrespective of reporting year. - Added enumerations for “AbsorbentCirculatorPumpType” under “DehydratorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/AbsorbentCirculatorPumpType): - Natural gas pneumatic - Air pneumatic - Electric - Electric/Pneumatic - Added enumerations for "IsStripperGasUsed" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/IsStripperGasUsed): - Yes - No - Added enumerations for "IsFlashTankSeparatorUsed" under “DehydratorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/IsFlashTankSeparatorUsed): - Yes - No - Added enumerations for "AbsorbentType" under “DehydratorsDetails”. (XPath =SubPartW/SubPartWReportingFormsDetails/DehydratorsDetails/GlycolDehydratorsLessThanSpecifiedValueDetails/GlycolDehydratorsLessThanSpecifiedValueRowDetails/AbsorbentType) - Triethylene glycol (TEG) - Diethylene glycol (DEG) - Ethylene glycol (EG) |
| Well venting | New Data Element: Applicable irrespective of reporting year. - Added element "AverageWellVentingFlowRate" under “WellVentingDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/WellVentingDetails/WellVentingCalculationMethodOneDetails/WellVentingCalculationMethodOneRowDetails/AverageWellVentingFlowRate) - Added element "CumulativeNumberOfUnloadingsVented" under “WellVentingDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellVentingDetails/WellVentingCalculationMethodTwoDetails/WellVentingCalculationMethodTwoRowDetails/CumulativeNumberOfUnloadingsVented) - Added element "CumulativeNumberOfUnloadingsVented" under “WellVentingDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellVentingDetails/WellVentingCalculationMethodThreeDetails/WellVentingCalculationMethodThreeRowDetails/CumulativeNumberOfUnloadingsVented) |
| Gas Well Completion Workover | New Data Element: Applicable irrespective of reporting year. - Added element "TwoYearDelayIndicator" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/TwoYearDelayIndicator) - Added element "APIWellNumbers" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/APIWellNumbers) - Added element "CompletionBackflowFlowRate" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/CompletionBackflowFlowRate) - Added element "WorkoverBackflowFlowRate" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/WorkoverBackflowFlowRate) - Added element "CompletionsNumberOfBackflowDays" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/CompletionsNumberOfBackflowDays) - Added element "WorkoversNumberOfBackflowDays" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/WorkoversNumberOfBackflowDays) - Added element "CompletionsGasRecoveredToSales" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/CompletionsGasRecoveredToSales) - Added element "WorkoversGasRecoveredToSales" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithHydraulicFracturingDetails/WellCompletionsWorkoversWithHydraulicFracturingRowDetails/WorkoversGasRecoveredToSales) - Added element "NumberOfCompletions" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithoutHydraulicFracturingDetails/WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails/NumberOfCompletions) - Added element "NumberOfWorkoversThatFlareOrVentGas" under “WellCompletionsWorkoversDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/WellCompletionsWorkoversDetails/WellCompletionsWorkoversWithoutHydraulicFracturingDetails/WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails/NumberOfWorkoversThatFlareOrVentGas) |
| Atmospheric Tanks | New Data Element: Applicable irrespective of reporting year. - Added element "TwoYearDelayIndicator" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodOneOrTwoDetails/AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails/TwoYearDelayIndicator) - Added element "APIWellNumbers" under “AtmosphericTanksDetails”.(XPath= SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodOneOrTwoDetails/AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails/APIWellNumbers) - Added element "TotalVolumeOfOil" under “AtmosphericTanksDetails”.(XPath= SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodOneOrTwoDetails/AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails/TotalVolumeOfOil) - Added element "AnnualCarbonDioxideRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodOneOrTwoDetails/AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails/AnnualCarbonDioxideRecovered) - Added element "AnnualMethaneRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodOneOrTwoDetails/AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails/AnnualMethaneRecovered) - Added element "TwoYearDelayIndicator" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodThreeOrFourDetails/AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails/TwoYearDelayIndicator) - Added element "APIWellNumbers" under “AtmosphericTanksDetails”.(XPath= SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodThreeOrFourDetails/AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails/APIWellNumbers) - Added element "TotalVolumeOfSalesOil" under “AtmosphericTanksDetails”.(XPath= SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodThreeOrFourDetails/AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails/TotalVolumeOfSalesOil) - Added element "AnnualCarbonDioxideRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodThreeOrFourDetails/AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails/AnnualCarbonDioxideRecovered) - Added element "AnnualMethaneRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodThreeOrFourDetails/AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails/AnnualMethaneRecovered) - Added element "AnnualCarbonDioxideRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodFiveDetails/AtmosphericTanksCalculationMethodFiveSubBasinRowDetails/AnnualCarbonDioxideRecovered) - Added element "AnnualMethaneRecovered" under “AtmosphericTanksDetails”.(XPath=SubPartW/ghg:SubPartWReportingFormsDetails/AtmosphericTanksDetails/AtmosphericTanksCalculationMethodFiveDetails/AtmosphericTanksCalculationMethodFiveSubBasinRowDetails/AnnualMethaneRecovered) |
| Flare Stacks | New Data Element: Applicable irrespective of reporting year. - Added element "GasSentToFlare" under “FlareStacksDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/FlareStacksDetails/ghg:UniqueFlareStacksDetails/UniqueFlareStacksRowDetails/GasSentToFlare) - Added element "FlareCombustionEfficiency" under “FlareStacksDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/FlareStacksDetails/ghg:UniqueFlareStacksDetails/UniqueFlareStacksRowDetails/FlareCombustionEfficiency) |
| Centrifugal Compressors | New Data Element: Applicable irrespective of reporting year. - Added element "CompressorCount" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CompressorCount) - Added element "TotalTimeInOperatingMode" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/TotalTimeInOperatingMode) - Added element "WetSealOilEmissionFactor" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/WetSealOilEmissionFactor) - Added element "BlowdownVentsEmissionFactor" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/BlowdownVentsEmissionFactor) - Added element "TotalTimeInShutdownMode" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/TotalTimeInShutdownMode) - Added element "IsolationValveEmissionFactor" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/IsolationValveEmissionFactor) |
| Reciprocating Compressors | New Data Element: Applicable irrespective of reporting year. - Added element "CompressorCount" under “ReciprocatingCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/ReciprocatingCompressorsDetails/CompressorCount) - Added element "TotalTimeInOperatingMode" under “ReciprocatingCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/ReciprocatingCompressorsDetails/ReciprocatingCompressorsModesDetails/ReciprocatingCompressorsModesRowDetails/TotalTimeInOperatingMode) - Added element "TotalTimeInStandbyMode" under “ReciprocatingCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/ReciprocatingCompressorsDetails/ReciprocatingCompressorsModesDetails/ReciprocatingCompressorsModesRowDetails/TotalTimeInStandbyMode) - Added element "BlowdownVentsEmissionFactor" under “ReciprocatingCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/ReciprocatingCompressorsDetails/ReciprocatingCompressorsModesDetails/ReciprocatingCompressorsModesRowDetails/BlowdownVentsEmissionFactor) - Added element "TotalTimeInNotOperatingMode" under “ReciprocatingCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/ReciprocatingCompressorsDetails/ReciprocatingCompressorsModesDetails/ReciprocatingCompressorsModesRowDetails/TotalTimeInNotOperatingMode) - Added element "IsolationValveEmissionFactor" under “CentrifugalCompressorsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CentrifugalCompressorsDetails/CentrifugalCompressorsModesDetails/CentrifugalCompressorsModesRowDetails/IsolationValveEmissionFactor) |
| Other Emissions from Equipment Leaks | New Data Element: Applicable irrespective of reporting year. - Added element "ComponentCountMethodologyOneDetails" under “OtherEmissionsFromEquipmentLeaksDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/OtherEmissionsFromEquipmentLeaksDetails/ComponentCountMethodologyOneDetails) - Added element "MajorEquipmentType" under “ComponentCountMethodologyOneDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/OtherEmissionsFromEquipmentLeaksDetails/ComponentCountMethodologyOneDetails/ComponentCountMethodologyOneRowDetails/MajorEquipmentType) - Added element "TotalCountForMajorEquipmentType" under “ComponentCountMethodologyOneDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/OtherEmissionsFromEquipmentLeaksDetails/ComponentCountMethodologyOneDetails/ComponentCountMethodologyOneRowDetails/TotalCountForMajorEquipmentType) - Added element "TotalCountForEachLeakSourceType" under “EstimatingEmissionsUsingPopulationCountsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/OtherEmissionsFromEquipmentLeaksDetails/EstimatingEmissionsUsingPopulationCountsDetails/EstimatingEmissionsUsingPopulationCountsRowDetails/TotalCountForEachLeakSourceType) Enumeration additions: Applicable irrespective of reporting year. - Added enumerations for “MajorEquipmentType” under “ComponentCountMethodologyOneDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/OtherEmissionsFromEquipmentLeaksDetails/ComponentCountMethodologyOneDetails/ComponentCountMethodologyOneRowDetails/MajorEquipmentType): - Wellheads - Separators - Meters/piping - Compressors - In-line heaters - Dehydrators - Heater-treater - Header |
| Enhanced Oil Recovery Injection Pump Blowdown | New Data Element: Applicable irrespective of reporting year. - Added element "CriticalPhaseGasVolume" under “EnhancedOilRecoveryInjectionPumpBlowdownDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/EnhancedOilRecoveryInjectionPumpBlowdownDetails/UniqueInjectionPumpBlowdownsDetails/UniqueInjectionPumpBlowdownsRowDetails/CriticalPhaseGasVolume) - Added element "NumberOfBlowdowns" under “EnhancedOilRecoveryInjectionPumpBlowdownDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/EnhancedOilRecoveryInjectionPumpBlowdownDetails/UniqueInjectionPumpBlowdownsDetails/UniqueInjectionPumpBlowdownsRowDetails/NumberOfBlowdowns) - Added element "CriticalPhaseGasDensity" under “EnhancedOilRecoveryInjectionPumpBlowdownDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/EnhancedOilRecoveryInjectionPumpBlowdownDetails/UniqueInjectionPumpBlowdownsDetails/UniqueInjectionPumpBlowdownsRowDetails/CriticalPhaseGasDensity) |
| Enhanced Oil Recovery Hydrocarbon Liquids Dissolved | New Data Element: Applicable irrespective of reporting year. - Added element "CrudeOilProduced" under “EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails/HydrocarbonLiquidsDissolvedDetails/HydrocarbonLiquidsDissolvedSubBasinRowDetails/CrudeOilProduced) - Added element "CarbonDioxideRetained" under “EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails/HydrocarbonLiquidsDissolvedDetails/HydrocarbonLiquidsDissolvedSubBasinRowDetails/CarbonDioxideRetained) |
| Combustion Emissions | New Data Element: Applicable irrespective of reporting year. - Added element "FuelType" under“CombustionEmissionsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/FuelType) - Added element "CumulativeFuelCombustedVolume" under“CombustionEmissionsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/CumulativeFuelCombustedVolume) - Added element "FuelVolumeUnits" under“CombustionEmissionsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/FuelVolumeUnits) - Added element "FuelType" under“CombustionEmissionsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/FuelType) - Added element "CumulativeFuelCombustedVolume" under“CombustionEmissionsDetails”.(XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/CumulativeFuelCombustedVolume) - Added element "FuelVolumeUnits" under“CombustionEmissionsDetails”. (XPath=SubPartW/SubPartWReportingFormsDetails/CombustionEmissionsDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails/InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails/FuelVolumeUnits) |

# Subpart W Overview

This document provides a step-by-step description of how to report data for Subpart W - Petroleum and Natural Gas Systems and overall total Subpart W greenhouse gas data for a facility using the XML schema.

This source category consists of the following industry segments [98.230(a)]:

- Offshore petroleum and natural gas production

- Onshore petroleum and natural gas production

- Onshore natural gas processing

- Onshore natural gas transmission compression

- Underground natural gas storage

- Liquefied natural gas (LNG) storage

- LNG import and export equipment

- Natural gas distribution

Data is to be reported by source type within each industry segment.  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit’s respective majority use industry segment.  [98.236(c)]

The following industry segments are treated as separate facilities and cannot be reported with other industry segments.

- Offshore petroleum and natural gas production

- Onshore petroleum and natural gas production

- Natural gas distribution

The other industry segments can be reported in the same annual GHG report.

Subpart W Reporting Diagram

The XML schema includes the following areas for reporting for Subpart W, as displayed in the reporting diagram:

- Subpart W Total Emissions:  includes the total greenhouse gas emissions required to be reported for Subpart W.

- Subpart W Summary Details:  includes information on each applicable industry segment, including annual throughput and emissions totals by industry segment and source category.

- Offshore petroleum and natural gas production:  Offshore petroleum and natural gas production is any platform structure, affixed temporarily or permanently to offshore submerged lands, that houses equipment to extract hydrocarbons from the ocean or lake floor and that processes and/or transfers such hydrocarbons to storage, transport vessels, or onshore. In addition, offshore production includes secondary platform structures connected to the platform structure via walkways, storage tanks associated with the platform structure and floating production and storage offloading equipment (FPSO). This source category does not include reporting of emissions from offshore drilling and exploration that is not conducted on production platforms.  

Note:  If your facility reports under this industry segment, you should only report for Subparts A and C in addition to this industry segment within Subpart W.

- Onshore petroleum and natural gas production:  Onshore petroleum and natural gas production means all equipment on a single well-pad or associated with a single well-pad (including but not limited to compressors, generators, dehydrators, storage vessels and portable non-self-propelled equipment which includes well drilling and completion equipment, workover equipment, gravity separation equipment, auxiliary non-transportation-related equipment and leased, rented or contracted equipment) used in the production, extraction, recovery, lifting, stabilization, separation or treating of petroleum and/or natural gas (including condensate). This equipment also includes associated storage or measurement vessels and all enhanced oil recovery (EOR) operations using CO2 or natural gas injection and all petroleum and natural gas production equipment located on islands, artificial islands, or structures connected by a causeway to land, an island, or an artificial island.  

Note 1:  Onshore petroleum and natural gas production is treated as an independent facility by 40 CFR 98. Therefore, if you own or operate onshore petroleum and natural gas production equipment that is co-located with any GHG-emitting equipment used under a different industry segment, you must conduct an assessment to determine applicability for the onshore petroleum and natural gas production facility, as defined below. The other GHG-emitting equipment would be assessed as a separate facility.  

Facility with respect to onshore petroleum and natural gas production for purposes of reporting under this subpart and for the corresponding subpart A requirements means all petroleum or natural gas equipment on a single well-pad or associated with a single well-pad and CO2 EOR operations that are under common ownership or common control including leased, rented, or contracted activities by an onshore petroleum and natural gas production owner or operator and that are located in a single hydrocarbon basin as defined in 98.238. Where a person or entity owns or operates more than one well in a basin, then all onshore petroleum and natural gas production equipment associated with all wells that the person or entity owns or operates in the basin would be considered one facility.

Note 2:  If your facility reports under this industry segment, you may only report for Subpart A in addition to this industry segment within Subpart W.

- Onshore natural gas processing:  Natural gas processing means the separation of natural gas liquids (NGLs) or non-methane gases from produced natural gas, or the separation of NGLs into one or more component mixtures. Separation includes one or more of the following: forced extraction of natural gas liquids, sulfur and carbon dioxide removal, fractionation of NGLs, or the capture of CO2 separated from natural gas streams. This segment also includes all residue gas compression equipment owned or operated by the natural gas processing plant. This industry segment includes processing plants that fractionate gas liquids and processing plants that do not fractionate gas liquids but have an annual average throughput of 25 MMscf per day or greater.

- Onshore natural gas transmission compression:  Onshore natural gas transmission compression means any stationary combination of compressors that move natural gas from production fields, natural gas processing plants, or other transmission compressors through transmission pipelines to natural gas distribution pipelines, LNG storage facilities, or into underground storage. In addition, a transmission compressor station includes equipment for liquids separation and tanks for the storage of water and hydrocarbon liquids. Residue (sales) gas compression that is part of onshore natural gas processing plants are included in the onshore natural gas processing segment and are excluded from this segment.

- Underground natural gas storage:  Underground natural gas storage means subsurface storage, including depleted gas or oil reservoirs and salt dome caverns that store natural gas that has been transferred from its original location for the primary purpose of load balancing (the process of equalizing the receipt and delivery of natural gas); natural gas underground storage processes and operations (including compression, dehydration and flow measurement and excluding transmission pipelines); and all the wellheads connected to the compression units located at the facility that inject and recover natural gas into and from the underground reservoirs.

- Liquefied natural gas (LNG) storage:  LNG storage means onshore LNG storage vessels located above ground, equipment for liquefying natural gas, compressors to capture and re-liquefy boil-off-gas, re-condensers and vaporization units for re-gasification of the liquefied natural gas.

- LNG import and export equipment:  LNG import equipment means all onshore or offshore equipment that receives imported LNG via ocean transport, stores LNG, re-gasifies LNG and delivers re-gasified natural gas to a natural gas transmission or distribution system. LNG export equipment means all onshore or offshore equipment that receives natural gas, liquefies natural gas, stores LNG and transfers the LNG via ocean transportation to any location, including locations in the United States.

- Natural gas distribution:  Natural gas distribution means the collection of all distribution pipelines and metering and regulating equipment at metering-regulating stations that are operated by a Local Distribution Company (LDC) within a single state that is regulated as a separate operating company by a public utility commission or that is operated as an independent municipally-owned distribution system. This segment also excludes customer meters and regulators, infrastructure and pipelines (both interstate and intrastate) delivering natural gas directly to major industrial users and farm taps upstream of the local distribution company inlet.

Note 1:  Natural gas distribution is treated as an independent facility by 40 CFR 98.  Therefore, if you own or operate a natural gas distribution system and other GHG-emitting equipment used under a different industry segment, you must conduct an assessment to determine applicability for the natural gas distribution facility as defined below. The other GHG-emitting equipment would be assessed as a separate facility as defined above.

Facility with respect to natural gas distribution for purposes of reporting under this subpart and for the corresponding subpart A requirements means the collection of all distribution pipelines and metering-regulating stations that are operated by a Local Distribution Company (LDC) within a single state that is regulated as a separate operating company by a public utility commission or that are operated as an independent municipally-owned distribution system.

Note 2:  If your LDC reports under this industry segment, you may only report for Subparts A and NN in addition to this industry segment within Subpart W. Because Subpart NN is a supplier category, it may be submitted either under the same facility registration as Subpart W or as a separate facility. Reporting determinations for Subparts NN and W are considered independently.

- Sub-basin Selection / Onshore Requirements Under 98.236(e)

- Natural Gas Pneumatic Devices [98.236(c)(1)]

- Natural Gas Driven Pneumatic Pumps [98.236(c)(2)]

- Acid Gas Removal Units [98.236(c)(3)]

- Dehydrators [98.236(c)(4)]

- Well Venting for Liquids Unloading [98.236(c)(5)]

- Gas Well Completions and Workovers [98.236(c)(6)]

- Blowdown Vent Stacks [98.236(c)(7)]

- Gas from Produced Oil Sent to Atmospheric Tanks [98.236(c)(8)]

- Transmission Tanks [98.236(c)(9)]

- Well Testing Venting and Flaring [98.236(c)(10)]

- Associated Gas Venting and Flaring [98.236(c)(11)]

- Flare Stacks [98.236(c)(12)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

- Local Distribution Companies [98.236(c)(16)]

- Enhanced Oil Recovery Injection Pump Blowdown [98.236(c)(17)]

- Enhanced Oil Recovery Hydrocarbon Liquids Dissolved CO2 [98.236(c)(18)]

- Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)]

- Offshore Sources [98.236(b)]

- Facility-Level Roll-up Emissions:  includes information on how to report total emissions for CO2e (excluding biogenic CO2) from Subpart W at the facility level.

Note:  If your facility is subject to reporting under Subpart W (Petroleum and Natural Gas Systems), EPA recommends that you also consider the following source categories in your facility applicability determination:

- Subpart C (General Stationary Fuel Combustion)

- Subpart D (Electricity Generation)

- Subpart G (Ammonia Manufacturing)

- Subpart P (Hydrogen Production)

- Subpart FF (Underground Coal Mines)

- Subpart MM (Suppliers of Petroleum Products)

- Subpart NN (Suppliers of Natural Gas and Natural Gas Liquids)

- Subpart PP (Suppliers of Carbon Dioxide)

- Subpart RR (Geologic Sequestration of Carbon Dioxide)

- Subpart UU (Injection of Carbon Dioxide).

These source categories are only provided as suggestions - additional Subparts may be relevant for a given facility/supplier and not all listed Subparts are relevant for all facilities/suppliers.

Subpart W Schema Diagram

## Subpart W Total Emissions

Greenhouse gas information details comprise a collection of data elements to report the total annual emissions of each greenhouse gas (GHG) listed in Table A-1 of 40 CFR 98 Mandatory Reporting of Greenhouse Gases reported under Subpart W, expressed in metric tons.

Greenhouse Gas Information Details Schema Diagram

Note: Data elements boxed in red are required.

For Subpart W, report total emissions for carbon dioxide (CO2), methane (CH4) and nitrous oxide (N2O) from all applicable industry segments and flares at the Subpart level as specified using the following guidelines [98.232].

- For offshore petroleum and natural gas production, report CO2, CH4 and N2O emissions from equipment leaks, vented emission and flare emission source types as identified in the data collection and emissions estimation study conducted by BOEMRE in compliance with 30 CFR 250.302 through 304. Offshore platforms do not need to report portable emissions.  [98.232(b)]

- For an onshore petroleum and natural gas production facility, report CO2, CH4 and N2O emissions from only the following source types on a single well-pad or associated with a single well-pad [98.232(c)]:

- Natural gas pneumatic device venting

- Natural gas driven pneumatic pump venting

- Well venting for liquids unloading

- Gas well venting during well completions and workovers without hydraulic fracturing

- Gas well venting during well completions and workovers with hydraulic fracturing.

- Flare stack emissions

- Storage tanks vented emissions from produced hydrocarbons

- Reciprocating compressor rod packing venting

- Well testing venting and flaring

- Associated gas venting and flaring from produced hydrocarbons

- Dehydrator vents

- EOR injection pump blowdown

- Acid gas removal vents

- EOR hydrocarbon liquids dissolved CO2

- Centrifugal compressor venting

- Equipment leaks from valves, connectors, open ended lines, pressure relief valves, pumps, flanges and other equipment leak sources (such as instruments, loading arms, stuffing boxes, compressor seals, dump lever arms and breather caps)

- You must use the methods in 98.233(z) and report under this subpart the emissions of CO2, CH4 and N2O from stationary or portable fuel combustion equipment that cannot move on roadways under its own power and drive train and that is located at an onshore petroleum and natural gas production facility as defined in 98.238. Stationary or portable equipment are the following equipment, which are integral to the extraction, processing, or movement of oil or natural gas: well drilling and completion equipment, workover equipment, natural gas dehydrators, natural gas compressors, electrical generators, steam boilers and process heaters.

- For onshore natural gas processing, report CO2, CH4 and N2O emissions from the following sources [98.232(d)]:

- Reciprocating compressor rod packing venting

- Centrifugal compressor venting

- Blowdown vent stacks

- Dehydrator vents

- Acid gas removal vents

- Flare stack emissions

- Equipment leaks from valves, connectors, open ended lines, pressure relief valves and meters

- For onshore natural gas transmission compression, report CO2, CH4 and N2O emissions from the following sources [98.232(e)]:

- Reciprocating compressor rod packing venting

- Centrifugal compressor venting

- Transmission storage tanks

- Blowdown vent stacks

- Natural gas pneumatic device venting

- Equipment leaks from valves, connectors, open ended lines, pressure relief valves and meters

- For underground natural gas storage, report CO2, CH4 and N2O emissions from the following sources [98.232(f)]:

- Reciprocating compressor rod packing venting

- Centrifugal compressor venting

- Natural gas pneumatic device venting

- Equipment leaks from valves, connectors, open ended lines, pressure relief valves and meters

- For LNG storage, report CO2, CH4 and N2O emissions from the following sources [98.232(g)]:

- Reciprocating compressor rod packing venting

- Centrifugal compressor venting

- Equipment leaks from valves; pump seals; connectors; vapor recovery compressors and other equipment leak sources

- LNG import and export equipment, report CO2, CH4 and N2O emissions from the following sources [98.232(h)]:

- Reciprocating compressor rod packing venting

- Centrifugal compressor venting

- Blowdown vent stacks

- Equipment leaks from valves, pump seals, connectors, vapor recovery compressors and other equipment leak sources

- For natural gas distribution, report CO2, CH4 and N2O emissions from the following sources [98.232(i)]:

- Meters, regulators and associated equipment at above grade transmission-distribution transfer stations, including equipment leaks from connectors, block valves, control valves, pressure relief valves, orifice meters, regulators and open ended lines

- Equipment leaks from vaults at below grade transmission-distribution transfer stations

- Meters, regulators and associated equipment at above grade metering-regulating station

- Equipment leaks from vaults at below grade metering-regulating stations

- Pipeline main equipment leaks

- Service line equipment leaks

- The emissions of CO2, CH4 and N2O emissions from stationary fuel combustion sources following the methods in 98.233(z)

Note:  You must follow the rounding rules found in Table 1.

Report under Subpart C (General Stationary Fuel Combustion Sources) the emissions of CO2, CH4 and N2O from each stationary fuel combustion unit by following the requirements of Subpart C except for facilities under onshore petroleum and natural gas production and natural gas distribution.  (Onshore petroleum and natural gas production facilities and natural gas distribution facilities must report stationary and portable combustion emissions as specified above.) [98.232(k)]

For offshore petroleum and natural gas production, combustion emissions other than flaring must be reported under Subpart C.

Report under Subpart PP (Suppliers of Carbon Dioxide) CO2 emissions captured and transferred off-site by following the requirements of Subpart PP. [98.232(l)]

The values to report for “GHGasQuantity” are equivalent to the following:

- For Carbon Dioxide, the value reported for the data element “TotalReportedCarbonDioxideEmissions” in Subpart W Summary Details.

- For Methane, the value reported for the data element “TotalReportedMethaneCarbonDioxideEquivalent” in Subpart W Summary Details divided by the Global Warming Potential, 25, to convert to metric tons CH4.

- For Nitrous Oxide, the value reported for the data element “TotalReportedNitrousCarbonDioxideEquivalent” in Subpart W Summary Details divided by the Global Warming Potential, 298, to convert to metric tons N2O.

Greenhouse Gas Information Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| GHGasInfoDetails | Parent Element: A collection of data elements containing the total annual emissions of each greenhouse gas (GHG) listed in Table A-1 of 40 CFR 98 Mandatory Reporting of Greenhouse Gases reported under this Subpart, expressed in metric tons. |
| GHGasName | Specify the name of the GHG. See list of allowable values: Carbon Dioxide. Methane. Nitrous Oxide. |
| GHGasQuantity | A collection of data elements that quantify the annual emissions required to report under Subpart W. Report the value in the child data element CalculatedValue using the guidelines above. Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Greenhouse Gas Information Details

<ghg:SubPartWDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Methane</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>785196.60</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Nitrous Oxide</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>2.209</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

<ghg:GHGasInfoDetails>

<ghg:GHGasName>Carbon Dioxide</ghg:GHGasName>

<ghg:GHGasQuantity massUOM="Metric Tons">

<ghg:CalculatedValue>2733773.7</ghg:CalculatedValue>

</ghg:GHGasQuantity>

</ghg:GHGasInfoDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

## Subpart W Summary Details

Summary data must be reported separately for each applicable industry segment.

Subpart W Reporting Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

Subpart W Summary Details Schema Diagram

Note: Data elements boxed in red are required.

The following summary data must be reported for each applicable industry segment:

- The number and name of the industry segment.

- Offshore petroleum and natural gas production [98.230(a)(1)]

- Onshore petroleum and natural gas production [98.230(a)(2)]

- Onshore natural gas processing [98.230(a)(3)]

- Onshore natural gas transmission compression [98.230(a)(4)]

- Underground natural gas storage [98.230(a)(5)]

- Liquefied natural gas (LNG) storage [98.230(a)(6)]

- LNG import and export equipment [98.230(a)(7)]

- Natural gas distribution [98.230(a)(8)]

- The annual gaseous and liquid throughput as determined by engineering estimate based on best available data for the industry segment.  For gaseous throughput, report in MMscf.  For liquid throughput, report in thousand barrels. [98.236(d)]

- The annual CO2, CH4 and N2O emissions in metric tons of CO2e for each GHG separately for the industry segment. [98.236(a)(2)].

  - The value to report for “TotalReportedCarbonDioxideEmissions” equals the sum of the values reported for “TotalReportedCarbonDioxideEmissions” for each applicable source category for the specified industry segment.

  - The value to report for “TotalReportedMethaneCarbonDioxideEquivalent” equals the sum of the values reported for “TotalReportedMethaneCarbonDioxideEquivalent” for each applicable source category for the specified industry segment.

  - The value to report for “TotalReportedNitrousCarbonDioxideEquivalent” equals the sum of the values reported for “TotalReportedNitrousCarbonDioxideEquivalent” for each applicable source category for the specified industry segment.

- The total annual CO2, CH4 and N2O combined emissions in metric tons of CO2e for the industry segment.

  - The value to report for “TotalReportedCarbonDioxideEquivalent” equals the sum of the values reported for “TotalReportedCarbonDioxideEquivalent” for each applicable source category for the specified industry segment.

Please see Required Sources to Report by Industry Segment for source types applicable to and required for each industry segment.

For each instance of the parent element “SubPartWReportingFormsDetails”, report data for one applicable industry segment and all of its applicable source types.  Report a separate instance of the parent element “SubpartWSourceReportingFormRowDetails” for each applicable source category for the specified industry segment.

Subpart W Summary Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| SubPartWReportingFormsDetails | Parent Element: A collection of data elements to report for each applicable industry segment in Subpart W. Report data for each applicable industry segment separately. |
| SubpartWSummaryDetails | Parent Element: A collection of data elements to report summary data for each applicable industry segment. |
| SelectedIndustrySegmentNumber | The number which corresponds to the applicable industry segment. See list of allowable values. 1 2 3 4 5 6 7 8 |
| SelectedIndustrySegmentName | The name of the industry segment. See list of allowable values. Offshore petroleum and natural gas production [98.230(a)(1)] Onshore petroleum and natural gas production [98.230(a)(2)] Onshore natural gas processing [98.230(a)(3)] Onshore natural gas transmission compression [98.230(a)(4)] Underground natural gas storage [98.230(a)(5)] Liquefied natural gas (LNG) storage [98.230(a)(6)] LNG import and export equipment [98.230(a)(7)] Natural gas distribution [98.230(a)(8)] |
| AnnualGaseousThroughput | The annual gaseous throughput in MMscf for the specified industry segment. [98.236(d)] Set the units of measure to “MMscf” in the attribute volUOM. Note: Report “0” if the facility did not have any gaseous throughput during the reporting year. |
| AnnualLiquidThroughput | The annual liquid throughput in thousand barrels for the specified industry segment. [98.236(d)] Set the units of measure to “thousand barrels” in the attribute volUOM. Note: Report “0” if the facility did not have any liquid throughput during the reporting year. |
| TotalReportedCarbonDioxideEmissions | Total reported CO2 emissions in metric tons for the specified industry segment. [98.236(a)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalReportedMethaneCarbonDioxideEquivalent | Total reported CH4 emissions in metric tons CO2e for the specified industry segment. [98.236(a)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalReportedNitrousCarbonDioxideEquivalent | Total reported N2O emissions in metric tons CO2e for the specified industry segment. [98.236(a)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalReportedCarbonDioxideEquivalent | Total reported CO2, CH4 and N2O combined emissions in metric tons CO2e for the specified industry segment. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| SubpartWSourceReportingFormDetails | Parent Element: A collection of data elements to report for each applicable industry segment in Subpart W. Report data for each applicable source category for each industry segment separately. |

Example for Subpart W Summary Details

<ghg:SubPartWReportingFormsDetails>

<ghg:SubpartWSummaryDetails>

<ghg:SelectedIndustrySegmentNumber>8</ghg:SelectedIndustrySegmentNumber>

<ghg:SelectedIndustrySegmentName>Natural gas distribution [98.230(a)(8)]</ghg:SelectedIndustrySegmentName>

<ghg:AnnualGaseousThroughput volUOM="MMscf">85821429</ghg:AnnualGaseousThroughput>

<ghg:AnnualLiquidThroughput volUOM="thousand barrels">0</ghg:AnnualLiquidThroughput>

<ghg:TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">808.8</ghg:TotalReportedCarbonDioxideEmissions>

<ghg:TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">562162.3</ghg:TotalReportedMethaneCarbonDioxideEquivalent>

<ghg:TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric Tons">11.3</ghg:TotalReportedNitrousCarbonDioxideEquivalent>

<ghg:TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons"> 562982.4</ghg:TotalReportedCarbonDioxideEquivalent>

<ghg:SubpartWSourceReportingFormDetails>

<See example for Subpart W Source Reporting Details>

</ghg:SubpartWSourceReportingFormDetails>

</ghg:SubpartWSummaryDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Subpart W Source Reporting Details Schema Diagram

Note: Data elements boxed in red are required.

For each applicable source type required for the industry segment indicated in the data element “SelectedIndustrySegmentName”, report the following:

- The name of the applicable source type.  Note:  Not all source types are applicable to every industry segment.  Please see Required Sources to Report by Industry Segment for source types applicable to and required for each industry segment.

- An indication that the source type is required for the industry segment.

- The annual CO2, CH4 and N2O emissions in metric tons of CO2e for each GHG separately for the applicable source type.  Note 1:  Do not report for “Sub-Basin Selection”.  Note 2:  There are some source types for which CH4 and N2O emissions do not apply.  If reporting for one of these sources, include only the annual emissions for applicable GHG(s).  Report “0” if the facility did not have any emissions for an applicable GHG.

- The value to report for “TotalReportedCarbonDioxideEmissions” for each source type is equivalent to the value reported for the data element “TotalCarbonDioxideEmissions” for the source type.

- The value to report for “TotalReportedMethaneCarbonDioxideEquivalent” for each source type is equivalent to the value reported for the data element “TotalMethaneCarbonDioxideEquivalent” for the source type.

- The value to report for “TotalReportedNitrousCarbonDioxideEquivalent” for each source type is equivalent to the value reported for the data element “TotalNitrousCarbonDioxideEquivalent” for the source type.

- The total annual CO2, CH4 and N2O combined emissions in metric tons of CO2e for the applicable source type.  Report “0” if the facility did not have any sources of the applicable type subject to reporting under 98.232 during the reporting year.  Note:  Do not report for “Sub-Basin Selection”.

- The value to report for “TotalReportedCarbonDioxideEquivalent” for each source type is the sum of the values reported for the data elements “TotalReportedCarbonDioxideEmissions”, “TotalReportedMethaneCarbonDioxideEquivalent” and “TotalReportedNitrousCarbonDioxideEquivalent” for the source type.

Note:  Emissions data is required for each applicable source type as defined in the rule for each industry segment for which the facility is required to report.  Report "0" for emissions for an applicable source type if the facility did not have any sources of that type subject to reporting under 98.232 in the reporting year, or there were no emissions of a particular greenhouse gas, or a particular greenhouse gas is not applicable to the source type.

Subpart W Source Reporting Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| SubpartWSourceReportingFormDetails | Parent Element: A collection of data elements to report for applicable source types for the specified industry segment. |
| SubpartWSourceReportingFormRowDetails | Parent Element: A collection of data elements to report for sub-basin identification (for onshore petroleum and natural gas production only) and each applicable source type for the specified industry segment. Report data for each applicable source type separately. |
| ReportingCategory | The applicable source type or sub-basin identification (for Onshore petroleum and natural gas production only) for the specified industry segment. Please see Required Sources to Report by Industry Segment for source types required for each industry segment. See list of allowable values. Sub-Basin Selection Natural Gas Pneumatic Devices [98.236(c)(1)] Natural Gas Driven Pneumatic Pumps [98.236(c)(2)] Acid Gas Removal Units [98.236(c)(3)] Dehydrators [98.236(c)(4)] Well Venting for Liquids Unloading [98.236(c)(5)] Gas Well Completions and Workovers [98.236(c)(6)] Blowdown Vent Stacks [98.236(c)(7)] Gas from Produced Oil Sent to Atmospheric Tanks [98.236(c)(8)] Transmission Tanks [98.236(c)(9)] Well Testing Venting and Flaring [98.236(c)(10)] Associated Gas Venting and Flaring [98.236(c)(11)] Flare Stacks [98.236(c)(12)] Centrifugal Compressors [98.236(c)(13)] Reciprocating Compressors [98.236(c)(14)] Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)] Local Distribution Companies [98.236(c)(16)] Enhanced Oil Recovery Injection Pump Blowdown [98.236(c)(17)] Enhanced Oil Recovery Hydrocarbon Liquids Dissolved CO2 [98.236(c)(18)] Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)] Offshore Sources [98.236(b)] |
| RequiredForIndustrySegment | An indication that the source type is required for the industry segment. Report “Yes”. |
| TotalReportedCarbonDioxideEmissions | Total reported CO2 emissions in metric tons for the specified source type. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any sources of the specified type subject to reporting under 98.232 during the reporting year. Do not report for “Sub-Basin Selection”. |
| TotalReportedMethaneCarbonDioxideEquivalent | Total reported CH4 emissions in metric tons CO2e for the specified source type. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any sources of the specified type subject to reporting under 98.232 during the reporting year or if methane is not required to be reported for the specified source type. Do not report for “Sub-Basin Selection”. |
| TotalReportedNitrousCarbonDioxideEquivalent | Total reported N2O emissions in metric tons CO2e for the specified source type. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any sources of the specified type subject to reporting under 98.232 during the reporting year or if nitrous oxide is not required to be reported for the specified source type. Do not report for “Sub-Basin Selection”. |
| TotalReportedCarbonDioxideEquivalent | Total reported CO2, CH4 and N2O combined emissions in metric tons CO2e for the specified source type. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any sources of the specified type subject to reporting under 98.232 during the reporting year. Do not report for “Sub-Basin Selection”. |

Example for Subpart W Source Reporting Details

<ghg:SubpartWSourceReportingFormDetails>

<ghg:SubpartWSourceReportingFormRowDetails>

<ghg:ReportingCategory>Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]</ghg:ReportingCategory>

<ghg:RequiredForIndustrySegment>Yes</ghg:RequiredForIndustrySegment>

<ghg:TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.4</ghg:TotalReportedCarbonDioxideEmissions>

<ghg:TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">244.2</ghg:TotalReportedMethaneCarbonDioxideEquivalent>

<ghg:TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:TotalReportedNitrousCarbonDioxideEquivalent>

<ghg:TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">244.6</ghg:TotalReportedCarbonDioxideEquivalent>

</ghg:SubpartWSourceReportingFormRowDetails>

<ghg:SubpartWSourceReportingFormRowDetails>

<ghg:ReportingCategory>Local Distribution Companies [98.236(c)(16)]</ghg:ReportingCategory>

<ghg:RequiredForIndustrySegment>Yes</ghg:RequiredForIndustrySegment>

<ghg:TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">806.0</ghg:TotalReportedCarbonDioxideEmissions>

<ghg:TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">561904.0</ghg:TotalReportedMethaneCarbonDioxideEquivalent>

<ghg:TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:TotalReportedNitrousCarbonDioxideEquivalent>

<ghg:TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">562710</ghg:TotalReportedCarbonDioxideEquivalent>

</ghg:SubpartWSourceReportingFormRowDetails>

<ghg:SubpartWSourceReportingFormRowDetails>

<ghg:ReportingCategory>Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)]</ghg:ReportingCategory>

<ghg:RequiredForIndustrySegment>Yes</ghg:RequiredForIndustrySegment>

<ghg:TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">2.4</ghg:TotalReportedCarbonDioxideEmissions>

<ghg:TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">14.1</ghg:TotalReportedMethaneCarbonDioxideEquivalent>

<ghg:TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric Tons">11.3</ghg:TotalReportedNitrousCarbonDioxideEquivalent>

<ghg:TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">27.8</ghg:TotalReportedCarbonDioxideEquivalent>

</ghg:SubpartWSourceReportingFormRowDetails>

</ghg:SubpartWSourceReportingFormDetails>

</ghg:SubpartWSummaryDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Required Sources to Report by Industry Segment

|  |  | 1 Offshore Petroleum and Natural Gas Production [98.230(a)(1)] | 2 Onshore Petroleum and Natural Gas Production [98.230(a)(2)] | 3 Onshore Natural Gas Processing [98.230(a)(3)] | 4 Onshore Natural Gas Transmission Compression [98.230(a)(4)] | 5 Underground Natural Gas Storage [98.230(a)(5)] | 6 Liquefied Natural Gas (LNG) Storage [98.230(a)(6)] | 7 LNG Import and Export Equipment [98.230(a)(7)] | 8 Natural Gas Distribution [98.230(a)(8)] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0 | Sub-basin Selection / Onshore Requirements Under 98.236(e) |  | X |  |  |  |  |  |  |
| 4.0 | Natural Gas Pneumatic Devices [98.236(c)(1)] |  | X |  | X | X |  |  |  |
| 5.0 | Natural Gas Driven Pneumatic Pumps [98.236(c)(2)] |  | X |  |  |  |  |  |  |
| 6.0 | Acid Gas Removal Units [98.236(c)(3)] |  | X | X |  |  |  |  |  |
| 7.0 | Dehydrators [98.236(c)(4)] |  | X | X |  |  |  |  |  |
| 8.0 | Well Venting for Liquids Unloading [98.236(c)(5)] |  | X |  |  |  |  |  |  |
| 9.0 | Well Completions and Workovers [98.236(c)(6)] |  | X |  |  |  |  |  |  |
| 10.0 | Blowdown Vent Stacks [98.236(c)(7)] |  |  | X | X |  |  | X |  |
| 11.0 | Gas from Produced Oil Sent to Atmospheric Tanks [98.236(c)(8)] |  | X |  |  |  |  |  |  |
| 12.0 | Transmission Tanks [98.236(c)(9)] |  |  |  | X |  |  |  |  |
| 13.0 | Well Testing Venting and Flaring [98.236(c)(10)] |  | X |  |  |  |  |  |  |
| 14.0 | Associated Gas Venting and Flaring [98.236(c)(11)] |  | X |  |  |  |  |  |  |
| 15.0 | Flare Stacks [98.236(c)(12)] |  | X | X |  |  |  |  |  |
| 16.0 | Centrifugal Compressors [98.236(c)(13)] |  | X | X | X | X | X | X |  |
| 17.0 | Reciprocating Compressors [98.236(c)(14)] |  | X | X | X | X | X | X |  |
| 18.0 | Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)] |  | X | X | X | X | X | X | X |
| 19.0 | Local Distribution Companies [98.236(c)(16)] |  |  |  |  |  |  |  | X |
| 20.0 | Enhanced Oil Recovery (EOR) Injection Pump Blowdown [98.236(c)(17)] |  | X |  |  |  |  |  |  |
| 21.0 | Enhanced Oil Recovery (EOR) Hydrocarbon Liquids Dissolved CO2 [98.236(c)(18)] |  | X |  |  |  |  |  |  |
| 22.0 | Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)] |  | X |  |  |  |  |  | X |
| 23.0 | Offshore Sources [98.236(b)] | X |  |  |  |  |  |  |  |

### Offshore petroleum and natural gas production

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for the applicable source type in metric tons of CO2e.

The applicable source type is Offshore Sources [98.236(b)].

### Onshore petroleum and natural gas production

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e.  [98.236(c)]  The applicable source types are:

- Sub-basin Selection / Onshore Requirements Under 98.236(e)

- Natural Gas Pneumatic Devices [98.236(c)(1)]

- Natural Gas Driven Pneumatic Pumps [98.236(c)(2)]

- Acid Gas Removal Units [98.236(c)(3)]

- Dehydrators [98.236(c)(4)]

- Well Venting for Liquids Unloading [98.236(c)(5)]

- Well Completions and Workovers [98.236(c)(6)]

- Gas from Produced Oil Sent to Atmospheric Tanks [98.236(c)(8)]

- Well Testing Venting and Flaring [98.236(c)(10)]

- Associated Gas Venting and Flaring [98.236(c)(11)]

- Flare Stacks [98.236(c)(12)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

- Enhanced Oil Recovery (EOR) Injection Pump Blowdown [98.236(c)(17)]

- Enhanced Oil Recovery (EOR) Hydrocarbon Liquids Dissolved CO2 [98.236(c)(18)]

- Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)]

Note:  When a covered source type routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type.  [98.236(c)]

### Onshore natural gas processing

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Acid Gas Removal Units [98.236(c)(3)]

- Dehydrators [98.236(c)(4)]

- Blowdown Vent Stacks [98.236(c)(7)]

- Flare Stacks [98.236(c)(12)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

Note:  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit's respective majority use segment. When a source type listed below routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type. [98.236(c)]

### Onshore natural gas transmission compression

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Natural Gas Pneumatic Devices [98.236(c)(1)]

- Blowdown Vent Stacks [98.236(c)(7)]

- Transmission Tanks [98.236(c)(9)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

Note:  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit's respective majority use segment. When a source type listed below routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type. [98.236(c)]

### Underground natural gas storage

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Natural Gas Pneumatic Devices [98.236(c)(1)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

Note:  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit's respective majority use segment. When a source type listed below routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type. [98.236(c)]

### Liquefied natural gas (LNG) storage

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

Note:  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit's respective majority use segment. When a source type listed below routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type. [98.236(c)]

### LNG import and export equipment

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Blowdown Vent Stacks [98.236(c)(7)]

- Centrifugal Compressors [98.236(c)(13)]

- Reciprocating Compressors [98.236(c)(14)]

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

Note:  If a facility operates under more than one industry segment, each piece of equipment should be reported under the unit's respective majority use segment. When a source type listed below routes gas to flare, separately report the emissions that were vented directly to the atmosphere without flaring and the emissions that resulted from flaring the gas. Both the vented and flared emissions will be reported under the respective source type and not under the flare source type. [98.236(c)]

### Natural gas distribution

Report CO2, CH4 and N2O emissions totals and the total CO2e emissions for each applicable source type in metric tons of CO2e. [98.236(c)]  The applicable source types are:

- Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]

- Local Distribution Companies [98.236(c)(16)]

- Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions [98.236(c)(19)]

## Sub-Basin Selection / Onshore Requirements Under 98.236(e)

This topic provides a step-by-step description of how to report basin and applicable sub-basin identification for a facility and onshore requirements under 98.236(e) for each oil sub-basin category.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

### Basin Identification

“Basin” means geologic provinces as defined by the American Association of Petroleum Geologists (AAPG) Geologic Note: AAPG-CSD Geologic Provinces Code Map: AAPG Bulletin, Prepared by Richard F. Meyer, Laure G. Wallace and Fred J. Wagner, Jr., Volume 75, Number 10 (October 1991) (incorporated by reference, see 98.7) and the Alaska Geological Province Boundary Map, Compiled by the American Association of Petroleum Geologists Committee on Statistics of Drilling in Cooperation with the USGS, 1978 (incorporated by reference, see 98.7).  Please use the following naming convention to identify the basin:

Three Digit Basin Number - Basin Name

Example:  120 - Atlantic Coast Basin

See the following link for the list of basin numbers and names:

http://www.ccdsupport.com/confluence/display/help/Subpart+W+Basin+and+County+Combinations

EPA recognizes that onshore petroleum and natural gas producers, whose facility is a “basin”, are also likely not to have a traditional facility address. Because of the large geographic size of the basins, identifying latitude/longitude coordinates might not always be practical. As a result, these reporters may use the address of the U.S. corporate headquarters, including its street address, city, state and zip code. To ensure that multiple facilities (basins) that share the same corporate address can be uniquely identified, we ask that you specify in your facility name, the code and name of the basin as identified by the American Association of Petroleum Geologists (e.g., MLH Production Inc. 470 Fort Worth Syncline). Please note that changes to the facility name and street address must be manually entered into e-GGRT on the Facility Management page, not via XML.

Basin Identification Schema Diagram

Note: Data elements boxed in red are required.

Basin Identification Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| OnshoreRequirementsDetails | Parent Element: A collection of data elements to report for the basin. |
| SelectedBasin | The basin identifier. See Basin Identification for the required naming convention. |
| OnshoreProductionRequirementsDetails | Parent Element: A collection of data elements to report for the sub-basin. |

Example for Basin Identification

<ghg:OnshoreRequirementsDetails>

<ghg:SelectedBasin>160 - Appalachian Basin</ghg:SelectedBasin>

<ghg:OnshoreProductionRequirementsDetails>

<See example for Sub-Basin Identification / Onshore Requirements Details>

</ghg:OnshoreProductionRequirementsDetails>

</ghg:OnshoreRequirementsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

### Sub-Basin Identification

“Sub-basin category”, for onshore natural gas production, means a subdivision of a basin into the unique combination of wells with the surface coordinates within the boundaries of an individual county and subsurface completion in one or more of each of the following five formation types: oil, high permeability gas, shale gas, coal seam or other tight reservoir rock. The distinction between high permeability gas and tight gas reservoirs shall be designated as follows: High permeability gas reservoirs with >0.1 millidarcy permeability and tight gas reservoirs with ≤0.1 millidarcy permeability. Permeability for a reservoir type shall be determined by engineering estimate. Wells that produce from high permeability gas, shale gas, coal seam, or other tight reservoir rock are considered gas wells; gas wells producing from more than one of these formation types shall be classified into only one type based on the formation with the most contribution to production as determined by engineering knowledge. All wells that produce hydrocarbon liquids and do not meet the definition of a gas well in this sub-basin category definition are considered to be in the oil formation. All emission sources that handle condensate from gas wells in high permeability gas, shale gas, or tight reservoir rock formations are considered to be in the formation that the gas well belongs to and not in the oil formation. [98.238]

Emissions information for several of the applicable source types must be reported by sub-basin category.  Please use the following naming convention to identify each sub-basin:

Three Digit Basin Number - COUNTY, State abbreviation (County Number) - Formation Type

Example:  160 - ASHLAND, OH (5) - Oil

See the following link for the list of county names and numbers:

http://www.ccdsupport.com/confluence/display/help/Subpart+W+Basin+and+County+Combinations

The following are the formation types:

- Oil

- High permeability gas

- Shale gas

- Coal seam

- Other tight reservoir rock

Wells that produce from high permeability gas, shale gas, coal seam, or other tight reservoir rock are considered gas wells; gas wells producing from more than one of these formation types shall be classified into only one type based on the formation with the most contribution to production as determined by engineering knowledge. All wells that produce hydrocarbon liquids and do not meet the definition of a gas well in this sub-basin category definition are considered to be in the oil formation. All emission sources that handle condensate from gas wells in high permeability gas, shale gas, or tight reservoir rock formations are considered to be in the formation that the gas well belongs to and not in the oil formation.  [98.238]

Sub-Basin Identification / Onshore Requirements Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For each sub-basin category, report the following:

- County, state abbreviation and county number for the sub-basin location in the following format:

COUNTY, State abbreviation (County Number)

- Formation type for the sub-basin.

- Sub-basin identifier in the following format:

Three Digit Basin Number - COUNTY, State abbreviation (County Number) - Formation Type

For each oil sub-basin category only, report the following [98.236(e)]:

- The best available estimate of API gravity in degrees.

- The best available estimate of gas to oil ratio in cubic feet of gas per barrel of oil.

- The best available estimate of average low pressure separator pressure in psia.

Sub-Basin Identification / Onshore Requirements Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| OnshoreProductionRequirementsDetails | Parent Element: A collection of data elements to report for applicable sub-basins in the basin. |
| OnshoreProductionRequirementsSubBasinRowDetails | Parent Element: A collection of data elements to report for each applicable sub-basin category in the basin. |
| SubBasinCounty | The county, state abbreviation and county number in which the applicable sub-basin is located. See Sub-Basin Identification for the required naming convention. |
| SubBasinFormationType | The formation type of the sub-basin. See list of allowable values. Oil High permeability gas Shale gas Coal seam Other tight reservoir rock |
| SubBasinIdentifier | Sub-basin ID. See Sub-Basin Identification for the required naming convention. |
| ApiGravity | Conditionally Required: For each oil sub-basin category only, report the best available estimate of API gravity in degrees for the specified sub-basin category. [98.236(e)] Set the units of measure to “degrees” in the attribute densityUOM. |
| GasToOilRatio | Conditionally Required: For each oil sub-basin category only, report the best available estimate of gas-to-oil ratio in cubic feet of gas per barrel of oil for the specified sub-basin category. [98.236(e)] Set the units of measure to “cubic feet of gas per barrel of oil” in the attribute fractionUOM. |
| AveragePressure | Conditionally Required: For each oil sub-basin category only, report the best available estimate of average low pressure separator pressure in psia for the specified sub-basin category. [98.236(e)] Set the units of measure to “psia” in the attribute pressureUOM. |

Example for Sub-Basin Identification / Onshore Requirements Details

<ghg:OnshoreProductionRequirementsDetails>

<ghg:OnshoreProductionRequirementsSubBasinRowDetails>

<ghg:SubBasinCounty>ASHLAND, OH (5)</ghg:SubBasinCounty>

<ghg:SubBasinFormationType>Oil</ghg:SubBasinFormationType>

<ghg:SubBasinIdentifier>160 – ASHLAND, OH (5) - Oil</ghg:SubBasinIdentifier>

<ghg:ApiGravity densityUOM="degrees">17.257</ghg:ApiGravity>

<ghg:GasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">28.487</ghg:GasToOilRatio>

<ghg:AveragePressure pressureUOM="psia">46.864</ghg:AveragePressure>

</ghg:OnshoreProductionRequirementsSubBasinRowDetails>

</ghg:OnshoreProductionRequirementsDetails>

</ghg:OnshoreRequirementsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Natural Gas Pneumatic Device Venting

This topic provides a step-by-step description of how to report pneumatic device information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas transmission compression

- Underground natural gas storage

Pneumatic Device Venting Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For pneumatic devices, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all pneumatic devices combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the data element “TotalCarbonDioxideEmissions” for each pneumatic device type.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the data element “TotalMethaneCarbonDioxideEquivalent” for each pneumatic device type.

- The value to report for “TotalCarbonDioxideEquivalent” is the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether the facility had any continuous high-bleed pneumatic devices subject to reporting under 98.232 in the reporting year.

- Whether the facility had any intermittent-bleed pneumatic devices subject to reporting under 98.232 in the reporting year.

- Whether the facility had any continuous low-bleed pneumatic devices subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report for natural gas pneumatic devices subject to reporting under 98.232 in the reporting year.

Pneumatic Device Venting Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| PneumaticDeviceVentingDetails | Parent Element: A collection of data elements to report for natural gas pneumatic device venting. [98.236(c)(1)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all natural gas pneumatic device venting combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all natural gas pneumatic device venting combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total reported CO2 and CH4 combined emissions in metric tons CO2e for the specified source type. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveHighBleedDevices | Indicate (Yes/No) if the facility had any continuous high-bleed pneumatic devices subject to reporting under 98.232 in the reporting year. |
| DoesFacilityHaveIntermittentBleedDevices | Indicate (Yes/No) if the facility had any intermittent bleed pneumatic devices subject to reporting under 98.232 in the reporting year. |
| DoesFacilityHaveLowBleedDevices | Indicate (Yes/No) if the facility had any continuous low-bleed pneumatic devices subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| PneumaticDeviceTypesDetails | Parent Element: A collection of data elements to report for pneumatic devices. Conditionally Required: Only enter data elements for natural gas pneumatic devices subject to reporting under 98.232 in the reporting year (refer to Equation W-1 of 98.233). |

Example for Pneumatic Device Venting Details

<ghg:PneumaticDeviceVentingDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">4768.2</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">494299.7</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">499067.9</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveHighBleedDevices>Yes</ghg:DoesFacilityHaveHighBleedDevices>

<ghg:DoesFacilityHaveIntermittentBleedDevices>Yes</ghg:DoesFacilityHaveIntermittentBleedDevices>

<ghg:DoesFacilityHaveLowBleedDevices>Yes</ghg:DoesFacilityHaveLowBleedDevices>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:PneumaticDeviceTypesDetails>

<See example for Pneumatic Device Types Details>

</ghg:PneumaticDeviceTypesDetails>

</ghg:PneumaticDeviceVentingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Pneumatic Device Types Details Schema Diagram

Conditionally Required

Note: Data elements boxed in red are required.

For natural gas pneumatic devices subject to reporting under 98.232 in the reporting year (refer to Equation W-1 of 98.233), report the annual CO2 and CH4 emissions at the facility level, expressed in metric tons CO2e for each gas, for each of the following pieces of equipment, as applicable [98.236(c)(1)(iv)]:

- continuous high-bleed pneumatic devices

- intermittent bleed pneumatic devices

- continuous low-bleed pneumatic devices

Pneumatic Device Types Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| PneumaticDeviceTypesDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any pneumatic devices subject to reporting under 98.232 in the reporting year. |
| PneumaticDeviceTypesRowDetails | Parent Element: A collection of data elements to report for each type of pneumatic device subject to reporting under 98.232 in the reporting year. |
| PneumaticDeviceType | Type of pneumatic device subject to reporting under 98.232 in the reporting year. See list of allowable values. High-bleed Pneumatic Devices Intermittent Bleed Pneumatic Devices Low-Bleed Pneumatic Devices |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons from all pneumatic devices of the specified type. [98.236(c)(1)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e from all pneumatic devices of the specified type. [98.236(c)(1)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| ActualCount | Enter the actual count of the applicable natural gas pneumatic devices. [98.236(c)(1)] |
| EstimatedCount | Conditionally Required: This element is conditionally required in the first two reporting years for an onshore production facility if the facility has not determined an actual count of all pneumatic devices. Beginning in the third year of reporting for an onshore production facility, and for all facilities in transmission compression and underground natural gas storage segments, no estimates are allowed. Enter the estimated count of the applicable natural gas pneumatic devices. [98.236(c)(1)] |

Example for Pneumatic Device Types Details

<ghg:PneumaticDeviceTypesDetails>

<ghg:PneumaticDeviceTypesRowDetails>

<ghg:PneumaticDeviceType>High-bleed Pneumatic Devices</ghg:PneumaticDeviceType>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">3851.5</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">399272.7</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:ActualCount>23</ghg:ActualCount>

<ghg:EstimatedCount>24</ghg:EstimatedCount>

</ghg:PneumaticDeviceTypesRowDetails>

<ghg:PneumaticDeviceTypesRowDetails>

<ghg:PneumaticDeviceType>Intermittent Bleed Pneumatic Devices</ghg:PneumaticDeviceType>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">724.1</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">75063.3</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:ActualCount>23</ghg:ActualCount>

<ghg:EstimatedCount>24</ghg:EstimatedCount>

</ghg:PneumaticDeviceTypesRowDetails>

<ghg:PneumaticDeviceTypesRowDetails>

<ghg:PneumaticDeviceType>Low-Bleed Pneumatic Devices</ghg:PneumaticDeviceType>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">192.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">19963.7</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:ActualCount>22</ghg:ActualCount>

<ghg:EstimatedCount>22</ghg:EstimatedCount>	</ghg:PneumaticDeviceTypesRowDetails>

</ghg:PneumaticDeviceTypesDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for a facility in the onshore petroleum and natural gas production industry segment.  The “EstimatedCount” data element would be excluded for facilities in the onshore natural gas transmissions compression and underground natural gas storage industry segments.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Natural Gas Driven Pneumatic Pumps

This topic provides a step-by-step description of how to report natural gas driven pneumatic pumps information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Natural Gas Driven Pneumatic Pumps Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For natural gas driven pneumatic pumps, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all natural gas driven pneumatic pumps combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any natural gas driven pneumatic pumps subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the value reported for the data element “TotalCarbonDioxideEmissions” for natural gas driven pneumatic pumps.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” is the sum of the values reported for the data element “TotalMethaneCarbonDioxideEquivalent” for natural gas driven pneumatic pumps.

- The value to report for “TotalCarbonDioxideEquivalent” is the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether the facility had any natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.

- A collection of data elements for natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year (refer to Equation W-2 of 98.233),

Natural Gas Driven Pneumatic Pumps Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| NaturalGasDrivenPneumaticPumpsDetails | Parent Element: A collection of data elements to report for natural gas driven pneumatic pumps. [98.236(c)(2)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all natural gas driven pneumatic pumps combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all natural gas pneumatic pumps combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total reported CO2 and CH4 combined emissions in metric tons CO2e for the specified source type. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any pneumatic devices subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHavePneumaticPumps | Indicate (Yes/No) if the facility had any natural gas pneumatic pumps subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| PneumaticPumpTypesDetails | Parent Element: A collection of data elements to report for natural gas driven pneumatic pumps Conditionally Required: Only report data for natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year (refer to Equation W-2 of 98.233), |

Example for Natural Gas Pneumatic Pumps Types Details

<ghg:NaturalGasDrivenPneumaticPumpsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">281.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">29188.6</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">29470.2</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHavePneumaticPumps>Yes</ghg:DoesFacilityHavePneumaticPumps>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM used 2/1/2011 – 6/3/2011. Applies to the count of pumps.</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:PneumaticPumpTypesDetails>

<See example for Pneumatic Pumps Types Details>

</ghg:PneumaticPumpTypesDetails>

</ghg:NaturalGasDrivenPneumaticPumpsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Pneumatic Device Types Details Schema Diagram

Note: Data elements boxed in red are required.

For natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year (refer to Equation W-2 of 98.233), report annual CO2 and CH4 emissions at the facility level, expressed in metric tons CO2e for each gas, for all natural gas driven pneumatic pumps combined. [98.236(c)(2)(ii)]

Pneumatic Pump Types Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| PneumaticPumpTypesDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year. |
| PneumaticPumpTypesRowDetails | Parent Element: A collection of data elements to report for natural gas driven pneumatic pumps subject to reporting under 98.232 in the reporting year. |
| PneumaticDeviceType | The type of pneumatic pump: "Natural Gas Driven Pneumatic Pumps". |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons from all natural gas driven pneumatic pumps. [98.236(c )(1)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e from all pneumatic pumps of the specified type. [98.236(c )(1)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalPneumaticPumpCount | Enter the total count of natural gas driven pneumatic pumps [98.236(c)(2)(i)]. |

Example for Pneumatic Pumps Types Details

<ghg:PneumaticPumpTypesDetails>

<ghg:PneumaticPumpTypesRowDetails>

<ghg:PneumaticPumpType>Natural Gas Driven Pneumatic Pumps</ghg:PneumaticPumpType>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">281.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">29188.6</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalPneumaticPumpCount>5</ghg: TotalPneumaticPumpCount >

</ghg:PneumaticPumpTypesRowDetails>

</ghg:PneumaticPumpTypesDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Acid Gas Removal Units

This topic provides a step-by-step description of how to report acid gas removal unit information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

Acid Gas Removal Units Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For acid gas removal units, report the following:

- The CO2 emissions total and the total CO2e emissions for all acid gas removal units combined in metric tons of CO2e (total CO2 emissions and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any acid gas removal units subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the data element “TotalCarbonDioxideEmissions” for each acid gas removal unit.

- The value to report for “TotalCarbonDioxideEquivalent” equals the value reported for “TotalCarbonDioxideEmissions”.

- Whether the facility had any acid gas removal units subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements for each acid gas removal (AGR) unit subject to reporting under 98.232.

Acid Gas Removal Units Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AcidGasRemovalUnitsDetails | Parent Element: A collection of data elements to report for acid gas removal units. [98.236(c)(3)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all acid gas removal units combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any acid gas removal units subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total reported CO2 emissions for all acid gas removal units. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any acid gas removal units subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveAcidGasRemovalUnits | Indicate (Yes/No) if the facility had any acid gas removal units subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| AcidGasRemovalNumberOfUnitsDetails | Parent Element: A collection of data elements to report for acid gas removal units. [98.236(c)(3)] Conditionally Required: Enter data for each acid gas removal (AGR) unit subject to reporting under 98.232 (refer to 98.233(d)). |

Example for Acid Gas Removal Units Details

<ghg:AcidGasRemovalUnitsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM=" Metric Tons">1101876.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">1101876.6</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveAcidGasRemovalUnits>Yes</ghg:DoesFacilityHaveAcidGasRemovalUnits>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:AcidGasRemovalNumberOfUnitsDetails>

<See example for Acid Gas Removal Number of Units Details>

</ghg:AcidGasRemovalNumberOfUnitsDetails>

</ghg:AcidGasRemovalUnitsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Acid Gas Removal Number of Units Details Schema Diagram

Note:  Data elements boxed in red are required.  Please see page 4 of this document 
for more information on conditionally required elements.

For each acid gas removal (AGR) unit subject to reporting under 98.232 (refer to 98.233(d)), report the following:

- For the onshore petroleum and natural gas production industry segment only, a sequential identification number for the AGR unit.  Note:  This number is used for reference only in the event that there is a validation message associated with the AGR unit data.

- For the onshore natural gas processing industry segment only, a unique unit ID or name for the AGR unit. [98.236(c)(3)(vi)]

- An indication of which calculation methodology was used to calculate emissions for the AGR unit. [98.236(c)(3)(vii)]

- Calculation Methodology 1 (98.233(d)(1))

- Calculation Methodology 2 (98.233(d)(2))

- Calculation Methodology 3 (98.233(d)(3))

- Calculation Methodology 4 (98.233(d)(4))

- The annual CO2 emissions for the AGR unit, expressed in metric tons. [98.236(c)(3)(v)]

- If Calculation Methodology 1 or Calculation Methodology 2 was used, report the annual average volumetric fraction of CO2 content in the vent from the AGR unit.  [98.236(c)(3)(ii)]

- If Calculation Methodology 3 was used, report both the annual average volume fraction of CO2 content of natural gas into each acid gas removal unit and the annual average volume fraction of CO2 content of natural gas out of each acid gas removal unit

Acid Gas Removal Number of Units Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AcidGasRemovalNumberOfUnitsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any acid gas removal units subject to reporting under 98.232 in the reporting year. |
| AcidGasRemovalNumberOfUnitsRowDetails | Parent Element: A collection of data elements to report for each acid gas removal unit subject to 98.232 in the reporting year. |
| UniqueIdentifier | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, a unique ID for the acid gas removal unit. Please number units sequentially. |
| UnitIdentifier | Conditionally Required: For the onshore natural gas processing industry segment only, a unique unit ID or name for the acid gas removal unit. [98.236(c)(3)(vi)] |
| CalculationMethodology | The calculation methodology used to calculate CO2 emissions for the specified acid gas removal unit. See list of allowable values. [98.236(c)(3)(vii)] Calculation Methodology 1 (98.233(d)(1)) Calculation Methodology 2 (98.233(d)(2)) Calculation Methodology 3 (98.233(d)(3)) Calculation Methodology 4 (98.233(d)(4)) |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for the specified acid gas removal unit. [98.236(c)(3)(v)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AverageVentCarbonDioxideFraction | Conditionally Required: If Calculation Methodology 1 is used, then report the annual average volumetric fraction of CO2 content in the vent from the acid gas removal unit. [98.236(c)(3)(ii)] Set the units of measure to “decimal fraction” in the attribute fractionUOM. The value must fall between 0 and 1. |
| AverageVentCarbonDioxideFractionMethod2 | Conditionally Required: If Calculation Methodology 2 is used, then report the annual average volumetric fraction of CO2 content in the vent from the acid gas removal unit. [98.236(c)(3)(ii)] Set the units of measure to “decimal fraction” in the attribute fractionUOM. The value must fall between 0 and 1. |
| AverageNaturalGasCarbonDioxideFractionIn | Conditionally Required: If Calculation Methodology 3 is used, enter the annual average volume fraction of CO2 content of natural gas into each acid gas removal unit [98.236(c)(3)(iii)] |
| AverageNaturalGasCarbonDioxideFractionOut | Conditionally Required: If Calculation Methodology 3 is used, enter the annual average volume fraction of CO2 content of natural gas out of each acid gas removal unit [98.236(c)(3)(iii)] |
| TotalThroughput | Enter the total throughput off each acid gas removal unit using a meter or engineering estimate based on process knowledge or best available data 98.236(c)(3)(i) |

Example for Acid Gas Removal Number of Units Details

<ghg:AcidGasRemovalNumberOfUnitsDetails>

<ghg:AcidGasRemovalNumberOfUnitsRowDetails>

<ghg:UniqueIdentifier>1</ghg:UniqueIdentifier>

<ghg:CalculationMethodology>Calculation Methodology 2 (98.233(d)(2))</ghg:CalculationMethodology>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">367292.2</ghg:TotalCarbonDioxideEmissions>

<ghg:AverageVentCarbonDioxideFraction fractionUOM="decimal fraction">0.33</ghg:AverageVentCarbonDioxideFraction>

<ghg: TotalThroughput>1000</ghg: TotalThroughput>

<ghg: AnnualCarbonDioxideRecovered>1000</ghg: AnnualCarbonDioxideRecovered>

</ghg:AcidGasRemovalNumberOfUnitsRowDetails>

<ghg:AcidGasRemovalNumberOfUnitsRowDetails>

<ghg:UniqueIdentifier>2</ghg:UniqueIdentifier>

<ghg:CalculationMethodology>Calculation Methodology 1 (98.233(d)(1))</ghg:CalculationMethodology>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">734584.4</ghg:TotalCarbonDioxideEmissions>

<ghg:AverageVentCarbonDioxideFraction fractionUOM="decimal fraction">0.33</ghg:AverageVentCarbonDioxideFraction>

<ghg: TotalThroughput>1000</ghg: TotalThroughput>

</ghg:AcidGasRemovalNumberOfUnitsRowDetails>

</ghg:AcidGasRemovalNumberOfUnitsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Dehydrators

This topic provides a step-by-step description of how to report dehydrator information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

Dehydrators Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For dehydrators, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all dehydrators combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “VentingCarbonDioxideEmissions” for all glycol dehydrators with a throughput less than 0.4 MMscfd.

- “FlaringCarbonDioxideEmissions” for all glycol dehydrators with a throughput less than 0.4 MMscfd.

- “TotalCarbonDioxideEmissions” for all absorbent desiccant dehydrators.

- “VentingCarbonDioxideEmissions” for each glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd.

- “FlaringCarbonDioxideEmissions” for each glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “VentingMethaneCarbonDioxideEquivalent” for all glycol dehydrators with a throughput less than 0.4 MMscfd.

- “FlaringMethaneCarbonDioxideEquivalent” for all glycol dehydrators with a throughput less than 0.4 MMscfd.

- “TotalMethaneCarbonDioxideEquivalent” for all absorbent desiccant dehydrators.

- “VentingMethaneCarbonDioxideEquivalent” for each glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd.

- “FlaringMethaneCarbonDioxideEquivalent” for each glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “FlaringNitrousCarbonDioxideEquivalent” for all glycol dehydrators with a throughput less than 0.4 MMscfd.

- “FlaringNitrousCarbonDioxideEquivalent” for each glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd.

- The value to report for “TotalCarbonDioxideEquivalent” is the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- If the facility had any glycol dehydrators with a throughput less than 0.4 MMscfd (refer to Equation W-5 of 98.233), a collection of data elements.

- If the facility has any absorbent desiccant dehydrators (refer to Equation W-6 of 98.233), a collection of data elements.

- If the facility had any glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd (refer to 98.233), a collection of data elements,

Dehydrators Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| DehydratorsDetails | Parent Element: A collection of data elements to report for dehydrators. [98.236(c)(4)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all dehydrators combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all dehydrators combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions for all dehydrators combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions for all dehydrators combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any glycol or absorbent desiccant dehydrators subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveDehydrators | Indicate (Yes/No) if the facility had any dehydrators subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| GlycolDehydratorsLessThanSpecifiedValueDetails | Parent Element: A collection of data elements to report for dehydrators. [98.236(c)(4)(ii)] Conditionally Required: Enter specified data if the facility had any glycol dehydrators with a throughput less than 0.4 MMscfd (refer to Equation W-5 of 98.233) |
| AbsorbentDessicantDehydratorsDetails | Parent Element: A collection of data elements to report for dehydrators. [98.236(c)(4)(iii)] Conditionally Required: Enter specified data if the facility has any absorbent desiccant dehydrators (refer to Equation W-6 of 98.233) |
| GlycolDehydratorsMoreThanSpecifiedValueDetails | Parent Element: A collection of data elements to report for dehydrators. [98.236(c)(4)(i)] Conditionally Required: Enter specified data if the facility had any glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd (refer to 98.233), |

Example for Dehydrators Details

<ghg:DehydratorsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">3571.9</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">68314.2</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">6.1</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">71892.2</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveDehydrators>Yes</ghg:DoesFacilityHaveDehydrators>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:GlycolDehydratorsLessThanSpecifiedValueDetails>

<See example for Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details>

</ghg:GlycolDehydratorsLessThanSpecifiedValueDetails>

<ghg:AbsorbentDesiccantDehydratorsDetails>

<See example for Absorbent Desiccant Dehydrators Details>

</ghg:AbsorbentDesiccantDehydratorsDetails>

<ghg:GlycolDehydratorsMoreThanSpecifiedValueDetails>

<See example for Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Data >

</ghg:GlycolDehydratorsMoreThanSpecifiedValueDetails>

</ghg:DehydratorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

If the facility had any glycol dehydrators with a throughput less than 0.4 MMscfd (refer to 98.233, Equation W-5 of 98.233), report the following for all combined:

- Which vent gas controls are used (refer to 98.233(e)(3) and (e)(4)). [98.236(c)(4)(ii)(B)]

- Annual CO2 and CH4 emissions at the facility level that resulted from venting gas directly to the atmosphere, expressed in metric tons CO2e for each gas, combined for all glycol dehydrators with annual average daily throughput of less than 0.4 MMscfd. [98.236(c)(4)(ii)(C)]

- Annual CO2, CH4 and N2O emissions at the facility level that resulted from the flaring of process gas, expressed in metric tons CO2e for each gas, combined for all glycol dehydrators with annual average daily throughput of less than 0.4 MMscfd. [98.236(c)(4)(ii)(D)]  Note:  Emissions associated with regenerator fire-box/fire-tubes should be included here.

- The number of glycol dehydrators (refer to [98.236(c)(4)(ii)(A)].

Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| GlycolDehydratorsLessThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any glycol dehydrators with a throughput <0.4 MMscfd subject to reporting under 98.232 in the reporting year. |
| GlycolDehydratorsLessThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for glycol dehydrators with a throughput <0.4 MMscfd. |
| VentGasControlsUsed | The vent gas control used for all glycol dehydrators with a throughput <0.4 MMscfd. See list of allowable values. [98.236(c)(4)(ii)(B)] If the vent gas control used is not on the list or if more than one control was used, report "Other / Multiple Vent Gas Controls" and describe all vent gas controls used in the following data element. Vapor Recovery Dehydrator Vents to Flares Regenerator fire-box/fire tubes Other / Multiple Vent Gas Controls No Vent Controls |
| VentGasControlsOtherDescription | Conditionally Required: If "Other / Multiple Vent Gas Controls" was reported above, describe the gas vent control(s) used. [98.236(c)(4)(ii)(B)] |
| VentingCarbonDioxideEmissions | Total CO2 emissions from venting gas directly to the atmosphere in metric tons for all glycol dehydrators with a throughput <0.4 MMscfd which used the specified vent gas controls. [98.236(c)(4)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | Total CH4 emissions from venting gas directly to the atmosphere in metric tons CO2e for all glycol dehydrators with a throughput <0.4 MMscfd which used the specified vent gas controls. [98.236(c)(4)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | Total CO2 emissions from flaring of process gas in metric tons for all glycol dehydrators with a throughput <0.4 MMscfd which used the specified vent gas controls. Note: Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(ii)(D)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | Total CH4 emissions from flaring of process gas in metric tons CO2e for all glycol dehydrators with a throughput <0.4 MMscfd which used the specified vent gas controls. Note: Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(ii)(D)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | Total N2O emissions from flaring of process gas in metric tons CO2e for all glycol dehydrators with a throughput <0.4 MMscfd which used the specified vent gas controls. Note: Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(ii)(D)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| GlycolDehydratorCount | Enter the number of glycol dehydrators (see [98.236(c)(4)(ii)(A)]. |

Example for Glycol Dehydrator Throughput Less Than 0.4 MMscfd Details

<ghg:GlycolDehydratorsLessThanSpecifiedValueDetails>

<ghg:GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<ghg:VentGasControlsUsed>Vapor Recovery</ghg:VentGasControlsUsed>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">32.2</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">5803.0</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">303.1</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">116.1</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.6</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:GlycolDehydratorCount>3</ghg:GlycolDehydratorCount>

</ghg:GlycolDehydratorsLessThanSpecifiedValueRowDetails>

</ghg:GlycolDehydratorsLessThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Absorbent Desiccant Dehydrators Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

If the facility has any absorbent desiccant dehydrators (refer to Equation W-6 of 98.233), report the following for all combined:

- Total count of desiccant dehydrators. [98.236(c)(4)(iii)(A)]

- Annual CO2 and CH4 emissions at the facility level, expressed in metric tons CO2e for each gas, for all absorbent desiccant dehydrators combined. [98.236(c)(4)(iii)(B)]

Absorbent Desiccant Dehydrators Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AbsorbentDesiccantDehydratorsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any absorbent desiccant dehydrators subject to reporting under 98.232 in the reporting year. |
| AbsorbentDesiccantDehydratorsRowDetails | Parent Element: A collection of data elements to report for absorbent desiccant dehydrators. |
| DeviceType | The type of device [98.236(c)(4)(iii)]: "Absorbent Desiccant dehydrators" |
| AbsorbentDesiccantDehydrators | Total count of absorbent desiccant dehydrators. [98.236(c)(4)(iii)(A)] |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for all absorbent desiccant dehydrators. [98.236(c)(4)(iii)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e for all absorbent desiccant dehydrators. [98.236(c)(4)(iii)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Absorbent Desiccant Dehydrators Details

<ghg:AbsorbentDesiccantDehydratorsDetails>

<ghg:AbsorbentDesiccantDehydratorsRowDetails>

<ghg:DeviceType>Absorbent Desiccant dehydrators</ghg:DeviceType>

<ghg:AbsorbentDesiccantDehydrators>55</ghg:AbsorbentDesiccantDehydrators>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">32.2</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">5803.0</ghg:TotalMethaneCarbonDioxideEquivalent>

</ghg:AbsorbentDesiccantDehydratorsRowDetails>

</ghg:AbsorbentDesiccantDehydratorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Schema Diagram

Note: Data elements boxed in red are required. 
Please see page 4 of this document for more information on conditionally required elements.

If the facility had any glycol dehydrators with a throughput greater than or equal to 0.4 MMscfd (refer to 98.233(e)(1)), report the following for each:

- For the onshore petroleum and natural gas production industry segment only, report a sequential identification number for the dehydrator.  Note:  This number is used for reference only in the event that there is a validation message associated with the dehydrator data.

- For the onshore natural gas processing industry segment only, report a unique unit ID or name for the glycol dehydrator. [98.236(c)(4)(L)]

- Which vent gas controls are used (refer to 98.233(e)(3) and (e)(4)). [98.236(c)(4)(i)(I)]

- Annual CO2 and CH4 emissions that resulted from venting gas directly to the atmosphere, expressed in metric tons CO2e for each gas. [98.236(c)(4)(i)(J)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring process gas from the dehydrator, expressed in metric tons CO2e for each gas. [98.236(c)(4)(i)(K)]  Note:  Emissions associated with regenerator fire-box/fire-tubes should be included here.

- Glycol dehydrator feed natural gas flow rate in MMscfd, determined by engineering estimate based on best available data. [98.236(c)(4)(i)(A)]

- Glycol dehydrator absorbent circulation pump type. [98.236(c)(4)(i)(B)]

- Whether stripper gas is used in glycol dehydrator. [98.236(c)(4)(i)(C)]

- Whether a flash tank separator is used in glycol dehydrator. [98.236(c)(4)(i)(D)]

- Type of absorbent. [98.236(c)(4)(i)(E)]

- Total time the glycol dehydrator is operating in hours. [98.236(c)(4)(i)(F)]

- Temperature, in degrees Fahrenheit and pressure, in psig, of the wet natural gas. [98.236(c)(4)(i)(G)]

- Concentration of CH4 and CO2 in wet natural gas. [98.236(c)(4)(i)(H)]

Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| GlycolDehydratorsMoreThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any glycol dehydrators with a throughput >0.4 MMscfd subject to reporting under 98.232 in the reporting year. |
| GlycolDehydratorsMoreThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each glycol dehydrator with a throughput >0.4 MMscfd. |
| UniqueIdentifier | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, a unique ID for the glycol dehydrator with a throughput >0.4 MMscfd. Please number dehydrators sequentially. |
| UnitIdentifier | Conditionally Required: For the onshore natural gas processing industry segment only, a unique unit ID or name for the glycol dehydrator with a throughput >0.4 MMscfd. [98.236(c)(4)(i)(L)] |
| VentGasControlsUsed | The vent gas control used for the specified dehydrator. See list of allowable values. [98.236(c)(4)(i)(I)] If the vent gas control used is not on the list or if more than one control was used, report "Other / Multiple Vent Gas Controls" and describe all vent gas controls used in the following data element. Vapor Recovery Dehydrator Vents to Flares Regenerator fire-box/fire tubes Other / Multiple Vent Gas Controls No Vent Controls |
| VentGasControlsOtherDescription | Conditionally Required: If "Other / Multiple Vent Gas Controls" was reported above, describe the gas vent control(s) used. [98.236(c)(4)(i)(I)] |
| VentingCarbonDioxideEmissions | CO2 emissions from venting gas directly to the atmosphere in metric tons for the specified dehydrator. [98.236(c)(4)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting gas directly to the atmosphere in metric tons CO2e for the specified dehydrator. [98.236(c)(4)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring and other combustion in metric tons for the specified dehydrator. Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(i)(K)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring and other combustion in metric tons CO2e for the specified dehydrator. Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(i)(K)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring and other combustion in metric tons CO2e for the specified dehydrator. Emissions associated with regenerator fire-box/fire-tubes should be included here. [98.236(c)(4)(i)(K)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NaturalGasFlowRate | Glycol dehydrator feed natural gas flow rate in MMsct, determined by engineering estimate based on best available data 98.236[(c)(4)(i)(A)] |
| AbsorbentCirculationPumpType | Glycol dehydrator absorbent circulation pump type. See list of allowable values. [98.236(c)(4)(i)(B)]. Natural gas pneumatic Air pneumatic Electric |
| IsStripperGasUsed | Report whether stripper gas is used in glycol dehydrator [98.236(c)(4)(i)(C)] |
| IsFlashTankSeparatorUsed | Report whether a flash tank separator is used in glycol dehydrator [98.236(c)(4)(i)(D)] |
| AbsorbentType | Report type of absorbent. See list of allowable values. [98.236(c)(4)(i)(E)] Triethylene glycol (TEG) Diethylene glycol (DEG) Ethylene glycol (EG) |
| GlycolDehydratorOperatingHours | Total time the glycol dehydrator is operating in hours [98.236(c)(4)(i)(F)] |
| WetNaturalGasTemperature | Temperature of the wet natural gas (degrees Fahrenheit) [98.236(c)(4)(i)(G)] |
| WetNaturalGasPressure | Pressure of the wet natural gas (psig) [98.236(c)(4)(i)(G)] |
| WetNaturalGasCarbonDioxideConcentration | Concentration of CO2 in wet natural gas (as a fraction) [98.236(c)(4)(i)(H)] |
| WetNaturalGasCarbonMethaneConcentration | Concentration of CH4 in wet natural gas (as a fraction) [98.236(c)(4)(i)(H)] |

Example for Glycol Dehydrator Throughput More Than 0.4 MMscfd Details Data

<ghg:GlycolDehydratorsMoreThanSpecifiedValueDetails>

<ghg:GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<ghg:UnitIdentifier>001</ghg:UnitIdentifier>

<ghg:VentGasControlsUsed>Other</ghg:VentGasControlsUsed>

<ghg:VentGasControlsOtherDescription>Vapor Recovery, Regenerator fire-box/fire tubes</ghg:VentGasControlsOtherDescription>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">123.0</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">22192.9</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">1158.7</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">443.8</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">2.1</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:NaturalGasFlowRate>5</ghg:NaturalGasFlowRate>

<ghg:AbsorbentCirculationPumpType>Electric</ghg:AbsorbentCirculationPumpType>

<ghg:IsStripperGasUsed>No</ghg:IsStripperGasUsed>

<ghg:IsFlashTankSeparatorUsed>No</ghg:IsFlashTankSeparatorUsed>

<ghg:AbsorbentType>Ethylene glycol (EG)</ghg:AbsorbentType>

<ghg:GlycolDehydratorOperatingHours>1100</ghg:GlycolDehydratorOperatingHours>

<ghg:WetNaturalGasTemperature>69</ghg:WetNaturalGasTemperature>

<ghg:WetNaturalGasPressure>94</ghg:WetNaturalGasPressure>

<ghg:WetNaturalGasCarbonDioxideConcentration>.33</ghg:WetNaturalGasCarbonDioxideConcentration>

<ghg:WetNaturalGasCarbonMethaneConcentration>.11</ghg:WetNaturalGasCarbonMethaneConcentration>

</ghg:GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<ghg:GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<ghg:UnitIdentifier>002</ghg:UnitIdentifier>

<ghg:VentGasControlsUsed>Vapor Recovery</ghg:VentGasControlsUsed>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">184.6</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">33289.5</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">1738.1</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">665.9</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">3.4</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:NaturalGasFlowRate>5</ghg:NaturalGasFlowRate>

<ghg:AbsorbentCirculationPumpType>Electric</ghg:AbsorbentCirculationPumpType>

<ghg:IsStripperGasUsed>No</ghg:IsStripperGasUsed>

<ghg:IsFlashTankSeparatorUsed>No</ghg:IsFlashTankSeparatorUsed>

<ghg:AbsorbentType>Ethylene glycol (EG)</ghg:AbsorbentType>

<ghg:GlycolDehydratorOperatingHours>1200</ghg:GlycolDehydratorOperatingHours>

<ghg:WetNaturalGasTemperature>68</ghg:WetNaturalGasTemperature>

<ghg:WetNaturalGasPressure>95</ghg:WetNaturalGasPressure>

<ghg:WetNaturalGasCarbonDioxideConcentration>.34</ghg:WetNaturalGasCarbonDioxideConcentration>

<ghg:WetNaturalGasCarbonMethaneConcentration>.12</ghg:WetNaturalGasCarbonMethaneConcentration>

</ghg:GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

</ghg:GlycolDehydratorsMoreThanSpecifiedValueDetails>

</ghg:DehydratorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Well Venting for Liquids Unloading

This topic provides a step-by-step description of how to report well venting for liquids unloading information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Well Venting Details Schema Diagram

Note: Data elements boxed in red are required. 
Please see page 4 of this document for more information on conditionally required elements.

For well venting for liquids unloading, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all well ventings for liquids unloading combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any well venting for liquids unloading subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “TotalCarbonDioxideEmissions” using Calculation Methodology 1.

- “TotalCarbonDioxideEmissions” using Calculation Methodology 2.

- “TotalCarbonDioxideEmissions” using Calculation Methodology 3.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “TotalMethaneCarbonDioxideEquivalent” using Calculation Methodology 1.

- “TotalMethaneCarbonDioxideEquivalent” using Calculation Methodology 2.

- “TotalMethaneCarbonDioxideEquivalent” using Calculation Methodology 3.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether the facility had any well venting for liquids unloading subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements if Calculation Methodology 1 was used to calculate emissions (refer to Equation W-7 of 98.233). [98.236(c)(5)(i)]

- A collection of data elements if Calculation Methodology 2 was used to calculate emissions (refer to Equation W-8 of 98.233). [98.236(c)(5)(ii)]

- A collection of data elements if Calculation Methodology 3 was used to calculate emissions (refer to Equation W-9 of 98.233). [98.236(c)(5)(ii)]

Well Venting Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellVentingDetails | Parent Element: A collection of data elements to report for well venting for liquids unloading. [98.236(c)(5)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all well venting for liquids unloading combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well venting for liquids unloading subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all well venting for liquids unloading combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well venting for liquids unloading subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2 and CH4 emissions for all well venting for liquids unloading combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well venting for liquids unloading subject to reporting under 98.232 during the reporting year. |
| DidFacilityHaveWellVenting | Indicate (Yes/No) if the facility had any well venting for liquids unloading subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| WellVentingCalculationMethodOneDetails | Parent Element: A collection of data elements to report for calculation methodology 1. Conditionally Required: Enter data elements if Calculation Methodology 1 was used to calculate emissions (refer to Equation W-7 of 98.233). [98.236(c)(5)(i)] |
| WellVentingCalculationMethodTwoDetails | Parent Element: A collection of data elements to report for calculation methodology 2. Conditionally Required: Enter data elements if Calculation Methodology 2 was used to calculate emissions (refer to Equation W-8 of 98.233). [98.236(c)(5)(ii)] |
| WellVentingCalculationMethodThreeDetails | Parent Element: A collection of data elements to report for calculation methodology 3. Conditionally Required: Enter data elements if Calculation Methodology 3 was used to calculate emissions (refer to Equation W-9 of 98.233). [98.236(c)(5)(ii)] |

Example for Well Venting Details

<ghg:WellVentingDetails>

<ghg:TotalCarbonDioxideEmissions massUOM=" Metric Tons ">13799.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">13313873.4</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM=" Metric Tons ">13327673.0</ghg:TotalCarbonDioxideEquivalent>

<ghg:DidFacilityHaveWellVenting>Yes</ghg:DidFacilityHaveWellVenting>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:WellVentingCalculationMethodOneDetails>

<See example for Well Venting Calculation Methodology 1 Details>

</ghg:WellVentingCalculationMethodOneDetails>

<ghg:WellVentingCalculationMethodTwoDetails>

<See example for Well Venting Calculation Methodology 2 Details>

</ghg:WellVentingCalculationMethodTwoDetails>

<ghg:WellVentingCalculationMethodThreeDetails>

<See example for Well Venting Calculation Methodology 3 Details>

</ghg:WellVentingCalculationMethodThreeDetails>

</ghg:WellVentingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Well Venting Calculation Methodology 1 Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

If Calculation Methodology 1 was used to calculate emissions (refer to Equation W-7 of 98.233), report the following [98.236(c)(5)(i)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention)

- The identity of each tubing diameter group and pressure group combination within each sub-basin category.

- <1 inch, <25 psig

- <1 inch, >25 psig and <60 psig

- <1 inch, >60 psig and <110 psig

- <1 inch, >110 psig and <200 psig

- <1 inch, >200 psig

- >1 inch and <2.375 inches, <25 psig

- >1 inch and <2.375 inches, >25 psig and <60 psig

- >1 inch and <2.375 inches, >60 psig and <110 psig

- >1 inch and <2.375 inches, >110 psig and <200 psig

- >1 inch and <2.375 inches, >200 psig

- >2.375 inches, <25 psig

- >2.375 inches, >25 psig and <60 psig

- >2.375 inches, >60 psig and <110 psig

- >2.375 inches, >110 psig and <200 psig

- >2.375 inches, >200 psig

Note: Use the exact values referenced in the data element description table listed below.

For each tubing diameter group and pressure group combination within each sub-basin category, report the following [98.236(c)(5)(i)]:

- Number of wells vented to the atmosphere for liquids unloading. [98.236(c)(5)(i)(A)]

- Number of plunger lifts. [98.236(c)(5)(i)(B)]

- Cumulative number of unloadings vented to the atmosphere. [98.236(c)(5)(i)(C)]

- For the single well selected for installation of the recording flow meter, report the following:

- Whether the selected well from the tubing diameter and pressure group combination had a plunger lift. [98.236(c)(5)(i)(B)]

- If the well had a plunger lift, report the tubing pressure, in psia for the well. [98.236(c)(5)(i)(G)]

- If the well did not have a plunger lift, report the casing pressure, in psia, for the well. [98.236(c)(5)(i)(F)]

- If the well had a plunger lift, report the internal tubing diameter in inches. [98.236(c)(5)(i)(E)]

- If the well did not have a plunger lift, report the internal casing diameter in inches. [98.236(c)(5)(i)(E)]

- The well depth of the well, in feet. [98.236(c)(5)(i)(E)]

- The annual CO2 and CH4 emissions, expressed in metric tons CO2e for each gas. [98.236(c)(5)(i)(H)]

- Average flow rate of the measured well venting (cubic feet per hour).  [98.236(c)(5)(i)(D)]

Well Venting Calculation Methodology 1 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellVentingCalculationMethodOneDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins using Calculation Methodology 1 for well venting for liquids unloading subject to reporting under 98.232 in the reporting year. |
| WellVentingCalculationMethodOneRowDetails | Parent Element: A collection of data elements to report for each sub-basin and each tubing diameter group and pressure group combination using Calculation Methodology 1 for well venting for liquids unloading. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(5)] See Sub-Basin Identification for the required naming convention. |
| TubingDiameterAndPressure | The tubing diameter group and pressure group combination within the specified sub-basin category. See list of allowable values. [98.236(c)(5)(i)] &lt;1 inch, &lt;25 psig &lt;1 inch, &gt;25 psig and &lt;60 psig &lt;1 inch, &gt;60 psig and &lt;110 psig &lt;1 inch, &gt;110 psig and &lt;200 psig &lt;1 inch, &gt;200 psig &gt;1 inch and &lt;2.375 inches, &lt;25 psig &gt;1 inch and &lt;2.375 inches, &gt;25 psig and &lt;60 psig &gt;1 inch and &lt;2.375 inches, &gt;60 psig and &lt;110 psig &gt;1 inch and &lt;2.375 inches, &gt;110 psig and &lt;200 psig &gt;1 inch and &lt;2.375 inches, &gt;200 psig &gt;2.375 inches, &lt;25 psig &gt;2.375 inches, &gt;25 psig and &lt;60 psig &gt;2.375 inches, &gt;60 psig and &lt;110 psig &gt;2.375 inches, &gt;110 psig and &lt;200 psig &gt;2.375 inches, &gt;200 psig |
| NumberOfWells | Number of wells vented for liquids unloading in the specified tubing diameter group and pressure group combination within the specified sub-basin category. [98.236(c)(5)(i)(A)] |
| NumberOfPlungerLifts | Number of plunger lifts in the specified tubing diameter group and pressure group combination within the specified sub-basin category. [98.236(c)(5)(i)(B)] |
| NumberOfUnloadingsVented | Cumulative number of unloadings vented to the atmosphere for the specified tubing diameter group and pressure group combination within the specified sub-basin category. [98.236(c)(5)(i)(C)] |
| DidWellHavePlungerLift | Indicate (Yes/No) if the well selected for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category had a plunger lift. [98.236(c)(5)(i)(B)] |
| TubingPressure | Conditionally Required: If the well selected for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category had a plunger lift, report the tubing pressure in psia. [98.236(c)(5)(i)(G] Set the units of measure to “psia” in the attribute pressureUOM. |
| CasingPressure | Conditionally Required: If the well selected for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category did not have a plunger lift, report the casing pressure in psia. [98.236(c)(5)(i)(F)] Set the units of measure to “psia” in the attribute pressureUOM. |
| InternalTubingDiameter | Conditionally Required: If the well selected for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category had a plunger lift, report the internal tubing diameter in inches. [98.236(c)(5)(i)(E)] Set the units of measure to “inches” in the attribute heightUOM. |
| InternalCasingDiameter | Conditionally Required: If the well selected for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category did not have a plunger lift, report the internal casing diameter in inches. [98.236(c)(5)(i)(E)] Set the units of measure to “inches” in the attribute heightUOM. |
| WellDepth | Depth of the well for the installation of the recording flow meter from the specified tubing diameter and pressure group within the specified sub-basin category in feet. [98.236(c)(5)(i)(E)] Set the units of measure to “feet” in the attribute heightUOM. |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for the specified tubing diameter group and pressure group combination within the specified sub-basin category. [98.236(c)(5)(i)(H)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e for the specified tubing diameter group and pressure group combination within the specified sub-basin category. [98.236(c)(5)(i)(H)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AverageWellVentingFlowRate | Average flow rate of the measured well venting (cubic feet per hour). [98.236(c)(5)(i)(D)] |

Example for Well Venting Calculation Methodology 1 Details

<ghg:WellVentingCalculationMethodOneDetails>

<ghg:WellVentingCalculationMethodOneRowDetails>

<ghg:SubBasinIdentifier>360 – ALFALFA, OK (3) - Oil</ghg:SubBasinIdentifier>

<ghg:TubingDiameterAndPressure>&lt;1 inch, &gt;200 psig</ghg:TubingDiameterAndPressure>

<ghg:NumberOfWells>612</ghg:NumberOfWells>

<ghg:NumberOfPlungerLifts>692</ghg:NumberOfPlungerLifts>

<ghg:NumberOfUnloadingsVented>75720</ghg:NumberOfUnloadingsVented>

<ghg:DidWellHavePlungerLift>No</ghg:DidWellHavePlungerLift>

<ghg:CasingPressure pressureUOM="psia">102.34</ghg:CasingPressure>

<ghg:InternalCasingDiameter heightUOM="inches">5.45</ghg:InternalCasingDiameter>

<ghg:WellDepth heightUOM="feet">41461.5</ghg:WellDepth>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">369.3</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">356217.2</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:AverageWellVentingFlowRate>1400</ghg:AverageWellVentingFlowRate>

</ghg:WellVentingCalculationMethodOneRowDetails>

</ghg:WellVentingCalculationMethodOneDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Well Venting Calculation Methodology 2 Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

If Calculation Methodology 2 (without plunger lifts) was used to calculate emissions (refer to Equation W-8 of 98.233), report the following for each sub-basin category [98.236(c)(5)(ii)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- Number of wells vented to the atmosphere for liquids unloading (without plunger lifts). [98.236(c)(5)(ii)(A)]

- Average internal casing diameter, in inches, of all of the wells in the sub-basin category. [98.236(c)(5)(ii)(D)]

- The annual CO2 and CH4 emissions, expressed in metric tons CO2e for each GHG gas. [98.236(c)(5)(ii)(E)]

- Cumulative number of unloadings vented to the atmosphere. [98.236(c)(5)(ii)(C)]

Well Venting Calculation Methodology 2 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellVentingCalculationMethodTwoDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins using Calculation Methodology 2 (without plunger lifts) for well venting for liquids unloading subject to reporting under 98.232 in the reporting year. |
| WellVentingCalculationMethodTwoRowDetails | Parent Element: A collection of data elements to report for each sub-basin category using Calculation Methodology 2 (without plunger lifts) for well venting for liquids unloading. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(5)] See Sub-Basin Identification for the required naming convention. |
| NumberOfWellsVentedWithoutPlungerLifts | Number of wells vented for liquids unloading (without plunger lifts) in the specified sub-basin category. [98.236(c)(5)(ii)(A)] |
| AverageInternalCasingDiameter | The average internal casing diameter in inches of all the wells in the specified sub-basin category. [98.236(c)(5)(ii)(D)] Set the units of measure to “inches” in the attribute heightUOM. |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for the specified sub-basin category. [98.236(c)(5)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e for the specified sub-basin category. [98.236(c)(5)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CumulativeNumberOfUnloadingsVented | Cumulative number of unloadings vented to the atmosphere 98.236(c)(5)(ii)(C) |

Example for Well Venting Calculation Methodology 2 Details

<ghg:WellVentingCalculationMethodTwoDetails>

<ghg:WellVentingCalculationMethodTwoRowDetails>

<ghg:SubBasinIdentifier>30 – CADDO, OK (15) - High permeability gas</ghg:SubBasinIdentifier>

<ghg:NumberOfWellsVentedWithoutPlungerLifts>612</ghg:NumberOfWellsVentedWithoutPlungerLifts>

<ghg:AverageInternalCasingDiameter heightUOM="inches">7.23</ghg:AverageInternalCasingDiameter>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">6530.5</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">6300719.5</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:CumulativeNumberOfUnloadingsVented >31</ghg: CumulativeNumberOfUnloadingsVented>

</ghg:WellVentingCalculationMethodTwoRowDetails>

</ghg:WellVentingCalculationMethodTwoDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Well Venting Calculation Methodology 3 Details Schema Diagram

Note: Data elements boxed in red are required. 
Please see page 4 of this document for more information on conditionally required elements.

If Calculation Methodology 3 (with plunger lifts) was used to calculate emissions (refer to Equation W-9 of 98.233), report the following for each sub-basin category [98.236(c)(5)(ii)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- Number of wells vented to the atmosphere for liquids unloading (with plunger lifts). [98.236(c)(5)(ii)(A)]

- Number of plunger lifts. [98.236(c)(5)(ii)(B)]

- Average internal tubing diameter, in inches, of all of the wells in the sub-basin category. [98.236(c)(5)(ii)(D)]

- The annual CO2 and CH4 emissions, expressed in metric tons CO2e for each GHG gas. [98.236(c)(5)(ii)(E)]

- Cumulative number of unloadings vented to the atmosphere. [98.236(c)(5)(ii)(C)]

Well Venting Calculation Methodology 3 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellVentingCalculationMethodThreeDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins using Calculation Methodology 3 (with plunger lifts) for well venting for liquids unloading subject to reporting under 98.232 in the reporting year. |
| WellVentingCalculationMethodThreeRowDetails | Parent Element: A collection of data elements to report for each sub-basin category using Calculation Methodology 3 (with plunger lifts) for well venting for liquids unloading. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(5)] See Sub-Basin Identification for the required naming convention. |
| NumberOfWellsVentedWithPlungerLifts | Number of wells vented for liquids unloading (with plunger lifts) in the specified sub-basin category. [98.236(c)(5)(ii)(A)] |
| NumberOfPlungerLifts | The number of plunger lifts in the specified sub-basin category. [98.236(c)(5)(ii)(B)] |
| AverageInternalTubingDiameter | The average internal tubing diameter in inches of all the wells in the specified sub-basin category. [98.236(c)(5)(ii)(D)] Set the units of measure to “inches” in the attribute heightUOM. |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for the specified sub-basin category. [98.236(c)(5)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e for the specified sub-basin category. [98.236(c)(5)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CumulativeNumberOfUnloadingsVented | Cumulative number of unloadings vented to the atmosphere [98.236(c)(5)(ii)(c)] |

Example for Well Venting Calculation Methodology 3 Details

<ghg:WellVentingCalculationMethodThreeDetails>

<ghg:WellVentingCalculationMethodThreeRowDetails>

<ghg:SubBasinIdentifier>360 – DEWEY, OK (43) - High permeability gas</ghg:SubBasinIdentifier>

<ghg:NumberOfWellsVentedWithPlungerLifts>692</ghg:NumberOfWellsVentedWithPlungerLifts>

<ghg:NumberOfPlungerLifts>692</ghg:NumberOfPlungerLifts>

<ghg:AverageInternalTubingDiameter heightUOM="inches">4.53</ghg:AverageInternalTubingDiameter>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">369.3</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">356217.2</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:CumulativeNumberOfUnloadingsVented>31</ghg:CumulativeNumberOfUnloadingsVented>

</ghg:WellVentingCalculationMethodThreeRowDetails>

</ghg:WellVentingCalculationMethodThreeDetails>

</ghg:WellVentingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Gas Well Completions and Workovers

This topic provides a step-by-step description of how to report gas well completions and workovers information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Gas Well Completions and Workovers Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For gas well completions and workovers, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all gas well completions and workovers combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any gas well completions or workovers subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “CompletionsVentingCarbonDioxideEmissions” for well completions with hydraulic fracturing.

- “CompletionsFlaringCarbonDioxideEmissions” for well completions with hydraulic fracturing.

- “WorkoversVentingCarbonDioxideEmissions” for workovers with hydraulic fracturing.

- “WorkoversFlaringCarbonDioxideEmissions” for workovers with hydraulic fracturing.

- “VentingCarbonDioxideEmissions” for well completions without hydraulic fracturing.

- “FlaringCarbonDioxideEmissions” for well completions without hydraulic fracturing.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “CompletionsVentingMethaneCarbonDioxideEquivalent” for well completions with hydraulic fracturing.

- “CompletionsFlaringMethaneCarbonDioxideEquivalent” for well completions with hydraulic fracturing.

- “WorkoversVentingMethaneCarbonDioxideEquivalent” for workovers with hydraulic fracturing.

- “WorkoversFlaringMethaneCarbonDioxideEquivalent” for workovers with hydraulic fracturing.

- “VentingMethaneCarbonDioxideEquivalent” for well completions without hydraulic fracturing.

- “FlaringMethaneCarbonDioxideEquivalent” for well completions without hydraulic fracturing.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “CompletionsFlaringNitrousCarbonDioxideEquivalent” for completions with hydraulic fracturing.

- “WorkoversFlaringNitrousCarbonDioxideEquivalent” for workovers with hydraulic fracturing.

- “FlaringNitrousCarbonDioxideEquivalent” for well completions without hydraulic fracturing.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any gas well completions or workovers with hydraulic fracturing subject to reporting under 98.232 in the reporting year.

- Whether the facility had any gas well completions or workovers without hydraulic fracturing subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements if the facility performed any gas well completions and/or workovers with hydraulic fracturing during the reporting year. [98.236(6)(i)]

- A collection of data elements if the facility performed any gas well completions and/or workovers without hydraulic fracturing during the reporting year. [98.236(6)(ii)]

Gas Well Completions and Workovers Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellCompletionsWorkoversDetails | Parent Element: A collection of data elements to report for gas well completions and workovers. [98.236(c)(6)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all gas well completions and workovers combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas well completions or workovers subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all gas well completions and workovers combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas well completions or workovers subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions for all gas well completions and workovers combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas well completions or workovers subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions for all gas well completions and workovers combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas well completions or workovers subject to reporting under 98.232 during the reporting year. |
| DidFacilityHaveCompletionsWithHydraulic | Indicate (Yes/No) if the facility had any gas well completions or workovers WITH hydraulic fracturing subject to reporting under 98.232 in the reporting year. |
| DidFacilityHaveCompletionsWithoutHydraulic | Indicate (Yes/No) if the facility had any gas well completions or workovers WITHOUT hydraulic fracturing subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| WellCompletionsWorkoversWithHydraulicFracturingDetails | Parent Element: A collection of data elements to report for gas well completions and workovers. [98.236(c)(6)] Conditionally Required: If the facility performed any gas well completions and/or workovers with hydraulic fracturing during the reporting year, then enter data under this parent element. [98.236(6)(i)] |
| WellCompletionsWorkoversWithoutHydraulicFracturingDetails | Parent Element: A collection of data elements to report for gas well completions and workovers. [98.236(c)(6)] Conditionally Required: If the facility performed any gas well completions and/or workovers without hydraulic fracturing during the reporting year, then enter data under this parent element. [98.236(6)(ii)] |

Example for Gas Well Completions and Workovers Details

<ghg:WellCompletionsWorkoversDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">52548.8</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">291108.6</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">26.2</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">343684.0</ghg:TotalCarbonDioxideEquivalent>

<ghg:DidFacilityHaveCompletionsWithHydraulic>Yes</ghg:DidFacilityHaveCompletionsWithHydraulic>

<ghg:DidFacilityHaveCompletionsWithoutHydraulic>Yes</ghg:DidFacilityHaveCompletionsWithoutHydraulic>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:WellCompletionsWorkoversWithHydraulicFracturingDetails>

<See example for Well Completions and Workovers with Hydraulic Fracturing>

</ghg:WellCompletionsWorkoversWithHydraulicFracturingDetails>

<ghg:WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

<See example for Well Completions and Workovers without Hydraulic Fracturing>

</ghg:WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

</ghg:WellCompletionsWorkoversDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Gas Well Completions and Workovers with Hydraulic Fracturing Schema Diagram 
(Part 1 of 2)

Note: Data elements boxed in red are required.

Gas Well Completions and Workovers with Hydraulic Fracturing Schema Diagram 
(Part 2 of 2)

Note: Data elements boxed in red are required.
If the facility performed any gas well completions and/or workovers with hydraulic fracturing during the reporting year, then report the following [98.236(6)(i)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- The well type (horizontal or vertical).

- For each sub-basin category and well type combination (refer to Equation W-10A and W-10B of 98.233), report the following [98.236(6)(i)]:

- The equation used to calculate emissions from gas well completions with hydraulic fracturing. [98.236(c)(6)(i)]

- The equation used to calculate emissions from gas well workovers with hydraulic fracturing. [98.236(c)(6)(i)]

- Total count of all types of completions combined in the reporting year. [98.236(6)(i)(A)]

- Total count of well workovers in the reporting year that flare gas. [98.236(6)(i)(C)]

- Total count of well workovers in the reporting year that vent gas to the atmosphere. [98.236(6)(i)(C)]

- Annual CO2 and CH4 emissions that resulted from venting gas directly to the atmosphere, expressed in metric tons CO2e for each gas, from gas well completions. [98.236(6)(i)(I)]

- Annual CO2, CH4 and N2O emissions that resulted from flares, expressed in metric tons CO2e for each gas, from gas well completions. [98.236(6)(i)(J)]

- Annual CO2 and CH4 emissions that resulted from venting gas directly to the atmosphere, expressed in metric tons CO2e for each gas, from well workovers. [98.236(6)(i)(I)]

- Annual CO2, CH4 and N2O emissions that resulted from flares, expressed in metric tons CO2e for each gas, from well workovers. [98.236(6)(i)(J)]

- The number of gas well completions in the reporting year which employed purposely designed equipment that separates natural gas from the backflow. [98.236(6)(i)(G)]

- The number of well workovers in the reporting year which employed purposely designed equipment that separates natural gas from the backflow. [98.236(6)(i)(H)]

- If using Equation W-10A, specify if the only wells in the sub-basin are wildcat or delineation wells subject to a 2 Year delay in reporting (Yes or No). [98.236(c)(6)(i)]

- The API Well Number(s). [98.236(c)(6)(i)(G)]

- The measured flow rate of backflow during well completion (cubic feet per hour). [98.236(c)(6)(i)(B)]

- The measured flow rate of backflow during well workover (cubic feet per hour). [98.236(c)(6)(i)(D)]

- The total number of days of backflow from all wells during completions. [98.236(c)(6)(i)(E)]

- The total number of days of backflow from all wells during workovers. [98.236(c)(6)(i)(F)]

Gas Well Completions and Workovers with Hydraulic Fracturing 
Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellCompletionsWorkoversWithHydraulicFracturingDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any gas well completions or workovers with hydraulic fracturing subject to reporting under 98.232 in the reporting year. |
| WellCompletionsWorkoversWithHydraulicFracturingRowDetails | Parent Element: A collection of data elements to report for gas well completions and workovers with hydraulic fracturing for each sub-basin and well type combination. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(6)] See Sub-Basin Identification for the required naming convention. |
| WellType | Well type. See list of allowable values. [98.236(c)(6)(i)] Horizontal Vertical |
| CompletionsEmissionsEquationUsed | The equation used to calculate emissions from gas well completions with hydraulic fracturing for the specified sub-basin category and well type combination. [98.236(c)(6)(i)] See list of allowable values. Eq. W-10A and Estimated Flow Rates (based Eq. W-11A or Eq. W-11B) Eq. W-10A and Measured Flow Rates Eq. W-10A and Both Estimated and Measured Flow Rates Equation W-10B |
| WorkoversEmissionsEquationUsed | The equation used to calculate emissions from gas well workovers with hydraulic fracturing for the specified sub-basin category and well type combination. [98.236(c)(6)(i)] See list of allowable values. Eq. W-10A and Estimated Flow Rates (based Eq. W-11A or Eq. W-11B) Eq. W-10A and Measured Flow Rates Eq. W-10A and Both Estimated and Measured Flow Rates Equation W-10B |
| TotalCompletions | Total count of all types of completions combined for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(A)] |
| TotalFlareGasWorkovers | Total count of workovers that flare gas for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(C)] |
| TotalVentGasWorkovers | Total count of workovers that vent gas to the atmosphere for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(C)] |
| CompletionsVentingCarbonDioxideEmissions | CO2 emissions from venting for completions in metric tons for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompletionsVentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting for completions in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompletionsFlaringCarbonDioxideEmissions | CO2 emissions from flaring for completions in metric tons for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompletionsFlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring for completions in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompletionsFlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring for completions in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| WorkoversVentingCarbonDioxideEmissions | CO2 emissions from venting for workovers in metric tons for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| WorkoversVentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting for workovers in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| WorkoversFlaringCarbonDioxideEmissions | CO2 emissions from flaring for workovers in metric tons for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| WorkoversFlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring for workovers in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| WorkoversFlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring for workovers in metric tons CO2e for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| SeparatingCompletions | Number of gas well completions that employed purposely designed equipment that separates natural gas from the backflow for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(G)] |
| SeparatingWorkovers | Number of workovers that employed purposely designed equipment that separates natural gas from the backflow for the specified sub-basin category and well type combination. [98.236(c)(6)(i)(H)] |
| TwoYearDelayIndicator | Are the only wells in the sub-basin Wildcat or Delineation well subject to a 2 Year Delay in reporting [98.23-(c)(6)(i)] Yes No |
| APIWellNumbers | Conditionally Required: If the 2-year delay indicator is “yes” and the report is for reporting year RY2013 or later, enter the unique permanent identification number assigned to the well(s) in compliance with American Petroleum Institute well registration requirements no later than 2 years after the applicable annual report is due. [98.236(c)(6)(i)(G)] Note that API well numbers are not required in annual reports for reporting years 2011 and 2012. |
| CompletionBackflowFlowRate | Measured flow rate of backflow during well completion (cubic feet per hour). [98.236(c)(6)(i)(B)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| WorkoverBackflowFlowRate | Measured flow rate of backflow during well workover (cubic feet per hour). [98.236(c)(6)(i)(D)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| CompletionsNumberOfBackflowDays | Total number of days of backflow from all wells during completions. [98.236(c)(6)(i)(E)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| WorkoversNumberOfBackflowDays | Total number of days of backflow from all wells during workovers. [98.236(c)(6)(i)(F)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| CompletionsGasRecoveredToSales | The amount of gas recovered to sales using engineering estimate based on best available data. [98.236(c)(6)(i)(G)] |
| WorkoversGasRecoveredToSales | The amount of gas recovered to sales using engineering estimate based on best available data 98.236(c)(6)(i)(H) |

Example for Gas Well Completions and Workovers with Hydraulic Fracturing Details

<ghg:WellCompletionsWorkoversWithHydraulicFracturingDetails>

<ghg:WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<ghg:SubBasinIdentifier>360 – ALFALFA, OK (3) - Shale gas</ghg:SubBasinIdentifier>

<ghg:WellType>Horizontal</ghg:WellType>

<ghg:CompletionsEmissionsEquationUsed>Equation W-10B</ghg:CompletionsEmissionsEquationUsed>

<ghg:WorkoversEmissionsEquationUsed>Eq. W-10A and Measured Flow Rates</ghg:WorkoversEmissionsEquationUsed>

<ghg:TotalCompletions>76</ghg:TotalCompletions>

<ghg:TotalFlareGasWorkovers>11</ghg:TotalFlareGasWorkovers>

<ghg:TotalVentGasWorkovers>11</ghg:TotalVentGasWorkovers>

<ghg:CompletionsVentingCarbonDioxideEmissions massUOM="Metric Tons">248.8</ghg:CompletionsVentingCarbonDioxideEmissions>

<ghg:CompletionsVentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">239952.7</ghg:CompletionsVentingMethaneCarbonDioxideEquivalent>

<ghg:CompletionsFlaringCarbonDioxideEmissions massUOM="Metric Tons">43967.1</ghg:CompletionsFlaringCarbonDioxideEmissions>

<ghg:CompletionsFlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">4995.0</ghg:CompletionsFlaringMethaneCarbonDioxideEquivalent>

<ghg:CompletionsFlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">22.2</ghg:CompletionsFlaringNitrousCarbonDioxideEquivalent>

<ghg:WorkoversVentingCarbonDioxideEmissions massUOM="Metric Tons">46.8</ghg:WorkoversVentingCarbonDioxideEmissions>

<ghg:WorkoversVentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">45073.2</ghg:WorkoversVentingMethaneCarbonDioxideEquivalent>

<ghg:WorkoversFlaringCarbonDioxideEmissions massUOM="Metric Tons">8259.0</ghg:WorkoversFlaringCarbonDioxideEmissions>

<ghg:WorkoversFlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">938.3</ghg:WorkoversFlaringMethaneCarbonDioxideEquivalent>

<ghg:WorkoversFlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">4.2</ghg:WorkoversFlaringNitrousCarbonDioxideEquivalent>

<ghg:SeparatingCompletions>6</ghg:SeparatingCompletions>

<ghg:SeparatingWorkovers>4</ghg:SeparatingWorkovers>

<ghg:TwoYearDelayIndicator>No</ghg:TwoYearDelayIndicator>

<ghg:CompletionBackflowFlowRate>1400</ghg:CompletionBackflowFlowRate>

<ghg:WorkoverBackflowFlowRate>200</ghg:WorkoverBackflowFlowRate>

<ghg:CompletionsNumberOfBackflowDays>120</ghg:CompletionsNumberOfBackflowDays>

<ghg:WorkoversNumberOfBackflowDays>15</ghg:WorkoversNumberOfBackflowDays>

<ghg:CompletionsGasRecoveredToSales>15000000</ghg:CompletionsGasRecoveredToSales>

<ghg:WorkoversGasRecoveredToSales>10000</ghg:WorkoversGasRecoveredToSales>

</ghg:WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

</ghg:WellCompletionsWorkoversWithHydraulicFracturingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Gas Well Completions and Workovers without Hydraulic Fracturing 
Schema Diagram

Note: Data elements boxed in red are required.

If the facility performed any gas well completions and/or workovers without hydraulic fracturing during the reporting year, then report the following [98.236(c)(6)(ii)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- For each sub-basin category (refer to Equation W-13 of 98.233), report the following [98.236(c)(6)(ii)]:

- Total number of days of gas venting to the atmosphere during backflow for completion. [98.236(c)(6)(ii)(C)]

- Annual CO2 and CH4 emissions that resulted from venting for gas well completions and workovers, expressed in metric tons CO2e for each gas. [98.236(c)(6)(ii)(D)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring for gas well completions and workovers, expressed in metric tons CO2e for each gas. [98.236(c)(6)(ii)(E)]

- Total count of completions in calendar year. [98.236(c)(6)(ii)(A)]

- Total count of workovers in calendar year that flare gas or vent gas to the atmosphere. [98.236(c)(6)(ii)(B)]

Gas Well Completions and Workovers without Hydraulic Fracturing 
Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellCompletionsWorkoversWithoutHydraulicFracturingDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any gas well completions or workovers without hydraulic fracturing subject to reporting under 98.232 in the reporting year. |
| WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails | Parent Element: A collection of data elements to report for gas well completions and workovers without hydraulic fracturing for each sub-basin and well type combination. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(6)] See Sub-Basin Identification for the required naming convention. |
| TotalGasVentingDays | Total number of days of gas venting during backflow for completion for the specified sub-basin category. [98.236(c)(6)(ii)(C)] |
| VentingCarbonDioxideEmissions | CO2 emissions from venting for completions and workovers in metric tons for the specified sub-basin category. [98.236(c)(6)(ii)(D)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting for completions and workovers in metric tons CO2e for the specified sub-basin category. [98.236(c)(6)(ii)(D)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring for completions and workovers in metric tons for the specified sub-basin category. [98.236(c)(6)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring for completions and workovers in metric tons CO2e for the specified sub-basin category. [98.236(c)(6)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring for completions and workovers in metric tons CO2e for the specified sub-basin category. [98.236(c)(6)(ii)(E)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NumberOfCompletions | Total count of completions in calendar year. [98.236(c)(6)(ii)(A)] |
| NumberOfWorkoversThatFlareOrVentGas | Total count of workovers in calendar year that flare gas or vent gas to the atmosphere. [98.236(c)(6)(ii)(B)] |

Example for Gas Well Completions and Workovers without Hydraulic Fracturing

<ghg:WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

<ghg:WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<ghg:SubBasinIdentifier>360 – BACA, CO (9) - Shale gas</ghg:SubBasinIdentifier>

<ghg:TotalGasVentingDays>31</ghg:TotalGasVentingDays>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">72.0</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">13.2</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1.5</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.1</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:NumberOfCompletions>27</ghg:NumberOfCompletions>

<ghg:NumberOfWorkoversThatFlareOrVentGas>14</ghg:NumberOfWorkoversThatFlareOrVentGas>

</ghg:WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

</ghg:WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Blowdown Vent Stacks

This topic provides a step-by-step description of how to report blowdown vent stacks information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore natural gas processing

- Onshore natural gas transmission compression

- LNG import and export equipment

Blowdown Vent Stacks Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For blowdown vent stacks, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all blowdown vent stacks combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any blowdown vent stacks subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “TotalBlowdownCo2Emissions” for all unique volumes blown down only once during the reporting year.

- “TotalCarbonDioxideEmissions” for each unique physical volume blown down more than once in the reporting year.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “TotalBlowdownCh4Co2Equivalent” for all unique volumes blown down only one during the reporting year.

- “TotalMethaneCarbonDioxideEquivalent” for each unique physical volume blown down more than once in the reporting year.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether the facility had any blowdown vent stacks subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

For the blowdown vent stack emission source, (refer to Equation W-14A and Equation W-14B of 98.233), if there were any unique volumes that were blown down only once during the reporting year, report the following for all combined [98.236(c)(7)(ii)].  Note: Report “0” if the facility did not have any unique volumes that were blown down only once during the reporting year.

- Total number of unique physical volumes that were blown down only once during the reporting year. [98.236(c)(7)(ii)(A)]

- Annual CO2 and CH4 emissions from all unique physical volumes as an aggregate per facility, expressed in metric tons CO2e for each gas. [98.236(c)(7)(ii)(B)]

For each unique physical volume that is blown down more than once during the reporting year, a collection of data elements for the blowdown vent stack emission source, (refer to Equation W-14A and Equation W-14B of 98.233).

Blowdown Vent Stacks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| BlowdownVentStacksDetails | Parent Element: A collection of data elements to report for blowdown vent stacks. [98.236(c)(7)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all blowdown vent stacks combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any blowdown vent stacks subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all blowdown vent stacks combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any blowdown vent stacks subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2 and CH4 emissions from all blowdown vent stacks combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any blowdown vent stacks subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveBlowdownVentStacks | Indicate (Yes/No) if the facility had any blowdown vent stacks subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| TotalNumberOfBlowdowns | Conditionally Required: If there were any unique volumes that were blown down only once during the reporting year, report the total number of unique volumes that were blown down only once during the reporting year. [98.236(c)(7)(ii)(A)]. Report “0” if the facility did not have any unique volumes that were blown down only once during the reporting year. |
| TotalBlowdownCo2Emissions | Conditionally Required: If there were any unique volumes that were blown down only once during the reporting year, report the total CO2 emissions in metric tons from all unique volumes that were blown down only once during the reporting year. [98.236(c)(7)(ii)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. Report “0” if the facility did not have any unique volumes that were blown down only once during the reporting year. |
| TotalBlowdownCh4Co2Equivalent | Conditionally Required: If there were any unique volumes that were blown down only once during the reporting year, report the total CH4 emissions in metric tons CO2e from all unique volumes that were blown down only once during the reporting year. [98.236(c)(7)(ii)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. Report “0” if the facility did not have any unique volumes that were blown down only once during the reporting year. |
| BlowdownVentStacksForEachUniquePhysicalVolumeDetails | Parent Element: A collection of data elements to report for the blowdown vent stack emission source, (refer to Equation W-14A and Equation W-14B of 98.233), for each unique physical volume that is blown down more than once during the reporting year. |

Example for Blowdown Vent Stacks Details

<ghg:BlowdownVentStacksDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">19.1</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">13812.8</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">13831.9</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveBlowdownVentStacks>Yes</ghg:DoesFacilityHaveBlowdownVentStacks>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>Description of BAMM</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:TotalNumberOfBlowdowns>12</ghg:TotalNumberOfBlowdowns>

<ghg:TotalBlowdownCo2Emissions massUOM="Metric Tons">9.9</ghg:TotalBlowdownCo2Emissions>

<ghg:TotalBlowdownCh4Co2Equivalent massUOM="Metric Tons">6960.5</ghg:TotalBlowdownCh4Co2Equivalent>

<ghg:BlowdownVentStacksForEachUniquePhysicalVolumeDetails>

<See example for Blowdown Vent Stacks for Each Physical Volume Details>

</ghg:BlowdownVentStacksForEachUniquePhysicalVolumeDetails>

</ghg:BlowdownVentStacksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Blowdown Vent Stacks for Each Physical Volume Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For the blowdown vent stack emission source, (refer to Equation W-14A and Equation W-14B of 98.233), for each unique physical volume that is blown down more than once during the reporting year, report the following [98.236(c)(7)(i)]:

- A unique name or ID number for the unique physical volume. [98.236(c)(7)(i)(C)]

- The equation used to calculate emissions (Equation W-14A or Equation W-14B).

- The total number of blowdowns for the unique physical volume in the reporting year. [98.236(c)(7)(i)(A)]

- Annual CO2 and CH4 emissions for the unique physical blowdown volume, expressed in metric tons CO2e for each gas. [98.236(c)(7)(i)(B)]

Blowdown Vent Stacks for Each Physical Volume Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| BlowdownVentStacksForEachUniquePhysicalVolumeDetails | Parent Element (Conditionally Required): A collection of data elements to report if there were any physical volumes that were blown down more than once during the reporting year subject to reporting under 98.232. |
| BlowdownVentStacksForEachUniquePhysicalVolumeRowDetails | Parent Element: A collection of data elements to report for each unique physical volume that is blown down more than once during the reporting year. |
| PhysicalVolumeIdentifier | Unique name or ID for the unique physical volume that is blown down more than once during the reporting year. [98.236(c)(7)(i)(C)] |
| EmissionsEquationUsed | The equation used to calculate natural gas venting emissions for the specified unique physical volume. See list of allowable values. W-14A W-14B |
| TotalBlowdowns | Report the total number of blowdowns for the specified unique physical volume in the reporting year. [98.236(c)(7)(i)(A)] |
| TotalCarbonDioxideEmissions | Total CO2 emissions in metric tons for the specified unique physical volume. [98.236(c)(7)(i)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Total CH4 emissions in metric tons CO2e for the specified unique physical volume. [98.236(c)(7)(i)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Blowdown Vent Stacks for Each Physical Volume Details

<ghg:BlowdownVentStacksForEachUniquePhysicalVolumeDetails>

<ghg:BlowdownVentStacksForEachUniquePhysicalVolumeRowDetails>

<ghg:PhysicalVolumeIdentifier>001</ghg:PhysicalVolumeIdentifier>

<ghg:EmissionsEquationUsed>W-14B</ghg:EmissionsEquationUsed>

<ghg:TotalBlowdowns>10</ghg:TotalBlowdowns>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">9.2</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">6852.3</ghg:TotalMethaneCarbonDioxideEquivalent>

</ghg:BlowdownVentStacksForEachUniquePhysicalVolumeRowDetails>

</ghg:BlowdownVentStacksForEachUniquePhysicalVolumeDetails>

</ghg:BlowdownVentStacksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Gas from Produced Oil Sent to Atmospheric Tanks

This topic provides a step-by-step description of how to report gas from produced oil sent to atmospheric tanks information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Atmospheric Tanks Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For gas from produced oil sent to atmospheric tanks, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all gas emitted from produced oil sent to atmospheric tanks combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “VentingCarbonDioxideEmissions” using Calculation Methodology 1 or 2.

- “FlaringCarbonDioxideEmissions” using Calculation Methodology 1 or 2.

- “VentingCarbonDioxideEmissions” using Calculation Methodology 3 or 4.

- “FlaringCarbonDioxideEmissions” using Calculation Methodology 3 or 4.

- “VentingCarbonDioxideEmissions” using Calculation Methodology 5.

- “FlaringCarbonDioxideEmissions” using Calculation Methodology 5.

- “VentingCarbonDioxideEmissions” from improperly functioning dump valves.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “VentingMethaneCarbonDioxideEquivalent” using Calculation Methodology 1 or 2.

- “FlaringMethaneCarbonDioxideEquivalent” using Calculation Methodology 1 or 2.

- “VentingMethaneCarbonDioxideEquivalent” using Calculation Methodology 3 or 4.

- “FlaringMethaneCarbonDioxideEquivalent” using Calculation Methodology 3 or 4.

- “VentingMethaneCarbonDioxideEquivalent” using Calculation Methodology 5.

- “FlaringMethaneCarbonDioxideEquivalent” using Calculation Methodology 5.

- “VentingMethaneCarbonDioxideEquivalent” from improperly functioning dump valves.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “FlaringNitrousCarbonDioxideEquivalent” using Calculation Methodology 1 or 2.

- “FlaringNitrousCarbonDioxideEquivalent” using Calculation Methodology 3 or 4.

- “FlaringNitrousCarbonDioxideEquivalent” using Calculation Methodology 5.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- If gas was emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separators and wells with throughput less than 10 barrels per day, using Calculation Methodology 5 of 98.233(j) Equation W-15 of 98.233, report the following by facility/basin [98.236(c)(8)(iii)]:

- Total volume of oil production in barrels per year. [98.236(c)(8)(iii)(C)]

- Best estimate of fraction of production sent to tanks with assumed vapor recovery system control measures. [98.236(c)(8)(iii)(D)]

- Best estimate of fraction of production sent to tanks with assumed flaring of tank vapors control measures. [98.236(c)(8)(iii)(D)]

- Count of hydrocarbon tanks on well pads. [98.236(c)(8)(iii)(E)]

- If a wellhead separator dump valve is functioning improperly during the reporting year (refer to Equation W-16 of 98.233), report the following for the facility/basin [98.236(c)(8)(iv)]:

- Count of wellhead separators used in calculation methods 1, 2, and 4 that had improperly functioning dump valves. [98.236(c)(8)(iv)(A)]

- A collection of data elements for gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separator with oil throughput greater than or equal to 10 barrels per day, using Calculation Methodology 1 or 2 of 98.233(j).

- A collection of data elements for gas emitted from produced oil sent to atmospheric tanks for wells with oil production greater than or equal to 10 barrels per day, using Calculation Methodology 3 or 4 of 98.233(j).

- A collection of data elements for gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separators and wells with throughput less than 10 barrels per day, using Calculation Methodology 5 of 98.233(j) Equation W-15 of 98.233.

- A collection of data elements if the facility had any sub-basins with a wellhead separator dump valve functioning improperly during the reporting year.

Atmospheric Tanks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AtmosphericTanksDetails | Parent Element: A collection of data elements to report for gas from produced oil sent to atmospheric tanks. [98.236(c)(8)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all gas from produced oil sent to atmospheric tanks combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all gas from produced oil sent to atmospheric tanks combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions for all gas from produced oil sent to atmospheric tanks combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions for all gas from produced oil sent to atmospheric tanks combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 during the reporting year. |
| DidFacilityHaveGasFromProducedOil | Indicate (Yes/No) if the facility had any gas from produced oil sent to atmospheric tanks subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| CalculationMethodFiveTotalVolumeOfOilProduction | Conditionally Required: Report only if calculation method five was used. Total volume of oil production in barrels per year for the facility/basin. [98.236(c)(8)(iii)(C)] Set the units of measure to “barrels per year” in the attribute volUOM. |
| CalculationMethodFiveProductionFractionWithVaporRecovery | Conditionally Required: Report only if calculation method five was used. Fraction of production sent to tanks with assumed vapor recovery system control measures for the facility/basin. [98.236(c)(8)(iii)(D)] Set the units of measure to “fraction (number between 0 and 1)” in the attribute fractionUOM. |
| CalculationMethodFiveProductionFractionWithFlaring | Conditionally Required: Report only if calculation method five was used. Fraction of production sent to tanks with assumed flaring of tank vapors control measures for the facility/basin. [98.236(c)(8)(iii)(D)] Set the units of measure to “fraction (number between 0 and 1)” in the attribute fractionUOM. |
| CalculationMethodFiveTanksOnWellpads | Conditionally Required: Report only if calculation method five was used. Count of hydrocarbon tanks on wellpads for the facility/basin. [98.236(c)(8)(iii)(E)]. |
| ImproperlyFunctioningDumpValveWellheadSeparatorCount | Conditionally Required: Report only if facility had improperly functioning dump valve wellhead separators. Count of wellhead separators that dump valve factor is applied in the specified sub-basin. [98.236(c)(8)(iv)(A)]. |
| AtmosphericTanksCalculationMethodOneOrTwoDetails | Parent Element: A collection of data elements to report for gas from produced oil sent to atmospheric tanks by sub-basin category. [98.236(c)(8)] Conditionally Required: For gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separator with oil throughput greater than or equal to 10 barrels per day, using Calculation Methodology 1 or 2 of 98.233(j). |
| AtmosphericTanksCalculationMethodThreeOrFourDetails | Parent Element: A collection of data elements to report for gas from produced oil sent to atmospheric tanks by sub-basin category. [98.236(c)(8)] Conditionally Required: For gas emitted from produced oil sent to atmospheric tanks for wells with oil production greater than or equal to 10 barrels per day, using Calculation Methodology 3 or 4 of 98.233(j). |
| AtmosphericTanksCalculationMethodFiveDetails | Parent Element: A collection of data elements to report for gas from produced oil sent to atmospheric tanks by sub-basin category. [98.236(c)(8)] Conditionally Required: For gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separators and wells with throughput less than 10 barrels per day, using Calculation Methodology 5 of 98.233(j) Equation W-15 of 98.233. |
| AtmosphericTanksWellheadSeparatorDumpValveDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins with a wellhead separator dump valve functioning improperly during the reporting year. |

Example for Atmospheric Tanks Details

<ghg:AtmosphericTanksDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">1296690.4</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">2235231.3</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">400.3</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">3532322.0</ghg:TotalCarbonDioxideEquivalent>

<ghg:DidFacilityHaveGasFromProducedOil>Yes</ghg:DidFacilityHaveGasFromProducedOil>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:CalculationMethodFiveTotalVolumeOfOilProduction volUOM="barrels per year">472292</ghg:CalculationMethodFiveTotalVolumeOfOilProduction>

<ghg:CalculationMethodFiveProductionFractionWithVaporRecovery fractionUOM="fraction (number between 0 and 1)">0.5</ghg:CalculationMethodFiveProductionFractionWithVaporRecovery>

<ghg:CalculationMethodFiveProductionFractionWithFlaring fractionUOM="fraction (number between 0 and 1)">0.6</ghg:CalculationMethodFiveProductionFractionWithFlaring>

<ghg:TanksOnWellpads>210</ghg:TanksOnWellpads>

<ghg:ImproperlyFunctioningDumpValveWellheadSeparators>2656</ghg:improperlyFunctioningDumpValveWellheadSeparators>

<ghg:AtmosphericTanksCalculationMethodOneOrTwoDetails>

<See example for Atmospheric Tanks Calculation Methodology 1 or 2 Details>

</ghg:AtmosphericTanksCalculationMethodOneOrTwoDetails>

<ghg:AtmosphericTanksCalculationMethodThreeOrFourDetails>

<See example for Atmospheric Tanks Calculation Methodology 3 or 4 Details>

</ghg:AtmosphericTanksCalculationMethodThreeOrFourDetails>

<ghg:AtmosphericTanksCalculationMethodFiveDetails>

<See example for Atmospheric Tanks Calculation Methodology 5 Details>

</ghg:AtmosphericTanksCalculationMethodFiveDetails>

<ghg:AtmosphericTanksWellheadSeparatorDumpValveDetails>

<See example for Atmospheric Tanks Wellhead Separator Dump Valve Details>

</ghg:AtmosphericTanksWellheadSeparatorDumpValveDetails>

</ghg:AtmosphericTanksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Note: The facility should report emissions collectively. Reporters are not restricted to using only one calculation methodology per sub-basin and may use the requisite methods to report collective emissions, by sub-basin, for their facility.

Atmospheric Tanks Calculation Methodology 1 or 2 Details Schema Diagram
 (Part 1 of 2)

Note: Data elements boxed in red are required.

Atmospheric Tanks Calculation Methodology 1 or 2 Details Schema Diagram 
(Part 2 of 2)

Conditionally Required

Note: Data elements boxed in red are required.

For gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separator with oil throughput greater than or equal to 10 barrels per day, using Calculation Methodology 1 or 2 of 98.233(j), report the following by sub-basin category [98.236(c)(8)(i)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- The calculation methodology used to calculate emissions (Calculation Methodology 1 or Calculation Methodology 2) [98.236(c)(8)(i)]

- Number of wellhead separators sending oil to atmospheric tanks. [98.236(c)(8)(i)(A)]

- Estimated average separator temperature, in degrees Fahrenheit. [98.236(c)(8)(i)(B)]

- Estimated average separator pressure, in psig. [98.236(c)(8)(i)(B)]

- Estimated average sales oil stabilized API gravity, in degrees. [98.236(c)(8)(i)(C)]

- Count of hydrocarbon tanks at well pads. [98.236(c)(8)(i)(D)]

- Best estimate of count of stock tanks not at well pads receiving your oil. [98.236(c)(8)(i)(E)]

- Count of tanks with vapor recovery system emissions control measures for tanks at well pads. [98.236(c)(8)(i)(G)]

- Count of tanks with flaring emissions control measures for tanks at well pads. [98.236(c)(8)(i)(G)]

- Best estimate of count of stock tanks assumed to have emissions control measures not at well pads, receiving your oil. [98.236(c)(8)(i)(H)]

- Range of concentrations of CO2 in flash gas. [98.236(c)(8)(i)(I)]

- Minimum concentration of CO2 in flash gas (volumetric fraction)

- Maximum concentration of CO2 in flash gas (volumetric fraction)

- Range of concentrations of CH4 in flash gas.  [98.236(c)(8)(i)(I)]

- Minimum concentration of CH4 in flash gas (volumetric fraction)

- Maximum concentration of CH4 in flash gas (volumetric fraction)

- Annual CO2 and CH4 emissions that resulted from venting gas to the atmosphere, expressed in metric tons CO2e for each gas, for all wellhead gas-liquid separators or storage tanks using Calculation Methodology 1 or 2 of 98.233(j). [98.236(c)(8)(i)(J)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring gas, expressed in metric tons CO2e for each gas, for all wellhead gas-liquid separators or storage tanks using Calculation Methodology 1 or 2 of 98.233(j). [98.236(c)(8)(i)(L)]

- Are the only wells in the sub-basin Wildcat or Delineation wells subject to a 2-year delay in reporting (Yes or No). [98.236(c)(8)(i)(F)]

- If the 2-year delay indicator is “yes” and the report is for reporting year RY2013 or later, the API Well Number(s) for the well(s). [98.236(c)(8)(i)(F)].

- Total volume of oil from all wellhead separators sent to tank(s) (bbls per yr). [98.236(c)(8)(i)(F) ]

- Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(i)(K)]

- Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(i)(K)]

Atmospheric Tanks Calculation Methodology 1 or 2 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AtmosphericTanksCalculationMethodOneOrTwoDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins with wellhead gas-liquid separators with oil throughput >10 barrels/day using Calculation Methodologies 1 or 2 subject to reporting under 98.232 in the reporting year. |
| AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails | Parent Element: A collection of data elements to report for each sub-basin with wellhead gas-liquid separator with oil throughput >10 barrels/day using Calculation Methodologies 1 or 2. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(8)(i)] See Sub-Basin Identification for the required naming convention. |
| CalculationMethodology | Calculation methodology used to calculate emissions. See list of allowable values. [98.236(c)(8)(i)] Calculation Methodology 1 Calculation Methodology 2 |
| NumberOfWellHeadSeparators | Number of wellhead separators sending oil to atmospheric tanks for the specified sub-basin. [98.236(c)(8)(i)(A)] |
| AverageSeparatorTemperature | Estimated average separator temperature in degrees Fahrenheit for the specified sub-basin. [98.236(c)(8)(i)(B)] Set the units of measure to “Fahrenheit” in the attribute tempUOM. |
| AveragePressure | Estimated average separator pressure in psig for the specified sub-basin. [98.236(c)(8)(i)(B)] Set the units of measure to “psig” in the attribute pressureUOM. |
| AverageAPIGravity | Estimated average sales oil stabilized API gravity in degrees for the specified sub-basin. [98.236(c)(8)(i)(C)] Set the units of measure to “degrees” in the attribute densityUOM. |
| HydrocarbonTanks | Count of hydrocarbon tanks at well pads in the specified sub-basin. [98.236(c)(8)(i)(D)] |
| StockTanks | Best estimate of count of stock tanks not at well pads receiving your oil in the specified sub-basin. [98.236(c)(8)(i)(E)] |
| TanksWithVaporRecovery | Count of tanks with vapor recovery system emission control measures at well pads for the specified sub-basin. [98.236(c)(8)(i)(G)] |
| TanksWithFlaring | Count of tanks with flaring emission control measures at well pads in the specified sub-basin. [98.236(c)(8)(i)(G)] |
| TanksWithEmissionControl | Best estimate of count of stock tanks assumed to have emission control measures, not at well pads, receiving your oil in the specified sub-basin. [98.236(c)(8)(i)(H)] |
| MinimumFlashMethaneConcentration | Minimum concentration of CH4 in flash gas as a volumetric fraction in the specified sub-basin. [98.236(c)(8)(i)(I)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MaximumFlashMethaneConcentration | Maximum concentration of CH4 in flash gas as a volumetric fraction in the specified sub-basin. [98.236(c)(8)(i)(I)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MinimumFlashGasCarbonDioxideConcentration | Minimum concentration of CO2 in flash gas as a volumetric fraction in the specified sub-basin. [98.236(c)(8)(i)(I)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MaximumFlashGasCarbonDioxideConcentration | Maximum concentration of CO2 in flash gas as a volumetric fraction in the specified sub-basin. [98.236(c)(8)(i)(I)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| VentingCarbonDioxideEmissions | CO2 emissions from venting in metric tons in the specified sub-basin. [98.236(c)(8)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting in metric tons CO2e in the specified sub-basin. [98.236(c)(8)(i)(J)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring in metric tons in the specified sub-basin. [98.236(c)(8)(i)(L)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring in metric tons CO2e in the specified sub-basin. [98.236(c)(8)(i)(L)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring in metric tons CO2e in the specified sub-basin. [98.236(c)(8)(i)(L)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TwoYearDelayIndicator | Specify whether the only wells in the sub-basin are Wildcat or Delineation wells subject to a 2 year delay in reporting [98.236(c)(8)(i)(F)] Yes No |
| APIWellNumbers | Conditionally Required: If the 2-year delay indicator is “yes” and the report is for reporting year RY2013 or later, enter the unique permanent identification number assigned to the well(s) in compliance with American Petroleum Institute well registration requirements no later than 2 years after the applicable annual report is due. [98.236(c)(8)(i)(F)] |
| TotalVolumeOfOil | Total volume of oil from all wellhead separators sent to tank(s) (bbls per yr) [98.236(c)(8)(i)(F)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| AnnualCarbonDioxideRecovered | Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(i)(K)] |
| AnnualMethaneRecovered | Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(i)(K)] |

Example for Atmospheric Tanks Calculation Methodology 1 or 2 Details

<ghg:AtmosphericTanksCalculationMethodOneOrTwoDetails>

<ghg:AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - ALFALFA, OK (3) - Oil</ghg:SubBasinIdentifier>

<ghg:CalculationMethodology>Calculation Methodology 1</ghg:CalculationMethodology>

<ghg:NumberOfWellHeadSeparators>2656</ghg:NumberOfWellHeadSeparators>

<ghg:AverageSeparatorTemperature tempUOM="Fahrenheit">80</ghg:AverageSeparatorTemperature>

<ghg:AveragePressure pressureUOM="psig">870</ghg:AveragePressure>

<ghg:AverageAPIGravity densityUOM="degrees">68</ghg:AverageAPIGravity>

<ghg:HydrocarbonTanks>1744</ghg:HydrocarbonTanks>

<ghg:StockTanks>1744</ghg:StockTanks>

<ghg:TanksWithVaporRecovery>872</ghg:TanksWithVaporRecovery>

<ghg:TanksWithFlaring>872</ghg:TanksWithFlaring>

<ghg:TanksWithEmissionControl>872</ghg:TanksWithEmissionControl>

<ghg:MinimumFlashMethaneConcentration fractionUOM="volumetric fraction">0.7</ghg:MinimumFlashMethaneConcentration>

<ghg:MaximumFlashMethaneConcentration fractionUOM="volumetric fraction">0.82</ghg:MaximumFlashMethaneConcentration>

<ghg:MinimumFlashGasCarbonDioxideConcentration fractionUOM="volumetric fraction">0.8</ghg:MinimumFlashGasCarbonDioxideConcentration>

<ghg:MaximumFlashGasCarbonDioxideConcentration fractionUOM="volumetric fraction">0.93</ghg:MaximumFlashGasCarbonDioxideConcentration>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">21680.5</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">724714.1</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">626013</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">14494.3</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">199.8</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:TwoYearDelayIndicator>No</ghg:TwoYearDelayIndicator>

<ghg:TotalVolumeOfOil>5.4</ghg:TotalVolumeOfOil>

<ghg:AnnualCarbonDioxideRecovered>25000</ghg:AnnualCarbonDioxideRecovered>

<ghg:AnnualMethaneRecovered>7000</ghg:AnnualMethaneRecovered>

</ghg:AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

</ghg:AtmosphericTanksCalculationMethodOneOrTwoDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Atmospheric Tanks Calculation Methodology 3 or 4 Details Schema Diagram

Conditionally Required

Note: Data elements boxed in red are required.

For gas emitted from produced oil sent to atmospheric tanks for wells with oil production greater than or equal to 10 barrels per day, using Calculation Methodology 3 or 4 of 98.233(j), report the following by sub-basin category [98.236(c)(8)(ii)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- The calculation methodology used to calculate emissions. [98.236(c)(8)(ii)]

- Total number of wells sending oil directly to tanks. [98.236(c)(8)(ii)(B)]

- Total number of wells sending oil to separators off the well pads. [98.236(c)(8)(ii)(C)]

- Sales oil API gravity range for wells sending oil directly to tanks and wells sending oil to separators off the well pads, in degrees. [98.236(c)(8)(ii)(D)]

- Minimum sales oil API Gravity for wells in 98.236(c)(8)(ii)(B) and 98.236(c)(8)(ii)(C) (degrees)

- Maximum sales oil API Gravity for wells in 98.236(c)(8)(ii)(B) and 98.236(c)(8)(ii)(C) (degrees)

- Count of hydrocarbon tanks on wellpads. [98.236(c)(8)(ii)(E)]

- Count of hydrocarbon tanks on well pads assumed to have vapor recovery system emissions control measures. [98.236(c)(8)(ii)(F)]

- Count of hydrocarbon tanks on well pads assumed to have flaring of tank vapors emissions control measures. [98.236(c)(8)(ii)(F)]

- Count of hydrocarbon tanks off well pads assumed to have vapor recovery system emissions control measures. [98.236(c)(8)(ii)(F)]

- Count of hydrocarbon tanks off well pads assumed to have flaring of tank vapors emissions control measures. [98.236(c)(8)(ii)(F)]

- Annual CO2 and CH4 emissions that resulted from venting gas to the atmosphere, expressed in metric tons CO2e for each gas, at the sub-basin level for Calculation Methodology 3 or 4 of 98.233(j). [98.236(c)(8)(ii)(G)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring gas, expressed in metric tons CO2e for each gas, at the sub-basin level for Calculation Methodology 3 or 4 of 98.233(j). [98.236(c)(8)(ii)(I)]

- Are the only wells in the sub-basin Wildcat or Delineation wells subject to a 2-year delay in reporting (Yes or No). [98.236(c)(8)(ii)(A)]

- If the 2-year delay indicator is “yes” and the report is for reporting year RY2013 or later, the API Well Number(s) for the well(s). [98.236(c)(8)(ii)(A)]

- Total volume of sales oil from all wells (bbls per yr). [98.236(c)(8)(ii)(A)]

- Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(ii)(H)]

- Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(ii)(H)]

Atmospheric Tanks Calculation Methodology 3 or 4 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AtmosphericTanksCalculationMethodThreeOrFourDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins with wellhead gas-liquid separators with oil throughput >10 barrels/day using Calculation Methodologies 3 or 4 subject to reporting under 98.232 in the reporting year. |
| AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails | Parent Element: A collection of data elements to report for each sub-basin with wellhead gas-liquid separator with oil throughput >10 barrels/day using Calculation Methodologies 3 or 4. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(8)(ii)] See Sub-Basin Identification for the required naming convention. |
| CalculationMethodology | Calculation methodology used to calculate emissions for the specified sub-basin. See list of allowable values. [98.236(c)(8)(ii)] Calculation Methodology 3 Calculation Methodology 4 |
| WellsSendingOilToTanks | Total number of wells sending oil directly to tanks in the specified sub-basin. [98.236(c)(8)(ii)(B)] |
| WellsSendingOilToSeparators | Total number of wells sending oil to separators off the wellpads in the specified sub-basin. [98.236(c)(8)(ii)(C)] |
| MinimumAPIGravity | Minimum sales oil API gravity for wells in 98.236(c)(8)(ii)(B) and 98.236(c)(8)(ii)(C) for the specified sub-basin in degrees. [98.236(c)(8)(ii)(D)] Set the units of measure to “degrees” in the attribute densityUOM. |
| MaximumAPIGravity | Maximum sales oil API gravity for wells in 98.236(c)(8)(ii)(B) and 98.236(c)(8)(ii)(C) for the specified sub-basin in degrees. [98.236(c)(8)(ii)(D)] Set the units of measure to “degrees” in the attribute densityUOM. |
| TanksOnWellpads | Count of hydrocarbon tanks on wellpads in the specified sub-basin. [98.236(c)(8)(ii)(E)] |
| TanksOnWellpadsWithVaporRecovery | Count of hydrocarbon tanks on wellpads assumed to have vapor recovery system emission control measures in the specified sub-basin. [98.236(c)(8)(ii)(F)] |
| TanksOnWellpadsWithFlaring | Count of hydrocarbon tanks on wellpads assumed to have flaring of tank vapors emission control measures in the specified sub-basin. [98.236(c)(8)(ii)(F)] |
| TanksOffWellpadsWithVaporRecovery | Count of hydrocarbon tanks off wellpads assumed to have vapor recovery system emission control measures in the specified sub-basin. [98.236(c)(8)(ii)(F)] |
| TanksOffWellpadsWithFlaring | Count of hydrocarbon tanks off wellpads assumed to have flaring of tank vapors emission control measures in the specified sub-basin. [98.236(c)(8)(ii)(F)] |
| VentingCarbonDioxideEmissions | CO2 emissions from venting in the specified sub-basin in metric tons. [98.236(c)(8)(ii)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting in the specified sub-basin in metric tons CO2e. [98.236(c)(8)(ii)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring in the specified sub-basin in metric tons. [98.236(c)(8)(ii)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring in the specified sub-basin in metric tons CO2e. [98.236(c)(8)(ii)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring in the specified sub-basin in metric tons CO2e. [98.236(c)(8)(ii)(I)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TwoYearDelayIndicator | Specify whether the only wells in the sub-basin are Wildcat or Delineation wells subject to a 2 year delay in reporting [98.236(c)(8)(i)(F)] Yes No |
| APIWellNumbers | Conditionally Required: If the 2-year delay indicator is “yes” and the report is for reporting year RY2013 or later, enter the unique permanent identification number assigned to the well(s) in compliance with American Petroleum Institute well registration requirements no later than 2 years after the applicable annual report is due. [98.236(c)(8)(ii)(A)] |
| TotalVolumeOfSalesOil | Total volume of sales oil from all wells (bbls per yr). [98.236(c)(8)(ii)(A)] Note that if the 2-year delay indicator is “yes”, then this data element must be reported no later than 2 years after the due date for the applicable annual report. |
| AnnualCarbonDioxideRecovered | Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(ii)(H)] |
| AnnualMethaneRecovered | Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(ii)(H)] |

Example for Atmospheric Tanks Calculation Methodology 3 or 4 Details

<ghg:AtmosphericTanksCalculationMethodThreeOrFourDetails>

<ghg:AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - BACA, CO (9) - Shale gas</ghg:SubBasinIdentifier>

<ghg:CalculationMethodology>Calculation Methodology 3</ghg:CalculationMethodology>

<ghg:WellsSendingOilToTanks>6973</ghg:WellsSendingOilToTanks>

<ghg:WellsSendingOilToSeparators>6973</ghg:WellsSendingOilToSeparators>

<ghg:MinimumAPIGravity densityUOM="degrees">15</ghg:MinimumAPIGravity>

<ghg:MaximumAPIGravity densityUOM="degrees">68</ghg:MaximumAPIGravity>

<ghg:TanksOnWellpads>1744</ghg:TanksOnWellpads>

<ghg:TanksOnWellpadsWithVaporRecovery>872</ghg:TanksOnWellpadsWithVaporRecovery>

<ghg:TanksOnWellpadsWithFlaring>872</ghg:TanksOnWellpadsWithFlaring>

<ghg:TanksOffWellpadsWithVaporRecovery>872</ghg:TanksOffWellpadsWithVaporRecovery>

<ghg:TanksOffWellpadsWithFlaring>872</ghg:TanksOffWellpadsWithFlaring>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">21680.5</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">724714.1</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">604332.5</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">14494.3</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">199.8</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:TwoYearDelayIndicator>No</ghg:TwoYearDelayIndicator>

<ghg:TotalVolumeOfOil>5.4</ghg:TotalVolumeOfOil>

<ghg:AnnualCarbonDioxideRecovered>25000</ghg:AnnualCarbonDioxideRecovered>

<ghg:AnnualMethaneRecovered>7000</ghg:AnnualMethaneRecovered>

</ghg:AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

</ghg:AtmosphericTanksCalculationMethodThreeOrFourDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details Schema Diagram

Note: Data elements boxed in red are required.

For gas emitted from produced oil sent to atmospheric tanks for wellhead gas-liquid separators and wells with throughput less than 10 barrels per day, using Calculation Methodology 5 of 98.233(j) Equation W-15 of 98.233, report the following by sub-basin category [98.236(c)(8)(iii)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- Annual CO2 and CH4 emissions that resulted from venting gas to the atmosphere, expressed in metric tons CO2e for each gas, at the sub-basin level for Calculation Methodology 5 of 98.233(j). [98.236(c)(8)(iii)(F)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring gas, expressed in metric tons CO2e for each gas, at the sub-basin level for Calculation Methodology 5 of 98.233(j). [98.236(c)(8)(iii)(H)]

- Number of wellhead separators. [98.236(c)(8)(iii)(A)]

- Number of wells without wellhead separators. [98.236(c)(8)(iii)(B)]

- Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(iii)(G)]

- Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(iii)(G)]

Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details 
Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AtmosphericTanksCalculationMethodFiveDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins with wellhead gas-liquid separators and wells with oil throughput <10 barrels/day using Calculation Methodology 5 subject to reporting under 98.232 in the reporting year. |
| AtmosphericTanksCalculationMethodFiveSubBasinRowDetails | Parent Element: A collection of data elements to report for each sub-basin with wellhead gas-liquid separators and wells with oil throughput <10 barrels/day using Calculation Methodology 5. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(8)(iii)] See Sub-Basin Identification for the required naming convention. |
| VentingCarbonDioxideEmissions | CO2 emissions from venting in the specified sub-basin in metric tons. [98.236(c)(8)(iii)(F)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting in the specified sub-basin in metric tons CO2e. [98.236(c)(8)(iii)(F)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring in the specified sub-basin in metric tons. [98.236(c)(8)(iii)(H)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring in the specified sub-basin in metric tons CO2e. [98.236(c)(8)(iii)(H)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring in the specified sub-basin in metric tons CO2e. 98.236(c)(8)(iii)(H)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NumberOfWellheadSeparators | Number of wellhead separators. [98.236(c)(8)(iii)(A)] |
| NumberOfWellsWithoutWellheadSeparators | Number of wells without wellhead separators. [98.236(c)(8)(iii)(B)] |
| AnnualCarbonDioxideRecovered | Annual CO2 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(iii)(G)] |
| AnnualMethaneRecovered | Annual CH4 gas quantities that were recovered (mt CO2e). [98.236(c)(8)(iii)(G)] |

Example for Atmospheric Tanks Calculation Methodology 5 Sub-Basin Details

<ghg:AtmosphericTanksCalculationMethodFiveDetails>

<ghg:AtmosphericTanksCalculationMethodFiveSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - CADDO, OK (15) - Coal seam</ghg:SubBasinIdentifier>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">123.5</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">5946.8</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">401.1</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">123.5</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.7</ghg:FlaringNitrousCarbonDioxideEquivalent>

<ghg:NumberOfWellheadSeparators>5</ghg:NumberOfWellheadSeparators>

<ghg:NumberOfWellsWithoutWellheadSeparators>73</ghg:NumberOfWellsWithoutWellheadSeparators>

<ghg:AnnualCarbonDioxideRecovered>50000</ghg:AnnualCarbonDioxideRecovered>

<ghg:AnnualMethaneRecovered>14000</ghg:AnnualMethaneRecovered>

</ghg:AtmosphericTanksCalculationMethodFiveSubBasinRowDetails>

</ghg:AtmosphericTanksCalculationMethodFiveDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details

Note: Data elements boxed in red are required.

If a wellhead separator dump valve is functioning improperly during the reporting year (refer to Equation W-16 of 98.233), report the following by sub-basin category [98.236(c)(8)(iv)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- Annual CO2 and CH4 emissions from improperly functioning dump valves, expressed in metric tons CO2e for each gas, at the sub-basin level (Equation W-16). [98.236(c)(8)(iv)(B)]  Note:  If reporting emissions for an improperly functioning wellhead separator dump valve, do not report emissions for separators reported under Calculation Methodologies 1, 2 or 4.

Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details 
Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AtmosphericTanksWellheadSeparatorDumpValveDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any sub-basins with a wellhead separator dump valve functioning improperly during the reporting year. |
| AtmosphericTanksWellheadSeparatorDumpValveSubBasinRowDetails | Parent Element: A collection of data elements to report for each sub-basin with a wellhead separator dump valve functioning improperly during the reporting year. |
| SubBasinIdentifier | Sub-basin ID. [98.236(c)(8)(iv)] See Sub-Basin Identification for the required naming convention. |
| VentingCarbonDioxideEmissions | CO2 emissions from improperly functioning dump valves at the specified sub-basin in metric tons. [98.236(c)(8)(iv)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If reporting emissions for an improperly functioning wellhead separator dump valve, do not report emissions for that separator under Calculation Methodologies 1, 2 or 4. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from improperly functioning dump valves at the specified sub-basin in metric tons CO2e. [98.236(c)(8)(iv)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If reporting emissions for an improperly functioning wellhead separator dump valve, do not report emissions for that separator under Calculation Methodologies 1, 2 or 4. |

Example for Atmospheric Tanks Wellhead Separator Dump Valve Sub-Basin Details

<ghg:AtmosphericTanksWellheadSeparatorDumpValveDetails>

<ghg:AtmosphericTanksWellheadSeparatorDumpValveSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - DEWEY, OK (43) - Other tight reservoir rock</ghg:SubBasinIdentifier>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">22459.3</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">750744.2</ghg:VentingMethaneCarbonDioxideEquivalent>

</ghg:AtmosphericTanksWellheadSeparatorDumpValveSubBasinRowDetails>

</ghg:AtmosphericTanksWellheadSeparatorDumpValveDetails>

</ghg:AtmosphericTanksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Transmission Tanks

This topic provides a step-by-step description of how to report transmission tank emissions information for a facility.  This section is applicable to and required for the onshore natural gas transmission compression industry segment only.

Transmission Tanks Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For transmission tanks, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “VentingCarbonDioxideEmissions” for each vent stack.

- “FlaringCarbonDioxideEmissions” for each vent stack.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “VentingMethaneCarbonDioxideEquivalent” for each vent stack.

- “FlaringMethaneCarbonDioxideEquivalent” for each vent stack.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data element:

- “FlaringNitrousCarbonDioxideEquivalent” for each vent stack.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any vent stack emissions from a compressor scrubber dump valve leak that were quantified per 98.233(k) in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report if the facility had any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year.

Transmission Tanks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| TransmissionTanksDetails | Parent Element: A collection of data elements to report for transmission tanks. [98.236(c)(9)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| DidFacilityHaveVentStackEmissions | Indicate (Yes/No) if the facility had any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| TransmissionStorageTanksDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |

Example for Transmission Tanks Details

<ghg:TransmissionTanksDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">169433.3</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">129044.2</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">12.5</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">298490</ghg:TotalCarbonDioxideEquivalent>	<ghg:DidFacilityHaveVentStackEmissions>Yes</ghg:DidFacilityHaveGasTransmissionTankEmissions>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:TransmissionStorageTanksDetails>

<See example for Transmission Storage Tanks Details>

</ghg:TransmissionStorageTanksDetails>

</ghg:TransmissionTanksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Transmission Storage Tanks Details Schema Diagram

Note: Data elements boxed in red are required.

If the facility had any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year, report the following for each vent stack [98.236(c)(9)]:

- A unique name or ID number for the vent stack monitored according to 40 CFR 98.233(k). [98.236(c)(9)(iii)]

- Annual CO2 and CH4 emissions that resulted from venting gas directly to the atmosphere, expressed in metric tons CO2e for each gas. [98.236(c)(9)(i)]

- Annual CO2, CH4 and N2O emissions that resulted from flaring process gas expressed in metric tons CO2e for each gas. [98.236(c)(9)(ii)]

Transmission Storage Tanks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| TransmissionStorageTanksDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any vent stack emissions from compressor scrubber dump valve leakage that were quantified per 98.233(k) in the reporting year. |
| TransmissionStorageTanksRowDetails | Parent Element: A collection of data elements to report for each vent stack. |
| VentStackIdentifier | A unique name or identifier associated with the vent stack. [98.236(c)(9)(iii)] |
| VentingCarbonDioxideEmissions | CO2 emissions from venting gas directly to the atmosphere from the specified vent stack in metric tons. [98.236(c)(9)(i)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting gas directly to the atmosphere from the specified vent stack in metric tons CO2e. [98.236(c)(9)(i)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring process gas from the specified vent stack in metric tons. [98.236(c)(9)(ii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring process gas from the specified vent stack in metric tons CO2e. [98.236(c)(9)(ii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring process gas from the specified vent stack in metric tons CO2e. [98.236(c)(9)(ii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Transmission Storage Tanks Details

<ghg:TransmissionStorageTanksDetails>

<ghg:TransmissionStorageTanksRowDetails>

<ghg:VentStackIdentifier>101</ghg:VentStackIdentifier>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">1</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">699.1</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">0</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:FlaringNitrousCarbonDioxideEquivalent>

</ghg:TransmissionStorageTanksRowDetails>

<ghg:TransmissionStorageTanksRowDetails>

<ghg:VentStackIdentifier>102</ghg:VentStackIdentifier>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">172.7</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">125828.6</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">169259.6</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">2516.5</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">12.5</ghg:FlaringNitrousCarbonDioxideEquivalent>

</ghg:TransmissionStorageTanksRowDetails>

</ghg:TransmissionStorageTanksDetails>

</ghg:TransmissionTanksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Well Testing Venting and Flaring

This topic provides a step-by-step description of how to report well testing information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Well Testing Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For well testing, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all well testing venting and flaring combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any well testing that resulted in venting or flaring subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “VentingCarbonDioxideEmissions” for the basin.

- “FlaringCarbonDioxideEmissions” for the basin.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “VentingMethaneCarbonDioxideEquivalent” for the basin.

- “FlaringMethaneCarbonDioxideEquivalent” for the basin.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the value reported for the following data element:

- “FlaringNitrousCarbonDioxideEquivalent” for the basin.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility performed well testing that resulted in venting or flaring subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report if the facility performed well testing that resulted in venting or flaring subject to reporting under 98.232 in the reporting year.

Well Testing Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellTestingDetails | Parent Element: A collection of data elements to report for well testing. [98.236(c)(10)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all well testing combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well testing that resulted in venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all well testing combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well testing that resulted in venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all well testing combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well testing that resulted in venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all well testing combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any well testing that resulted in venting or flaring subject to reporting under 98.232 during the reporting year. |
| DidFacilityPerformWellTesting | Indicate (Yes/No) if the facility performed well testing that resulted in venting or flaring subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| WellTestingBasinsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility performed well testing that resulted in venting or flaring subject to reporting under 98.232 in the reporting year. |

Example for Well Testing Details

<ghg:WellTestingDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">170.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">208.8</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">234.6</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">614.0</ghg:TotalCarbonDioxideEquivalent>

<ghg:DidFacilityPerformWellTesting>Yes</ghg:DidFacilityPerformWellTesting>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:WellTestingBasinsDetails>

<See example for Well Basin Testing Details>

</ghg:WellTestingBasinsDetails>

</ghg:WellTestingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Well Testing Basin Details Schema Diagram

Note: Data elements boxed in red are required.

If the facility performed well testing that resulted in venting or flaring in the reporting year (refer to Equation W-17A or W-17B of 98.233), report the following [98.236(c)(10)]:

- The unique identity of the basin (see Basin Identification for the naming convention). [98.236(c)(10)]

- Number of wells tested in the basin in the reporting year. [98.236(c)(10)(i)]

- Average gas to oil ratio for the basin in cubic feet of gas per barrel of oil. [98.236(c)(10)(ii)]

- Average number of days that the wells were tested in the basin. [98.236(c)(10)(iii)]

- The annual CO2 and CH4 emissions at the basin/facility level, expressed in metric tons CO2e for each gas, from well testing venting. [98.236(c)(10)(iv)]

- The annual CO2, CH4 and N2O emissions at the basin/facility level, expressed in metric tons CO2e for each gas, from well testing flaring. [98.236(c)(10)(v)]

Well Testing Basin Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| WellTestingBasinsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility performed well testing that resulted in venting or flaring subject to reporting under 98.232 in the reporting year. |
| WellTestingBasinsRowDetails | Parent Element: A collection of data elements to report if the facility performed well testing that resulted in venting or flaring in the reporting year. |
| BasinIdentifier | Basin ID. [98.236(c)(10)] See Basin Identification for the required naming convention. |
| NumberOfWellsTested | Number of wells tested in reporting year. [98.236(c)(10)(i)] |
| AverageGasToOilRatio | Average gas to oil ratio in cubic feet of gas per barrel oil. [98.236(c)(10)(ii)] Set the units of measure to “cubic feet of gas per barrel of oil” in the attribute fractionUOM. |
| AverageNumberOfDaysTested | Average number of days that wells was tested. [98.236(c)(10)(iii)] |
| VentingCarbonDioxideEmissions | Total CO2 emissions from venting from the basin in metric tons. [98.236(c)(10)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | Total CH4 emissions from venting from the basin in metric tons CO2e. [98.236(c)(10)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | Total CO2 emissions from flaring for the basin in metric tons. [98.236(c)(10)(v)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | Total CH4 emissions from flaring for the basin in metric tons CO2e. [98.236(c)(10)(v)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | Total N2O emissions from flaring for the basin in metric tons CO2e. [98.236(c)(10)(v)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Well Basin Testing Details

<ghg:WellTestingBasinsDetails>

<ghg:WellTestingBasinsRowDetails>

<ghg:BasinIdentifier>360 – Anadarko Basin</ghg:BasinIdentifier>

<ghg:NumberOfWellsTested>54</ghg:NumberOfWellsTested>

<ghg:AverageGasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">12.3</ghg:AverageGasToOilRatio>

<ghg:AverageNumberOfDaysTested>87</ghg:AverageNumberOfDaysTested>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">74.2</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">85.3</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">96.4</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">123.5</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">234.6</ghg:FlaringNitrousCarbonDioxideEquivalent>

</ghg:WellTestingBasinsRowDetails>

</ghg:WellTestingBasinsDetails>

</ghg:WellTestingDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Associated Gas Venting and Flaring

This topic provides a step-by-step description of how to report associated gas venting and flaring information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

Associated Gas Venting and Flaring Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For associated gas venting and flaring, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all associated gas venting and flaring combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)] Note:  Report “0” if the facility did not have any associated gas venting and flaring subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “VentingCarbonDioxideEmissions” for the basin.

- “FlaringCarbonDioxideEmissions” for the basin.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “VentingMethaneCarbonDioxideEquivalent” for the basin.

- “FlaringMethaneCarbonDioxideEquivalent” for the basin.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the value reported for the following data element:

- “FlaringNitrousCarbonDioxideEquivalent” for the basin.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any associated gas venting or flaring subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report if the facility had any associated gas venting or flaring subject to reporting under 98.232 in the reporting year.

Associated Gas Venting and Flaring Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AssociatedGasVentingFlaringDetails | Parent Element: A collection of data elements to report for associated gas venting and flaring. [98.236(c)(11)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all associated gas venting and flaring reported combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any associated gas venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all associated gas venting and flaring reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any associated gas venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all associated gas venting and flaring reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any associated gas venting or flaring subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all associated gas venting and flaring reported combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have associated any gas venting or flaring subject to reporting under 98.232 during the reporting year. |
| DidFacilityHaveGasVenting | Indicate (Yes/No) if the facility had any associated gas venting or flaring subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| AssociatedGasVentingFlaringBasinsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any associated gas venting or flaring subject to reporting under 98.232 in the reporting year. |

Example for Associated Gas Venting and Flaring Details

<ghg:AssociatedGasVentingFlaringDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">68649.9</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">19057.1</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.3</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">87707.3</ghg:TotalCarbonDioxideEquivalent>

<ghg:DidFacilityHaveGasVenting>Yes</ghg:DidFacilityHaveGasVenting>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:AssociatedGasVentingFlaringBasinsDetails>

<See example for Associated Gas Venting and Flaring Basins Details>

</ghg:AssociatedGasVentingFlaringBasinsDetails>

</ghg:AssociatedGasVentingFlaringDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Associated Gas Venting and Flaring Basins Details Schema Diagram

Note: Data elements boxed in red are required.

If the facility had any associated gas venting or flaring (refer to Equation W-18 of 98.233), report the following for the basin/facility [98.236(c)(11)]:

- The unique identity of the basin (see Basin Identification for the naming convention). [98.236(c)(11)]

- Number of wells venting associated gas in the reporting year. [98.236(c)(11)(i)]

- Number of wells flaring associated gas in the reporting year. [98.236(c)(11)(i)]

- Average gas to oil ratio for the basin in cubic feet of gas per barrel of oil. [98.236(c)(11)(ii)]

- The annual CO2 and CH4 emissions at the basin/facility level, expressed in metric tons CO2e for each gas, from associated gas venting. [98.236(c)(11)(iii)]

- The annual CO2, CH4 and N2O emissions at the basin/facility level, expressed in metric tons CO2e for each gas, from associated gas flaring. [98.236(c)(11)(iv)]

Associated Gas Venting and Flaring Basins Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AssociatedGasVentingFlaringBasinsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any associated gas venting or flaring subject to reporting under 98.232 in the reporting year. |
| AssociatedGasVentingFlaringBasinsRowDetails | Parent Element: A collection of data elements to report for the basin. |
| BasinIdentifier | Basin ID. [98.236(c)(11)] See Basin Identification for the required naming convention. |
| NumberOfWellsVenting | Number of wells venting associated gas. [98.236(c)(11)(i)] |
| NumberOfWellsFlaring | Number of wells flaring associated gas. [98.236(c)(11)(i)] |
| AverageGasToOilRatio | Average gas to oil ratio for the basin in cubic feet of gas per barrel of oil. [98.236(c)(11)(ii)] Set the units of measure to "cubic feet of gas per barrel of oil" in the attribute fractionUOM. |
| VentingCarbonDioxideEmissions | CO2 emissions from venting from the basin in metric tons. [98.236(c)(11)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| VentingMethaneCarbonDioxideEquivalent | CH4 emissions from venting from the basin in metric tons CO2e. [98.236(c)(11)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringCarbonDioxideEmissions | CO2 emissions from flaring for the basin in metric tons. [98.236(c)(11)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringMethaneCarbonDioxideEquivalent | CH4 emissions from flaring for the basin in metric tons CO2e. [98.236(c)(11)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| FlaringNitrousCarbonDioxideEquivalent | N2O emissions from flaring for the basin in metric tons CO2e. [98.236(c)(11)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Associated Gas Venting and Flaring Basins Details

<ghg:AssociatedGasVentingFlaringBasinsDetails>

<ghg:AssociatedGasVentingFlaringBasinsRowDetails>

<ghg:BasinIdentifier>360 - Anadarko Basin</ghg:BasinIdentifier>

<ghg:NumberOfWellsVenting>18</ghg:NumberOfWellsVenting>

<ghg:NumberOfWellsFlaring>34</ghg:NumberOfWellsFlaring>

<ghg:AverageGasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">622921275.7</ghg:AverageGasToOilRatio>

<ghg:VentingCarbonDioxideEmissions massUOM="Metric Tons">19.8</ghg:VentingCarbonDioxideEmissions>

<ghg:VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">19049.4</ghg:VentingMethaneCarbonDioxideEquivalent>

<ghg:FlaringCarbonDioxideEmissions massUOM="Metric Tons">68630.1</ghg:FlaringCarbonDioxideEmissions>

<ghg:FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">7.7</ghg:FlaringMethaneCarbonDioxideEquivalent>

<ghg:FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.3</ghg:FlaringNitrousCarbonDioxideEquivalent>

</ghg:AssociatedGasVentingFlaringBasinsRowDetails>

</ghg:AssociatedGasVentingFlaringBasinsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Flare Stacks

This topic provides a step-by-step description of how to report flare stack information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

Flare Stack Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For flare stacks, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all flare stacks combined which are not reported for another source type in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any flare stacks subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “CombustedAndUncombustedCarbonDioxide” for all flare stacks for which a CEMS was used.

- “UncombustedCarbonDioxideEmissions” for all flare stacks for which a CEMS was not used.

- “CombustedCarbonDioxideEmissions” for all flare stacks for which a CEMS was not used.

- The value to report for “UncombustedMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data element:

- “UncombustedMethaneCarbonDioxideEquivalent” for all flare stacks for which a CEMS was not used.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data element:

- “NitrousCarbonDioxideEquivalent” for all flare stacks for which a CEMS was not used.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any flare stacks subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report if the facility had any flare stacks subject to reporting under 98.232 in the reporting year.

Flare Stack Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| FlareStacksDetails | Parent Element: A collection of data elements to report for flare stacks. [98.236(c)(12)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all flare stacks reported combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any flare stacks subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all flare stacks reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any flare stacks subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all flare stacks reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any flare stacks subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all flare stacks reported combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any flare stacks subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveFlareStacks | Indicate (Yes/No) if the facility had any flare stacks subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| UniqueFlareStacksDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any flare stacks subject to reporting under 98.232 in the reporting year. |

Example for Flare Stack Details

<ghg:FlareStacksDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">158068.8</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">8.8</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.3</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">158077.9</ghg:TotalCarbonDioxideEquivalent>	<ghg:DoesFacilityHaveFlareStacks>Yes</ghg:DoesFacilityHaveFlareStacks>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>Description of BAMM</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:UniqueFlareStacksDetails>

<See example for Unique Flare Stacks Details>

</ghg:UniqueFlareStacksDetails>

</ghg:FlareStacksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Unique Flare Stacks Details Schema Diagram

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

If the facility had flare stacks (refer to Equation W-19, W-20 and W-21 of 98.233), report the following for each flare [98.236(c)(12)]:

- For the onshore petroleum and natural gas production industry segment only, a sequential identification number for the flare stack.  Note:  This number is used for reference only in the event that there is a validation message associated with the flare stack data.

- For the onshore natural gas processing industry segment only, a unique name or ID number for the flare stack.  [98.236(c)(12)(x)]

- Whether the flare has a continuous flow monitor. [98.236(c)(12)(i)]

- The percent of gas sent to un-lit flare determined by engineering estimate and process knowledge based on best available data and operating records. [98.236(c)(12)(iii)]

- Whether the flare has a continuous gas analyzer. [98.236(c)(12)(iv)]

- Whether a CEMS was used to measure CO2 emissions for the flare stack. [98.236(c)(12)(xi)]

- If a CEMS was used to measure CO2 emissions for the flare stack, report the combusted CO2 and uncombusted CO2 as a combined number in metric tons.  Note:  Report "0" if the flare is reported for another source type.  [98.236(c)(12)(xi)]

- If a CEMS was not used to measure CO2 emissions for the flare stack, report the uncombusted CO2 emissions, in metric tons CO2e (refer to Equation W-20 of 98.233).  Note:  Report "0" if the flare is reported for another source type.  [98.236(c)(12)(vii)]

- If a CEMS was not used to measure CO2 emissions for the flare stack, report the uncombusted CH4 emissions, in metric tons CO2e (refer to Equation W-19 of 98.233).  Note:  Report "0" if the flare is reported for another source type.  [98.236(c)(12)(vi)]

- If a CEMS was not used to measure CO2 emissions for the flare stack, report the combusted CO2 emissions, in metric tons CO2e (refer to Equation W-21 of 98.233).  Note:  Report "0" if the flare is reported for another source type.  [98.236(c)(12)(viii)]

- If a CEMS was not used to measure CO2 emissions for the flare stack, report the N2O emissions, in metric tons CO2e.  Note:  Report "0" if the flare is reported for another source type.  [98.236(c)(12)(ix)]

- Volume of gas sent to flare (cubic feet per year). [98.236(c)(12)(ii)]

- Flare combustion efficiency. [98.236(c)(12)(v)]

Note:  If flare emissions for a particular flare are reported for another source type, you must still report all required data elements, but report "0" for the emissions values.

Unique Flare Stacks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| UniqueFlareStacksDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any flare stacks subject to reporting under 98.232 in the reporting year. |
| UniqueFlareStacksRowDetails | Parent Element: A collection of data elements to report for each flare stack. |
| UniqueIdentifier | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, a unique ID for the flare stack. Please number flare stacks sequentially. |
| FlareStackIdentifier | Conditionally Required: For the onshore natural gas processing industry segment only, a unique name or ID for the flare stack. [98.236(c)(12)(x)] |
| DoesFlareHaveContinuousFlowMonitor | Indicate (Yes/No) if the specified flare has a continuous flow monitor. [98.236(c)(12)(i)] |
| PercentOfGasSent | Percent of gas sent to un-lit flare. [98.236(c)(12)(iii)] Set the units of measure to “Number (between 0 and 100)” in the attribute percentUOM. |
| DoesFlareHaveContinuousGasAnalyzer | Indicate (Yes/No) if the specified flare has a continuous gas analyzer. [98.236(c)(12)(iv)] |
| WereCemsUsed | Indicate (Yes/No) if CEMS was used to measure CO2 emissions for the specified flare stack. [98.236(c)(12)(xi)] |
| CombustedAndUncombustedCarbonDioxide | Conditionally Required: If CEMS was used, report the combusted and uncombusted CO2 combined in metric tons from the specified flare stack. [98.236(c)(12)(xi)] Note: Report "0" if the flare emissions are reported on another source type. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| UncombustedCarbonDioxideEmissions | Conditionally Required: If CEMS was not used, report the uncombusted CO2 emissions in metric tons from the specified flare stack. [98.236(c)(12)(vii)] Note: Report "0" if the flare emissions are reported on another source type. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| UncombustedMethaneCarbonDioxideEquivalent | Conditionally Required: If CEMS was not used, report the uncombusted CH4 emissions in metric tons CO2e from the specified flare stack. [98.236(c)(12)(vi)] Note: Report "0" if the flare emissions are reported on another source type. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CombustedCarbonDioxideEmissions | Conditionally Required: If CEMS was not used, report the combusted CO2 emissions in metric tons from the specified flare stack. [98.236(c)(12)(viii)] Note: Report "0" if the flare emissions are reported on another source type. |
| NitrousCarbonDioxideEquivalent | Conditionally Required: If CEMS was not used, report the N2O emissions in metric tons CO2e from the specified flare stack. [98.236(c)(12)(ix)] Note: Report "0" if the flare emissions are reported on another source type. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| GasSentToFlare | Volume of gas sent to flare (cubic feet per year). [98.236(c)(12)(ii)] |
| FlareCombustionEfficiency | Enter the combustion efficiency of the flare as a decimal fraction between 0 and 1. [98.236(c)(12)(v)] |

Example for Unique Flare Stacks Details

<ghg:UniqueFlareStacksDetails>

<ghg:UniqueFlareStacksRowDetails>

<ghg:UniqueIdentifier>1</ghg:UniqueIdentifier>

<ghg:DoesFlareHaveContinuousFlowMonitor>Yes</ghg:DoesFlareHaveContinuousFlowMonitor>

<ghg:PercentOfGasSent percentUOM="Number (between 0 and 100)">55</ghg:PercentOfGasSent>

<ghg:DoesFlareHaveContinuousGasAnalyzer>Yes</ghg:DoesFlareHaveContinuousGasAnalyzer>

<ghg:WereCemsUsed>Yes</ghg:WereCemsUsed>

<ghg:CombustedAndUncombustedCarbonDioxide massUOM="Metric Tons">79034.4</ghg:CombustedAndUncombustedCarbonDioxide>

<ghg:GasSentToFlare>10000</ghg:GasSentToFlare>

<ghg:FlareCombustionEfficiency>.8</ghg:FlareCombustionEfficiency>

</ghg:UniqueFlareStacksRowDetails>

<ghg:UniqueFlareStacksRowDetails>

<ghg:UniqueIdentifier>2</ghg:UniqueIdentifier>

<ghg:DoesFlareHaveContinuousFlowMonitor>No</ghg:DoesFlareHaveContinuousFlowMonitor>

<ghg:PercentOfGasSent percentUOM="Number (between 0 and 100)">65</ghg:PercentOfGasSent>

<ghg:DoesFlareHaveContinuousGasAnalyzer>No</ghg:DoesFlareHaveContinuousGasAnalyzer>

<ghg:WereCemsUsed>No</ghg:WereCemsUsed>

<ghg:UncombustedCarbonDioxideEmissions massUOM="Metric Tons">22.8</ghg:UncombustedCarbonDioxideEmissions>

<ghg:UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">8.8</ghg:UncombustedMethaneCarbonDioxideEquivalent>

<ghg:CombustedCarbonDioxideEmissions>79011.6</ghg:CombustedCarbonDioxideEmissions>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.3</ghg:NitrousCarbonDioxideEquivalent>

<ghg:GasSentToFlare>10000</ghg:GasSentToFlare>

<ghg:FlareCombustionEfficiency>.8</ghg:FlareCombustionEfficiency>

</ghg:UniqueFlareStacksRowDetails>

</ghg:UniqueFlareStacksDetails>

</ghg:FlareStacksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Centrifugal Compressors

This topic provides a step-by-step description of how to report centrifugal compressor information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

- Onshore natural gas transmission compression

- Underground natural gas storage

- Liquefied natural gas (LNG) storage

- LNG import and export equipment

Centrifugal Compressors Details Schema Diagram

Conditionally

Required

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For centrifugal compressors, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all centrifugal compressors combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any centrifugal compressors subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” depends upon the industry segment:

- For centrifugal compressors in the onshore petroleum and natural gas production industry segment only, the value to report for “TotalCarbonDioxideEmissions” equals the value reported for the data element “TotalAnnualCompressorCarbonDioxideEmissions”.

- For centrifugal compressors in all other industry segments, the value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for data element “TotalAnnualCarbonDioxideEmissions”.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” depends upon the industry segment:

- For centrifugal compressors in the onshore petroleum and natural gas production industry segment only, the value to report for “TotalMethaneCarbonDioxideEquivalent” equals the value reported for the data element “TotalAnnualCompressorMethaneCarbonDioxideEquivalent”.

- For centrifugal compressors in all other industry segments, the value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for data element “TotalAnnualMethaneCarbonDioxideEquivalent”.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” depends upon the industry segment:

- For centrifugal compressors in the onshore petroleum and natural gas production industry segment only, report “0” for “TotalNitrousCarbonDioxideEquivalent”.

- For centrifugal compressors in all other industry segments, the value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for data element “TotalAnnualNitrousCarbonDioxideEquivalent”.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any centrifugal compressors with wet or dry seals subject to reporting under 98.232 in the reporting year.

- Whether BAMM was used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- For centrifugal compressors in the onshore petroleum and natural gas production industry segment only (refer to Equation W-25 of 98.233), report the following:

- Total annual CO2 emissions in metric tons for all centrifugal compressors collectively. [98.236(c)(13)(v)(B)]

- Total annual CH4 emissions in metric tons of CO2e for all centrifugal compressors collectively. [98.236(c)(13)(v)(B)]

- The total count of compressors. [98.236(c)(13)(v)(A)]

Note:  For facilities in industry segments other than onshore petroleum and natural gas production, emissions must be reported for centrifugal compressors in operating mode and not operating, depressurized node, even if the compressor was not measured in that mode.

Centrifugal Compressors Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| CentrifugalCompressorsDetails | Parent Element: A collection of data elements to report for centrifugal compressors. [98.236(c)(13)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all centrifugal compressors combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any centrifugal compressors subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all centrifugal compressors combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any centrifugal compressors subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all centrifugal compressors combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any centrifugal compressors subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all centrifugal compressors combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any centrifugal compressors subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveCentrifugalCompressors | Indicate (Yes/No) if the facility had any centrifugal compressors with wet or dry seals subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| TotalAnnualCompressorCarbonDioxideEmissions | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, the total annual compressor CO2 emissions in metric tons for all compressors combined. [98.236(c)(13)(v)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualCompressorMethaneCarbonDioxideEquivalent | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, the total annual compressor CH4 emissions in metric tons CO2e for all compressors combined. [98.236(c)(13)(v)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompressorCount | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, specify the total count of compressors [98.236(c)(13)(v)(A)] |
| CentrifugalCompressorsModesDetails | Parent Element (Conditionally Required): For all applicable industry segments except the onshore petroleum and natural gas production industry segment, a collection of data elements to report if there were any centrifugal compressors subject to reporting under 98.232 in the reporting year. |

Example for Centrifugal Compressors Details

<ghg:CentrifugalCompressorsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">1.9</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">9.8</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">1.5</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">13.2</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveCentrifugalCompressors>Yes</ghg:DoesFacilityHaveCentrifugalCompressors>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:CentrigfugalCompressorsModesDetails>

<See example for Centrifugal Compressors Modes Details>

</ghg:CentrifugalCompressorsModesDetails>

</ghg:CentrifugalCompressorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for applicable industry segments other than the onshore petroleum and natural gas production industry segment.

Example for Centrifugal Compressors Details for Onshore Petroleum and Natural Gas Production

<ghg:CentrifugalCompressorsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">65.5</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">8644.7</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">8710.2</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveCentrifugalCompressors>Yes</ghg:DoesFacilityHaveCentrifugalCompressors>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:TotalAnnualCompressorCarbonDioxideEmissions massUOM="Metric Tons">65.5</ghg:TotalAnnualCompressorCarbonDioxideEmissions>

<ghg:TotalAnnualCompressorMethaneCarbonDioxideEquivalent massUOM="Metric Tons">8644.7</ghg:TotalAnnualCompressorMethaneCarbonDioxideEquivalent>

<ghg:CompressorCount>3</ghg:CompressorCount>

</ghg:CentrifugalCompressorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for the onshore petroleum and natural gas production industry segment only.

Centrifugal Compressors Modes Details (Part 1 of 3) Schema Diagram

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For the onshore natural gas processing, onshore natural gas transmission compression, underground natural gas storage, liquefied natural gas (LNG) storage and LNG import and export equipment industry segments only, if the facility had any centrifugal compressors subject to reporting under 98.232 during the reporting year, then report the following for each centrifugal compressor:

- The unique identity of the compressor.

- Whether BAMM was used to estimate emissions from the compressor.

- The compressor’s seal type (wet or dry). [98.236(c)(13)(i)(A)]

- For centrifugal compressors in operating mode with wet seals only (refer to Equations W-22 through W-24 of 98.233), report the following:

  - The annual throughput using an engineering calculation based on best available data in MMscf. [98.236(c)(13)(i)(C)]

  - Number of wet seals connected to the degassing vent. [98.236(c)(13)(i)(A)]

  - Type of meters used for making measurements. [98.236(c)(13)(i)(D)]

  - Fraction of vent gas recovered for fuel. [98.236(c)(13)(i)(B)]

  - Fraction of vent gas recovered for sales. [98.236(c)(13)(i)(B)]

  - Fraction of vent gas recovered and flared. 98.236(c)(13)(i)(B)]

  - Seal Vent emissions, report the following:

Note 1:  Report either measured seal vent CO2 and CH4 emissions or not measured seal vent CO2 and CH4 emissions, but not both pairs of data or one GHG from each pair.

Note 2:  If a centrifugal compressor was measured and had no emissions for a particular GHG while in operating mode during the reporting year, then report “0” for that GHG’s measured seal vent emissions.

Note 3:  If a centrifugal compressor was never in the operating mode during the reporting year, then report “0” for not measured seal vent CO2 and CH4 emissions.

    - Measured seal vent CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(i)(G)]

    - Measured seal vent CH4 emissions in metric tons of CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(i)(G)]

OR

    - Not measured seal vent CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(i)(G)]

    - Not measured seal vent CH4 emissions in metric tons of CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(i)(G)]

Centrifugal Compressors Modes Details (Part 2 of 3) Schema Diagram

Note: Please see page 4 of this document for more information on conditionally required elements.

- For centrifugal compressors in operating mode with wet or dry seals, report the following:

Note 1:  Report either measured blowdown vent CO2 and CH4 emissions or not measured blowdown vent CO2 and CH4 emissions, but not both pairs of data or one GHG from each pair.

Note 2:  If a centrifugal compressor was measured and had no emissions for a particular GHG while in operating mode during the reporting year, then report “0” for that GHG’s measured blowdown vent emissions.

Note 3:  If a centrifugal compressor was never in the operating mode during the reporting year, then report “0” for not measured blowdown vent CO2 and CH4 emissions.

  - Measured blowdown vent CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(ii)(C)]

  - Measured blowdown vent CH4 emissions in metric tons of CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(ii)(C)]

OR

  - Not measured blowdown vent CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(ii)(C)]

  - Not measured blowdown vent CH4 emissions in metric tons of CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(ii)(C)]

- For centrifugal compressors in not operating, depressurized mode with wet or dry seals (refer to Equations W-22 through W-24 of 98.233), report the following:

Note 1:  Report either measured isolation valve leakage CO2 and CH4 emissions or not measured isolation valve CO2 and CH4 emissions, but not both pairs of data or one GHG from each pair.

Note 2:  If a centrifugal compressor was measured and had no emissions for a particular GHG while in not operating, depressurized mode during the reporting year, then report “0” for that GHG’s measured isolation valve leakage emissions.

Note 3:  If a centrifugal compressor was never in the not operating, depressurized mode during the reporting year, then report “0” for not measured isolation valve leakage CO2 and CH4 emissions.

  - Measured isolation valve leakage CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iii)(C)]

  - Measured isolation valve leakage CH4 emissions in metric tons CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iii)(C)]

OR

  - Not measured isolation valve leakage CO2 emissions in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iii)(C)]

  - Not measured isolation valve leakage CH4 emissions in metric tons CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iii)(C)]

Centrifugal Compressors Modes Details (Part 3 of 3) Schema Diagram

Conditionally

Required

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

- For all centrifugal compressors, report if the compressor vented emissions to a flare.

- For centrifugal compressors which vented emissions to a flare, report the following:

- Total annual CO2 emissions from flaring for all modes of operation combined in metric tons. [98.236(c)]

- Total annual CH4 emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)]

- Total annual N2O emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)]

- For each centrifugal compressor, report the total annual CO2 emissions from all modes of operation combined in metric tons (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iv)]  The value to report for “TotalAnnualCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “MeasuredSealVentCarbonDioxideEmissions” for each centrifugal compressor.

- “NotMeasuredSealVentCarbonDioxideEmissions” for each centrifugal compressor.

- “MeasuredBlowdownVentCarbonDioxideEmissions” for each centrifugal compressor.

- “NotMeasuredBlowdownVentCarbonDioxideEmissions” for each centrifugal compressor.

- “MeasuredIsolationValveCarbonDioxideEmissions” for each centrifugal compressor.

- “NotMeasuredIsolationValveCarbonDioxideEmissions” for each centrifugal compressor.

- “FlaringCarbonDioxideEmissions” for each centrifugal compressor.

- For each centrifugal compressor, report total annual CH4 emissions from all modes of operation combined in metric tons of CO2e (refer to Equation W-22 through W-24 of 98.233). [98.236(c)(13)(iv)]  The value to report for “TotalAnnualMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “MeasuredSealVentMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “NotMeasuredSealVentMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “MeasuredBlowdownVentMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “NotMeasuredBlowdownVentMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “MeasuredIsolationValveMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- “FlaringMethaneCarbonDioxideEquivalent” for each centrifugal compressor.

- For each centrifugal compressor, report total annual N2O emissions from all modes of operation combined in metric tons of CO2e. [98.236(c)]  The value to report for “TotalAnnualNitrousCarbonDioxideEquivalent” equals the value reported for the data element “FlaringNitrousCarbonDioxideEquivalent”, or “0” if no emissions were flared.

Note:  If a compressor had no emissions in all modes, report “0”.

- For compressors with wet or dry seals in operating mode, report the total time in operating mode (hours). [98.236(c)(13)(i)(F)] [98.236(c)(13)(ii)(A)]

- For compressors with wet seals in operating mode, enter the reporter emission factor for wet seal oil degassing vents in cubic feet per hour. [98.236(c)(13)(i)(E]

- For compressors with wet or dry seals in operating mode, report the reporter emission factor for blowdown vents (cubic feet per hour). [98.236(c)(13)(ii)(B)]

- For wet and dry seal centrifugal compressors in non-operating, depressurized mode, report the total time in shutdown, depressurized mode (hours). [98.236(c)(13)(iii)(A)]

- For wet and dry seal centrifugal compressors in non-operating, depressurized mode, report the reporter emission factor for isolation valve emissions (cubic feet per hour). [98.236(c)(13)(iii)(B)]

Centrifugal Compressors Modes Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| CentrifugalCompressorsModesDetails | Parent Element (Conditionally Required): For all applicable industry segments except the onshore petroleum and natural gas production industry segment, a collection of data elements to report if there were any centrifugal compressors subject to reporting under 98.232 in the reporting year. |
| CentrifugalCompressorsModesRowDetails | Parent Element: A collection of data elements to report for each centrifugal compressor. |
| CompressorIdentifier | Unique name or ID number for the centrifugal compressor. |
| WereBammUsedForCompressor | Indicate (Yes/No) if BAMM were used to calculate GHG emissions for the specified centrifugal compressor. |
| SealType | The seal type of the specified compressor. See list of allowable values. Wet Dry |
| AnnualThroughput | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the annual throughput using an engineering calculation based on best available data in million SCF. [98.236(c)(13)(i)(C)] Set the units of measure to "MMscf" in the attribute volUOM |
| NumberOfWetSeals | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the number of wet seals connected to the degassing vent. [98.236(c)(13)(i)(A)] |
| MeterTypeUsed | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the type of meter used for making measurements. [98.236(c)(13)(i)(D)] See list of allowable values. Temporary Continuous BAMM |
| VentGasFractionRecoveredForFuel | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the fraction of vent gas recovered for fuel. [98.236(c)(13)(i)(B)] Set the units of measure to "decimal fraction" (a number between 0 and 1) in the attribute fractionUOM. |
| VentGasFractionRecoveredForSales | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the fraction of vent gas recovered for sales. [98.236(c)(13)(i)(B)] Set the units of measure to "decimal fraction" (a number between 0 and 1) in the attribute fractionUOM. |
| VentGasFractionRecoveredAndFlared | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the fraction of vent gas recovered and flared. [98.236(c)(13)(i)(B)] Set the units of measure to "decimal fraction" (a number between 0 and 1) in the attribute fractionUOM. |
| MeasuredSealVentCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the measured seal vent CO2 emissions for the specified compressor in metric tons. [98.236(c)(13)(i)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both seal vent CO2 emissions and seal vent CH4 emissions as both measured or both not measured. |
| MeasuredSealVentMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the measured seal vent CH4 emissions for the specified compressor in metric tons CO2e. [98.236(c)(13)(i)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both seal vent CO2 emissions and seal vent CH4 emissions as both measured or both not measured. |
| NotMeasuredSealVentCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the not measured seal vent CO2 emissions for the specified compressor in metric tons. [98.236(c)(13)(i)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both seal vent CO2 emissions and seal vent CH4 emissions as both measured or both not measured. |
| NotMeasuredSealVentMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in operating mode with wet seals, the not measured seal vent CH4 emissions for the specified compressor in metric tons CO2e. [98.236(c)(13)(i)(G)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both seal vent CO2 emissions and seal vent CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in operating mode with wet or dry seals, the measured blowdown vent CO2 emissions in metric tons for the specified compressor. [98.236(c)(13)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in operating mode with wet or dry seals, the measured blowdown vent CH4 emissions metric tons CO2e. [98.236(c)(13)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in operating mode with wet or dry seals, the not measured blowdown vent CO2 emissions in metric tons for the specified compressor. [98.236(c)(13)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in operating mode with wet or dry seals, the not measured blowdown vent CH4 emissions metric tons CO2e. [98.236(c)(13)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredIsolationValveCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in not operating, depressurized mode with wet or dry seals, the measured isolation valve leakage CO2 emissions in metric tons. [98.236(c)(13)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| MeasuredIsolationValveMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in not operating, depressurized mode with wet or dry seals, the measured isolation valve leakage CH4 emissions in metric tons CO2e. [98.236(c)(13)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| NotMeasuredIsolationValveCarbonDioxideEmissions | Conditionally Required: For centrifugal compressors in not operating, depressurized mode with wet or dry seals, the not measured isolation valve leakage CO2 emissions in metric tons. [98.236(c)(13)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent | Conditionally Required: For centrifugal compressors in not operating, depressurized mode with wet or dry seals, the not measured isolation valve leakage CH4 emissions in metric tons CO2e. [98.236(c)(13)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| DidCompressorVentEmissionsToFlare | Indicate (Yes/No) if the compressor vented emissions to a flare in the reporting year. |
| TotalAnnualFlaringCarbonDioxideEmissions | Conditionally Required: If the compressor vented emissions to flares, the total annual CO2 emissions from flaring for all modes of operation combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualFlaringMethaneCarbonDioxideEquivalent | Conditionally Required: If the compressor vented emissions to flares, the total annual CH4 emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualFlaringNitrousCarbonDioxideEquivalent | Conditionally Required: If the compressor vented emissions to flares, the total annual N2O emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualCarbonDioxideEmissions | For each centrifugal compressor, the total annual CO2 emissions from all modes of operation combined in metric tons. [98.236(c)(13)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualMethaneCarbonDioxideEquivalent | For each centrifugal compressor, the total annual CH4 emissions from all modes of operation combined in metric tons CO2e. [98.236(c)(13)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualNitrousCarbonDioxideEquivalent | For each centrifugal compressor, the total annual N2O emissions from all modes of operation combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Enter “0” if not emissions were flared. |
| TotalTimeInOperatingMode | Conditionally Required: For compressors with wet or dry seals in operational mode, report the total time in operating mode (hours). [98.236(c)(13)(i)(F)] [98.236(c)(13)(ii)(A)] |
| WetSealOilEmissionFactor | Conditionally Required: For compressors with wet seals in operational mode, report the reporter emission factor for wet seal oil degassing vents in cubic feet per hour. [98.236(c)(13)(i)(E] |
| BlowdownVentsEmissionFactor | Conditionally Required: For compressors with wet or dry seals in operational mode, report the reporter emission factor for blowdown vents (cubic feet per hour). [98.236(c)(13)(ii)(B)] |
| TotalTimeInShutdownMode | Conditionally Required: For wet and dry seal centrifugal compressors in non-operating, depressurized mode, report the total time in shutdown, depressurized mode (hours). [98.236(c)(13)(iii)(A)] |
| IsolationValveEmissionFactor | Conditionally Required: For wet and dry seal centrifugal compressors in non-operating, depressurized mode, report the reporter emission factor for isolation valve emissions (cubic feet per hour). [98.236(c)(13)(iii)(B)] |

Example for a Centrifugal Compressor With Wet Seals in Operating Mode

<ghg:CentrifugalCompressorsModesDetails>

<ghg:CentrifugalCompressorsModesRowDetails>

<ghg:CompressorIdentifier>1</ghg:CompressorIdentifier>

<ghg:WereBAMMUsedForCompressor>No</ghg:WereBAMMUsedForCompressor>

<ghg:AnnualThroughput volUOM="MMscf">1493215.0</ghg:AnnualThroughput>

<ghg:SealType>Wet</ghg:SealType>

<ghg:NumberOfWetSeals>5</ghg:NumberOfWetSeals>

<ghg:MeterTypeUsed>Temporary</ghg:MeterTypeUsed>

<ghg:VentGasFractionRecoveredForFuel fractionUOM="fraction (number between 0 and 1)">0.3</ghg:VentGasFractionRecoveredForFuel>

<ghg:VentGasFractionRecoveredForSales fractionUOM="fraction (number between 0 and 1)">0.4</ghg:VentGasFractionRecoveredForSales>

<ghg:VentGasFractionRecoveredAndFlared fractionUOM="fraction (number between 0 and 1)">0.3</ghg:VentGasFractionRecoveredAndFlared>

<ghg:MeasuredSealVentCarbonDioxideEmissions massUOM="Metric Tons">1.6</ghg:MeasuredSealVentCarbonDioxideEmissions>

<ghg:MeasuredSealVentMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1111.3</ghg:MeasuredSealVentMethaneCarbonDioxideEquivalent>

<ghg:MeasuredBlowdownVentCarbonDioxideEmissions massUOM="Metric Tons">0.0</ghg:MeasuredBlowdownVentCarbonDioxideEmissions>

<ghg:MeasuredBlowdownVentMethaneCarbonDioxideEquivalent massUOM="Metric Tons">46.6</ghg:MeasuredBlowdownVentMethaneCarbonDioxideEquivalent>

<ghg:DidCompressorVentEmissionsToFlare>Yes</ghg:DidCompressorVentEmissionsToFlare >

<ghg:TotalAnnualFlaringCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:TotalAnnualFlaringCarbonDioxideEmissions>

<ghg:TotalAnnualFlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.2</ghg: TotalAnnualFlaringMethaneCarbonDioxideEquivalent >

<ghg:TotalAnnualFlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.2</ghg: TotalAnnualFlaringNitrousCarbonDioxideEquivalent >

<ghg:TotalAnnualCarbonDioxideEmissions massUOM="Metric Tons">1.7</ghg:TotalAnnualCarbonDioxideEmissions>

<ghg:TotalAnnualMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1273.4</ghg:TotalAnnualMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualNitrousCarbonDioxideEquivalent>200</ghg:TotalAnnualNitrousCarbonDioxideEquivalent>

<ghg:TotalTimeInOperatingMode>3000</ghg:TotalTimeInOperatingMode>

<ghg:WetSealOilEmissionFactor>4</ghg:WetSealOilEmissionFactor>

<ghg:BlowdownVentsEmissionFactor>2</ghg:BlowdownVentsEmissionFactor>

</ghg:CentrifugalCompressorsModesRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data. This example shows how to report a centrifugal compressor that has wet seals in operating mode.

Example for a Centrifugal Compressors with Dry Seals in Operating Mode

<ghg:CentrifugalCompressorsModesRowDetails>

<ghg:CompressorIdentifier>2</ghg:CompressorIdentifier>

<ghg:WereBammUsedForCompressor>No</ghg:WereBammUsedForCompressor>

<ghg:SealType>Dry</ghg:SealType>

<ghg:NotMeasuredBlowdownVentCarbonDioxideEmissions massUOM="Metric Tons">0.0</ghg:NotMeasuredBlowdownVentCarbonDioxideEmissions>

<ghg:NotMeasuredBlowdownVentMethaneCarbonDioxideEquivalent massUOM="Metric Tons">46.6</ghg:NotMeasuredBlowdownVentMethaneCarbonDioxideEquivalent>

<ghg:NotMeasuredIsolationValveCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:NotMeasuredIsolationValveCarbonDioxideEmissions>

<ghg:NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent massUOM="Metric Tons">115.5</ghg:NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent>

<ghg:DidCompressorVentEmissionsToFlare>No</ghg:DidCompressorVentEmissionsToFlare >

<ghg:TotalAnnualCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:TotalAnnualCarbonDioxideEmissions>

<ghg:TotalAnnualMethaneCarbonDioxideEquivalent massUOM="Metric Tons">162.1</ghg:TotalAnnualMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualNitrousCarbonDioxideEquivalent>200</ghg:TotalAnnualNitrousCarbonDioxideEquivalent>

<ghg:TotalTimeInOperatingMode>3000</ghg:TotalTimeInOperatingMode>

<ghg:BlowdownVentsEmissionFactor>2</ghg:BlowdownVentsEmissionFactor>

</ghg:CentrifugalCompressorsModesRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data. This example shows how to report a centrifugal compressor that has dry seals in operating mode.

Example for a Centrifugal Compressor which did not Operate in the Reporting Year

<ghg:CentrifugalCompressorsModesRowDetails>

<ghg:CompressorIdentifier>3</ghg:CompressorIdentifier>

<ghg:WereBammUsedForCompressor>No</ghg:WereBammUsedForCompressor>

<ghg:SealType>Dry</ghg:SealType>

<ghg:MeasuredIsolationValveCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:MeasuredIsolationValveCarbonDioxideEmissions>

<ghg:MeasuredIsolationValveMethaneCarbonDioxideEquivalent massUOM="Metric Tons">115.5</ghg:MeasuredIsolationValveMethaneCarbonDioxideEquivalent>

<ghg:DidCompressorVentEmissionsToFlare>No</ghg:DidCompressorVentEmissionsToFlare>

<ghg:TotalAnnualCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:TotalAnnualCarbonDioxideEmissions>

<ghg:TotalAnnualMethaneCarbonDioxideEquivalent massUOM="Metric Tons">115.5</ghg:TotalAnnualMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</ghg:TotalAnnualNitrousCarbonDioxideEquivalent>

<ghg:TotalTimeInShutdownMode>3000</ghg:TotalTimeInShutdownMode>

<ghg:IsolationValveEmissionFactor>4</ghg:IsolationValveEmissionFactor>

</ghg:CentrifugalCompressorsModesRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data. This example shows how to report a centrifugal compressor that did not operate in the reporting year.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Reciprocating Compressors

This topic provides a step-by-step description of how to report reciprocating compressor information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

- Onshore natural gas transmission compression

- Underground natural gas storage

- Liquefied natural gas (LNG) storage

- LNG import and export equipment

Reciprocating Compressors Details Schema Diagram

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For reciprocating compressors, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all reciprocating compressors combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any reciprocating compressors subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” depends upon the industry segment:

- For reciprocating compressors in the onshore petroleum and natural gas production industry segment only, the value to report for “TotalCarbonDioxideEmissions” equals the value reported for the data element “TotalAnnualCompressorCarbonDioxideEmissions”.

- For reciprocating compressors in all other industry segments, the value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for data element “TotalAnnualCarbonDioxideEmissions”.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” depends upon the industry segment:

- For reciprocating compressors in the onshore petroleum and natural gas production industry segment only, the value to report for “TotalMethaneCarbonDioxideEquivalent” equals the value reported for the data element “TotalAnnualCompressorMethaneCarbonDioxideEquivalent”.

- For reciprocating compressors in all other industry segments, the value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for data element “TotalAnnualMethaneCarbonDioxideEquivalent”.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” depends upon the industry segment:

- For reciprocating compressors in the onshore petroleum and natural gas production industry segment only, report “0” for “TotalNitrousCarbonDioxideEquivalent”.

- For reciprocating compressors in all other industry segments, the value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for data element “TotalAnnualNitrousCarbonDioxideEquivalent”.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any reciprocating compressors subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- For all reciprocating compressors in the onshore petroleum and natural gas production industry segment only (refer to Equation W-25 of 98.233), report the following:

- Total annual CO2 emissions for all reciprocating compressors from all modes of operation combined in metric tons. [98.236(c)(14)(v)(B)]

- Total annual CH4 emissions for all reciprocating compressors from all modes of operation combined in metric tons CO2e. [98.236(c)(14)(v)(B)]

- Total count of compressors. [98.236(c)(14)(v)(A)]

- For all applicable industry segments except the onshore petroleum and natural gas production industry segment, a collection of data elements to report if the facility had any reciprocating compressors subject to reporting under 98.232 in the reporting year.

Reciprocating Compressors Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ReciprocatingCompressorsDetails | Parent Element: A collection of data elements to report for reciprocating compressors. [98.236(c)(13)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all reciprocating compressors combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any reciprocating compressors subject to reporting under 98.232 during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all reciprocating compressors combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any reciprocating compressors subject to reporting under 98.232 during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all reciprocating compressors combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any reciprocating compressors subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2 and CH4 emissions from all reciprocating compressors combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any reciprocating compressors subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveReciprocatingCompressors | Indicate (Yes/No) if the facility had any reciprocating compressors subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| TotalAnnualCompressorCarbonDioxideEmissions | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, the total annual compressor CO2 emissions in metric tons for all compressors combined. [98.236(c)(14)(v)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualCompressorMethaneCarbonDioxideEquivalent | Conditionally Required: For the onshore petroleum and natural gas production industry segment only, the total annual compressor CH4 emissions in metric tons CO2e for all compressors combined. [98.236(c)(14)(v)(B)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CompressorCount | Specify the count of compressors. [98.236(c)(14)(v)(A)] |
| ReciprocatingCompressorsModesDetails | Parent Element (Conditionally Required): For all applicable industry segments except the onshore petroleum and natural gas production industry segment, a collection of data elements to report if the facility had any reciprocating compressors subject to reporting under 98.232 in the reporting year. |

Example for Reciprocating Compressors Details

<ghg:ReciprocatingCompressorsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">2.2</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1455.2</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">15.1</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">11472.5</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveReciprocatingCompressors>Yes</ghg:DoesFacilityHaveCentrifugalCompressors>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:ReciprocatingCompressorsModesDetails>

<See example for Reciprocating Compressors Modes Details>

</ghg:ReciprocatingCompressorsModesDetails>

</ghg:ReciprocatingCompressorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for applicable industry segments other than the onshore petroleum and natural gas production industry segment.

Example for Reciprocating Compressors Details for Onshore Petroleum and Natural Gas Production

<ghg:ReciprocatingCompressorsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">55.5</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">7644.7</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">7715.1</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveReciprocatingCompressors>Yes</ghg:DoesFacilityHaveCentrifugalCompressors>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:CompressorCount>5</ghg:CompressorCount>

<ghg:TotalAnnualCompressorCarbonDioxideEmissions massUOM="Metric Tons">55.5</ghg:TotalAnnualCompressorCarbonDioxideEmissions>

<ghg:TotalAnnualCompressorMethaneCarbonDioxideEquivalent massUOM="Metric Tons">7644.7</ghg:TotalAnnualCompressorMethaneCarbonDioxideEquivalent>

</ghg:ReciprocatingCompressorsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for the onshore petroleum and natural gas production industry segment only.

Reciprocating Compressors Modes Details (Part 1 of 3) Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For reciprocating compressors in the onshore natural gas processing, onshore natural gas transmission compression, underground natural gas storage, liquefied natural gas (LNG) storage, LNG import and export equipment industry segments, if the facility had any reciprocating compressors subject to reporting under 98.232 during the reporting year, then report the following for each reciprocating compressor:

- The unique identity of the compressor.

- Whether BAMM was used to estimate emissions from the compressor.

- For reciprocating compressors in operating mode, report the following:

  - Annual throughput using an engineering calculation based on best available data in MMscf. [98.236(c)(13)(i)(C)]

  - Whether the blowdown vents were manifolded to rod packing vents.

  - Rod packing emissions values:

Note 1:  Report either measured rod packing CO2 and CH4 emissions or not measured rod packing CO2 and CH4 emissions values, but not both pairs of data or one GHG from each pair.

Note 2:  If a reciprocating compressor was measured and had no emissions for a particular GHG while in operating mode during the reporting year, then report “0” for that GHG’s measured rod packing emissions.

Note 3:  If a reciprocating compressor was never in the operating mode during the reporting year, then report “0” for not measured rod packing CO2 and CH4 emissions.

    - Measured rod packing CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(i)(C)]

    - Measured rod packing CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(i)(C)]

OR

    - Not measured rod packing CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(i)(C)]

    - Not measured rod packing CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(i)(C)]

- For reciprocating compressors with blowdown vents which were not manifolded to rod packing vents, when in operating mode, report the following:

Note 1:  Report either measured blowdown vent CO2 and CH4 emissions or not measured blowdown vent CO2 and CH4 emissions, but not both pairs of data, or one GHG from each pair.

Note 2:  If a reciprocating compressor was measured and had no emissions for a particular GHG while in operating mode during the reporting year, then report “0” for that GHG’s measured blowdown vent emissions.

Note 3:  If a reciprocating compressor was never in the operating mode during the reporting year, then report “0” for not measured blowdown vent CO2 and CH4 emissions.

    - Measured blowdown vent CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

    - Measured blowdown vent CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

OR

    - Not measured blowdown vent CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

    - Not measured blowdown vent CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

Reciprocating Compressors Modes Details (Part 2 of 3) Schema Diagram

Note: Please see page 4 of this document for more information on conditionally required elements.

- For reciprocating compressors with blowdown vents which were not manifolded to rod packing vents, when in standby pressurized mode (refer to Equations W-26 through W-28), report the following:

Note 1:  Report either measured blowdown vent CO2 and CH4 emissions or not measured blowdown vent CO2 and CH4 emissions, but not both pairs of data, or one GHG from each pair.

Note 2:  If a reciprocating compressor was measured and had no emissions for a particular GHG while in standby pressurized mode during the reporting year, then report “0” for that GHG’s measured blowdown vent emissions.

Note 3:  If a reciprocating compressor was never in the standby pressurized mode during the reporting year, then report “0” for not measured blowdown vent CO2 and CH4 emissions.

  - Measured blowdown vent CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

  - Measured blowdown vent CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

OR

  - Not measured blowdown vent CO2 emissions in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

  - Not measured blowdown vent CH4 emissions in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(ii)(C)]

- For reciprocating compressors in not operating, depressurized mode (refer to Equations W-26 through W-28 of 98.233), report the following:

Note 1:  Report either measured isolation valve leakage CO2 and CH4 emissions or not measured isolation valve leakage CO2 and CH4 emissions, but not both pairs of data, or one GHG from each pair.

Note 2:  If a reciprocating compressor was measured and had no emissions for a particular GHG while in not operating, depressurized mode during the reporting year, then report “0” for that GHG’s measured isolation valve leakage emissions.

Note 3:  If a reciprocating compressor was never in the not operating, depressurized mode during the reporting year, then report “0” for not measured isolation valve leakage CO2 and CH4 emissions

  - Measured isolation valve leakage CO2 emissions in not operating, depressurized mode in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(iii)(C)]

  - Measured isolation valve leakage CH4 emissions in not operating, depressurized mode in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(iii)(C)]

OR

  - Not measured isolation valve leakage CO2 emissions in not operating, depressurized mode in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(iii)(C)]

  - Not measured isolation valve leakage CH4 emissions in not operating, depressurized mode in metric tons CO2e (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(iii)(C)]

Reciprocating Compressors Modes Details (Part 3 of 3) Schema Diagram

Note: Boxed data elements are required for all compressors.

- For all reciprocating compressors, report if the compressor vented emissions to a flare.

- For reciprocating compressors which vented emissions to a flare, report the following:

- Total annual CO2 emissions from flaring for all modes of operation combined in metric tons.  [98.236(c)]

- Total annual CH4 emissions from flaring for all modes of operation combined in metric tons CO2e.  [98.236(c)]

- Total annual N2O emissions from flaring for all modes of operation combined in metric tons CO2e.  [98.236(c)]

- For each reciprocating compressor, report total annual CO2 emissions from all modes of operation combined in metric tons (refer to Equation W-26 through W-28 of 98.233). [98.236(c)(14)(iv)]  The value to report for “TotalAnnualCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “MeasuredRodPackingCarbonDioxideEmissions” for each reciprocating compressor.

- “NotMeasuredRodPackingCarbonDioxideEmissions” for each reciprocating compressor.

- “MeasuredBlowdownVentOperatingCarbonDioxideEmissions” for each reciprocating compressor.

- “NotMeasuredBlowdownVentOperatingCarbonDioxideEmissions” for each reciprocating compressor.

- “MeasuredBlowdownVentStandbyCarbonDioxideEmissions” for each reciprocating compressor.

- “NotMeasuredBlowdownVentStandbyCarbonDioxideEmissions” for each reciprocating compressor.

- “MeasuredIsolationValveCarbonDioxideEmissions” for each reciprocating compressor.

- “NotMeasuredIsolationValveCarbonDioxideEmissions” for each reciprocating compressor.

- “FlaringCarbonDioxideEmissions” for each reciprocating compressor.

- For each reciprocating compressor, report total annual CH4 emissions from all modes of operation combined in metric tons CO2e (refer to Equation W-27 and Equation W-28 of 98.233). [98.236(c)(14)(iv)]  The value to report for “TotalAnnualMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “MeasuredRodPackingMethaneEmissions” for each reciprocating compressor.

- “NotMeasuredRodPackingMethaneEmissions” for each reciprocating compressor.

- “MeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “NotMeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “MeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “NotMeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “MeasuredIsolationValveMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- “FlaringMethaneCarbonDioxideEquivalent” for each reciprocating compressor.

- For each reciprocating compressor, report total annual N2O emissions from all modes of operation combined in metric tons CO2e. [98.236(c)]  The value to report for “TotalAnnualNitrousCarbonDioxideEquivalent” equals the value reported for the data element “FlaringNitrousCarbonDioxideEquivalent”.

Note:  If a compressor had no emissions in all operating modes, report “0”.

- For reciprocating compressors rod packing emissions with or without a vent in operating mode, report total time in operating mode (hours). [98.236(c)(14)(i)(B)]

- For reciprocating compressors blowdown vents not manifold to rod packing vents, in standby pressurized mode, report the total time in standby, pressurized mode (hours). [98.236(c)(14)(ii)(A)]

- For reciprocating compressors blowdown vents not manifold to rod packing vents, report the emission factor for blowdown vents (cubic feet per hour). [98.236(c)(14)(ii)(B)]

- For reciprocating compressors in not operating, depressurized mode, report the total time the compressor is in not operating, depressurized mode (hours). [98.236(c)(14)(iii)(A)]

- For reciprocating compressors in not operating, depressurized mode, report the reporter emission factor for isolation valve emissions (cubic feet per hour). [98.236(c)(14)(iii)(B)]

Reciprocating Compressors Modes Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ReciprocatingCompressorsModesDetails | Parent Element (Conditionally Required): For all applicable industry segments except the onshore petroleum and natural gas production industry segment, a collection of data elements to report if the facility had any reciprocating compressors subject to reporting under 98.232 in the reporting year. |
| ReciprocatingCompressorsModesRowDetails | Parent Element: A collection of data elements to report for each reciprocating compressor. |
| CompressorIdentifier | Unique name or ID number for the reciprocating compressor. |
| WereBammUsedForCompressor | Indicate (Yes/No) if BAMM were used to calculate GHG emissions for the specified reciprocating compressor. |
| AnnualThroughput | Conditionally Required: For reciprocating compressors in operating mode, the annual throughput using an engineering calculation based on best available data in million SCF. [98.236(c)(14)(i)(A)] Set the units of measure to "MMscf" in the attribute volUOM. |
| WereBlowdownVentsManifolded | Conditionally Required: Report only if compressor was in operating mode or standy pressurized mode at some point suring the year. Indicate whether the blowdown vents were manifolded to rod packing vents for the specified compressor. Yes No |
| MeasuredRodPackingCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in operating mode, the measured rod packing CO2 emissions for the specified reciprocating compressor, when in operating mode (refer to Equation W-26 of 98.233). [98.236(c)(14)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both rod packing CO2 emissions and rod packing CH4 emissions as both measured or both not measured. |
| MeasuredRodPackingMethaneEmissions | Conditionally Required: For reciprocating compressors in operating mode, the measured rod packing CH4 emissions for the specified reciprocating compressor, when in operating mode, in metric tons CO2e (refer to Equation W-26 of 98.233). [98.236(c)(14)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both rod packing CO2 emissions and rod packing CH4 emissions as both measured or both not measured. |
| NotMeasuredRodPackingCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in operating mode, the not measured rod packing CO2 emissions for the specified reciprocating compressor, when in operating mode (refer to Equations W-27 and W-28 of 98.233). [98.236(c)(14)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both rod packing CO2 emissions and rod packing CH4 emissions as both measured or both not measured. |
| NotMeasuredRodPackingMethaneEmissions | Conditionally Required: For reciprocating compressors in operating mode, the not measured rod packing CH4 emissions for the specified reciprocating compressor, when in operating mode, in metric tons CO2e (refer to Equations W-26 and W-28 of 98.233). [98.236(c)(14)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both rod packing CO2 emissions and rod packing CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentOperatingCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in operating mode with blowdown vents which were not manifolded to rod packing vents, the measured blowdown vent CO2 emissions for the specified reciprocating compressor, when in operating mode (refer to Equations W-27 and W-28 of 98.233). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in operating mode with blowdown vents which were not manifolded to rod packing vents, the measured blowdown vent CH4 emissions for the specified reciprocating compressor, when in operating mode, in metric tons CO2e (refer to Equations W-27 and W-28 of 98.233). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentOperatingCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in operating mode with blowdown vents which were not manifolded to rod packing vents, the not measured blowdown vent CO2 emissions for the specified reciprocating compressor, when in operating mode (refer to Equations W-27 and W-28 of 98.233). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in operating mode with blowdown vents which were not manifolded to rod packing vents, the not measured blowdown vent CH4 emissions for the specified reciprocating compressor, when in operating mode, in metric tons CO2e (refer to Equations W-27 and W-28 of 98.233). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentStandbyCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in standby pressurized mode, the measured blowdown vent CO2 emissions for the specified reciprocating compressor vents not manifold to rod packing vents (refer to Equations W-26 through W-28). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in standby pressurized mode, the measured blowdown vent CH4 emissions for the specified reciprocating compressor vents not manifold to rod packing vents in metric tons of CO2e (refer to Equations W-26 through W-28). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentStandbyCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in standby pressurized mode, the not measured blowdown vent CO2 emissions for the specified reciprocating compressor vents not manifold to rod packing vents (refer to Equations W-26 through W-28). Report for the specified compressor. [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| NotMeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in standby pressurized mode, the not measured blowdown vent CH4 emissions for the specified reciprocating compressor vents not manifold to rod packing vents in metric tons of CO2e (refer to Equations W-26 through W-28). [98.236(c)(14)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both blowdown vent CO2 emissions and blowdown vent CH4 emissions as both measured or both not measured. |
| MeasuredIsolationValveCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, the measured isolation valve leakage CO2 emissions for the specified reciprocating compressor. [98.236(c)(14)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| MeasuredIsolationValveMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, the measured isolation valve leakage CH4 emissions for the specified reciprocating compressor in metric tons of CO2e. [98.236(c)(14)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| NotMeasuredIsolationValveCarbonDioxideEmissions | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, the not measured isolation valve leakage CO2 emissions for the specified reciprocating compressor. [98.236(c)(14)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, the not measured isolation valve leakage CH4 emissions for the specified reciprocating compressor in metric tons of CO2e. [98.236(c)(14)(iii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: If the compressor has no emissions in this mode, report “0”. Report both isolation valve CO2 emissions and isolation valve CH4 emissions as both measured or both not measured. |
| DidCompressorVentEmissionsToFlare | Indicate (Yes/No) if the compressor vented emissions to a flare in the reporting year. |
| TotalAnnualFlaringCarbonDioxideEmissions | Conditionally Required: If the compressor vented emissions to flares, the total annual CO2 emissions from flaring for all modes of operation combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualFlaringMethaneCarbonDioxideEquivalent | Conditionally Required: If the compressor vented emissions to flares, the total annual CH4 emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualFlaringNitrousCarbonDioxideEquivalent | Conditionally Required: If the compressor vented emissions to flares, the total annual N2O emissions from flaring for all modes of operation combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualCarbonDioxideEmissions | For each reciprocating compressor, the total annual compressor CO2 emissions from all modes of operation combined for the specified compressor. [98.236(c)(14)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualMethaneCarbonDioxideEquivalent | For each reciprocating compressor, the total annual compressor CH4 emissions from all modes of operation combined for the specified compressor, in metric tons CO2e. [98.236(c)(14)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalAnnualNitrousCarbonDioxideEquivalent | For each reciprocating compressor, the total annual compressor N2O emissions from all modes of operation combined for the specified compressor, in metric tons CO2e. [98.236(c)(14)(iv)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalTimeInOperatingMode | Conditionally Required: For reciprocating compressors rod packing emissions with or without a vent in operating mode, report total time in operating mode (hours). [98.236(c)(14)(i)(B)] |
| TotalTimeInStandbyMode | Conditionally Required: For reciprocating compressors blowdown vents not manifold to rod packing vents, in standby pressurized mode, report the total time in standby, pressurized mode (hours). [98.236(c)(14)(ii)(A)] |
| BlowdownVentsEmissionFactor | Conditionally Required: For reciprocating compressors blowdown vents not manifold to rod packing vents, report the emission factor for blowdown vents (cubic feet per hour). [98.236(c)(14)(ii)(B)] |
| TotalTimeInNotOperatingMode | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, report the total time the compressor is in not operating, depressurized mode (hours). [98.236(c)(14)(iii)(A)] |
| IsolationValveEmissionsFactor | Conditionally Required: For reciprocating compressors in not operating, depressurized mode, report the reporter emission factor for isolation valve emissions (cubic feet per hour). [98.236(c)(14)(iii)(B)] |

Example for a Reciprocating Compressor in Operating and Standby, Pressurized Modes

<ghg:ReciprocatingCompressorsModesDetails>

<ghg:ReciprocatingCompressorsModesRowDetails>

<ghg:CompressorIdentifier>1</ghg:CompressorIdentifier>

<ghg:WereBammUsedForCompressor>No</ghg:WereBammUsedForCompressor>

<ghg:AnnualThroughput volUOM="MMscf">581563</ghg:AnnualThroughput>

<ghg:WereBlowdownVentsManifolded>No</ghg: WereBlowdownVentsManifolded>

<ghg:MeasuredRodPackingCarbonDioxideEmissions massUOM="Metric Tons">0.0</ghg:MeasuredRodPackingCarbonDioxideEmissions>

<ghg:MeasuredRodPackingMethaneEmissions massUOM="Metric Tons">13.2</ghg:MeasuredRodPackingMethaneEmissions>

<ghg:MeasuredBlowdownVentOperatingCarbonDioxideEmissions massUOM="Metric Tons">0.2</ghg:MeasuredBlowdownVentOperatingCarbonDioxideEmissions>

<ghg:MeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">133.8</ghg:MeasuredBlowdownVentOperatingMethaneCarbonDioxideEquivalent>

<ghg:MeasuredBlowdownVentStandbyCarbonDioxideEmissions massUOM="Metric Tons">0.1</ghg:MeasuredBlowdownVentStandbyCarbonDioxideEmissions>

<ghg:MeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent massUOM="Metric Tons">55.8</ghg:MeasuredBlowdownVentStandbyMethaneCarbonDioxideEquivalent>

<ghg:NotMeasuredIsolationValveCarbonDioxideEmissions massUOM="Metric Tons">0.9</ghg:NotMeasuredIsolationValveCarbonDioxideEmissions>

<ghg:NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent massUOM="Metric Tons">626.2</ghg:NotMeasuredIsolationValveMethaneCarbonDioxideEquivalent>

<ghg:DidCompressorVentEmissionsToFlare>Yes</ghg:DidCompressorVentEmissionsToFlare >

<ghg:TotalAnnualFlaringCarbonDioxideEmissions massUOM="Metric Tons">3.4</ghg:TotalAnnualFlaringCarbonDioxideEmissions>

<ghg:TotalAnnualFlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">113.1</ghg:TotalAnnualFlaringMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualFlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">1.6</ghg:TotalAnnualFlaringNitrousCarbonDioxideEquivalent >

<ghg:TotalAnnualCarbonDioxideEmissions massUOM="Metric Tons">1.3</ghg:TotalAnnualCarbonDioxideEmissions>

<ghg:TotalAnnualMethaneCarbonDioxideEquivalent massUOM="Metric Tons">829.0</ghg:TotalAnnualMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualNitrousCarbonDioxideEquivalent massUOM="Metric Tons">1.6</ghg:TotalAnnualNitrousCarbonDioxideEquivalent>

<ghg:TotalTimeInOperatingMode>3000</ghg:TotalTimeInOperatingMode>

<ghg:TotalTimeInStandbyMode>1000</ghg:TotalTimeInStandbyMode>

<ghg:BlowdownVentsEmissionFactor>6</ghg:BlowdownVentsEmissionFactor>

</ghg:ReciprocatingCompressorsModesRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for applicable industry segments other than the onshore petroleum and natural gas production industry segment.

Example for a Reciprocating Compressor which did not Operate in the Reporting Year

<ghg:ReciprocatingCompressorsModesRowDetails>

<ghg:CompressorIdentifier>2</ghg:CompressorIdentifier>

<ghg:WereBammUsedForCompressor>No</ghg:WereBammUsedForCompressor>

<ghg:MeasuredIsolationValveCarbonDioxideEmissions massUOM="Metric Tons">0.9</ghg:MeasuredIsolationValveCarbonDioxideEmissions>

<ghg:MeasuredIsolationValveMethaneCarbonDioxideEquivalent massUOM="Metric Tons">626.2</ghg:MeasuredIsolationValveMethaneCarbonDioxideEquivalent>

<ghg:DidCompressorVentEmissionsToFlare>No</ghg:DidCompressorVentEmissionsToFlare >

<ghg:TotalAnnualCarbonDioxideEmissions massUOM="Metric Tons">0.9</ghg:TotalAnnualCarbonDioxideEmissions>

<ghg:TotalAnnualMethaneCarbonDioxideEquivalent massUOM="Metric Tons">626.2</ghg:TotalAnnualMethaneCarbonDioxideEquivalent>

<ghg:TotalAnnualNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</ghg:TotalAnnualNitrousCarbonDioxideEquivalent>

<ghg:TotalTimeInNotOperatingMode>100</ghg:TotalTimeInNotOperatingMode>

<ghg:IsolationValveEmissionsFactor>2</ghg:IsolationValveEmissionsFactor>

</ghg:ReciprocatingCompressorsModesRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data for applicable industry segments other than the onshore petroleum and natural gas production industry segment.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Other Emissions from Equipment Leaks Estimated Using Emission Factors

This topic provides a step-by-step description of how to report other emissions from equipment leaks estimated using emissions factors for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Onshore natural gas processing

- Onshore natural gas transmission compression

- Underground natural gas storage

- Liquefied natural gas (LNG) storage

- LNG import and export equipment

- Natural gas distribution

Other Emissions From Equipment Leaks Details Schema Diagram

Conditionally

Required

Conditionally

Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For other emissions from equipment leaks estimated using emission factors, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all reciprocating compressors combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note 1:  Report “0” if the facility did not have any equipment leaks subject to reporting under 98.232 during the reporting year.  Note 2:  The natural gas distribution industry segment must report “0” here and report equipment leaks under the local distribution company’s source type.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “CarbonDioxideEmissions” for each component type that used emissions factors for estimating emissions for equipment leaks found in each leak survey.

- “CarbonDioxideEmissions” for each component type that used emissions factors for estimating emissions for equipment leaks calculated using population counts and factors.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “MethaneCarbonDioxideEquivalent” for each component type that used emissions factors for estimating emissions for equipment leaks found in each leak survey.

- “MethaneCarbonDioxideEquivalent” for each component type that used emissions factors for estimating emissions for equipment leaks calculated using population counts and factors.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether the facility had any equipment leaks subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

- A collection of data elements to report if the facility had any component types that used emission factors for estimating emissions for equipment leaks found in each leak survey in the reporting year.

- For applicable industry segments other than natural gas distribution, a collection of data elements to report if any component type (major equipment type for onshore production) used emission factors for estimating emissions for equipment leaks calculated using population counts and factors in the reporting year.

- For onshore production (refer to  98.230(a)(2)), a collection of data elements related to methodology one major equipment type and count (see Table W-1B and Table W-1C) by facility.

Other Emissions From Equipment Leaks Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| OtherEmissionsFromEquipmentLeaksDetails | Parent Element: A collection of data elements to report for other emissions from equipment leaks estimated using emission factors. [98.236(c)(15)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all equipment leaks combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note 1: Report “0” if the facility did not have any applicable equipment leaks subject to reporting under 98.232 during the reporting year. Note 2: The natural gas distribution industry segment must report “0” here and report equipment leaks under the local distribution company’s source type. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all equipment leaks combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note 1: Report “0” if the facility did not have any applicable equipment leaks subject to reporting under 98.232 during the reporting year. Note 2: The natural gas distribution industry segment must report “0” here and report equipment leaks under the local distribution company’s source type. |
| TotalCarbonDioxideEquivalent | Total combined CO2 and CH4 emissions for all equipment leaks combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note 1: Report “0” if the facility did not have any applicable equipment leaks subject to reporting under 98.232 during the reporting year. Note 2: The natural gas distribution industry segment must report “0” here and report equipment leaks under the local distribution company’s source type. |
| DoesFacilityHaveEquipmentLeaks | Indicate (Yes/No) if the facility had any equipment leaks subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| EstimatingEmissionsFoundInLeakSurveyDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any component types that used emission factors for estimating emissions for equipment leaks found in each leak survey in the reporting year. |
| EstimatingEmissionsUsingPopulationCountsDetails | Parent Element (Conditionally Required): For applicable industry segments other than natural gas distribution, a collection of data elements to report if any component type (major equipment type for onshore production) used emission factors for estimating emissions for equipment leaks calculated using population counts and factors in the reporting year. |
| ComponentCountMethodologyOneDetails | Parent Element (Conditionally Required): For onshore production (refer to 98.230(a)(2)), a collection of data elements related to methodology one major equipment type and count (see Table W-1B and Table W-1C) by facility. |

Example for Other Emissions From Equipment Leaks Details

<ghg:OtherEmissionsFromEquipmentLeaksDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">7.7</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">5338.9</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">5346.6</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveEquipmentLeaks>Yes</ghg:DoesFacilityHaveEquipmentLeaks>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:EstimatingEmissionsFoundInLeakSurveyDetails>

<See example for Estimating Emissions Found in Leak Survey Details>

</ghg:EstimatingEmissionsFoundInLeakSurveyDetails>

<ghg:EstimatingEmissionsUsingPopulationCountsDetails>

<See example for Estimating Emissions Using Population Counts Details>

</ghg:EstimatingEmissionsUsingPopulationCountsDetails>

<ghg:ComponentCountMethodologyOneDetails>

<See example for Component Count Methodology One Details>

</ghg:ComponentCountMethodologyOneDetails>

</ghg:OtherEmissionsFromEquipmentLeaksDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Estimating Emissions Found in Leak Survey Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For each component type (major equipment type for the onshore petroleum and natural gas production industry segment) for which there is a leaker emission factor in Table W-2 for the onshore natural gas processing industry segment, Table W-3 for the onshore natural gas transmission compression industry segment, Table W-4 for the underground natural gas storage industry segment, Table W-5 for the liquefied natural gas (LNG) storage industry segment, Table W-6 for the LNG import and export equipment industry segment and Table W-7 for the natural gas distribution industry segment that uses emission factors for estimating emissions (refer to 98.233(q) and (r)), for equipment leaks found in each leak survey (refer to 98.233(q)), report the following [98.236(c)(15)(i)]:

- Date of each complete survey. [98.236(c)(15)(i)(A)]

- For each complete survey date, report the total count of leaks found. [98.236(c)(15)(i)(A)]

- For the onshore natural gas processing industry segment only, report the following:

- The minimum concentration of CO2 (refer to Equation W-30 of 98.233). [98.236(c)(15)(i)(B)]

- The maximum concentration of CO2 (refer to Equation W-30 of 98.233). [98.236(c)(15)(i)(B)]

- The minimum concentration of CH4 (refer to Equation W-30 of 98.233). [98.236(c)(15)(i)(B)]

- The maximum concentration of CH4 (refer to Equation W-30 of 98.233). [98.236(c)(15)(i)(B)]

For applicable industry segments other than natural gas distribution, report the annual CO2 and CH4 emissions, in metric tons CO2e for each gas (refer to parameter GHGi in Equation W-30 of 98.233), by component type. [98.236(c)(15)(i)(C)]

Estimating Emissions Found in Leak Survey Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| EstimatingEmissionsFoundInLeakSurveyDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any component types that used emission factors for estimating emissions for equipment leaks found in each leak survey in the reporting year. |
| EstimatingEmissionsFoundInLeakSurveyRowDetails | Parent Element: A collection of data elements to report for each component type that uses emission factors for estimating emissions for equipment leaks found in each leak survey. |
| ComponentType | Component type for the equipment leak. See list of allowable values. [98.236(c)(15)] Compressor Components, Gas Service - Valve Compressor Components, Gas Service - Connector Compressor Components, Gas Service - Open-Ended Line Compressor Components, Gas Service - Pressure Relief Valve Compressor Components, Gas Service - Meter Non-compressor Components, Gas Service - Valve Non-compressor Components, Gas Service - Connector Non-compressor Components, Gas Service - Open-Ended Line Non-compressor Components, Gas Service - Pressure Relief Valve Non-compressor Components, Gas Service - Meter Storage Station, Gas Service - Valve Storage Station, Gas Service - Connector Storage Station, Gas Service - Open-Ended Line Storage Station, Gas Service - Pressure Relief Valve Storage Station, Gas Service - Meter LNG Storage, LNG Service - Valve LNG Storage, LNG Service - Pump Seal LNG Storage, LNG Service - Connector LNG Storage, LNG Service - Other LNG Storage, LNG Terminal - Valve LNG Storage, LNG Terminal - Pump Seal LNG Storage, LNG Terminal - Connector LNG Storage, LNG Terminal - Other LDC, T-D Stations - Connector LDC, T-D Stations - Block Valve LDC, T-D Stations - Control Valve LDC, T-D Stations - Pressure Relief Valve LDC, T-D Stations - Orifice Meter LDC, T-D Stations - Regulator LDC, T-D Stations - Open-ended Line |
| FirstCompleteSurveyDate | Date of the first complete survey (YYYY-MM-DD). [98.236(c)(15)(i)(A)] |
| FirstSurveyLeaksFound | Total count of leaks found in the first complete survey. [98.236(c)(15)(i)(A)] Note: Report “0” if there were no leaks found. |
| SecondCompleteSurveyDate | Conditionally Required: If a second survey was completed, the date of the second complete survey (YYYY-MM-DD). [98.236(c)(15)(i)(A)] |
| SecondSurveyLeaksFound | Conditionally Required: If a second survey was completed, the total count of leaks found in the second complete survey. [98.236(c)(15)(i)(A)] Note: Report “0” if there were no leaks found. |
| ThirdCompleteSurveyDate | Conditionally Required: If a third survey was completed, the date of the third complete survey (YYYY-MM-DD). [98.236(c)(15)(i)(A)] |
| ThirdSurveyLeaksFound | Conditionally Required: If a third survey was completed, the total count of leaks found in the third complete survey. [98.236(c)(15)(i)(A)] Note: Report “0” if there were no leaks found. |
| FourthCompleteSurveyDate | Conditionally Required: If a fourth survey was completed, the date of the fourth complete survey (YYYY-MM-DD). [98.236(c)(15)(i)(A)] |
| FourthSurveyLeaksFound | Conditionally Required: If a fourth survey was completed, the total count of leaks found in the fourth complete survey. [98.236(c)(15)(i)(A)] Note: Report “0” if there were no leaks found. |
| MinimumCarbonDioxideConcentration | Conditionally Required: For the onshore natural gas processing industry segment only, for the range of concentration of CO2 in the complete surveys, the minimum concentration of CO2 (volumetric fraction). [98.236(c)(15)(i)(B)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MaximumCarbonDioxideConcentration | Conditionally Required: For the onshore natural gas processing industry segment only, for the range of concentration of CO2 in the complete surveys, the maximum concentration of CO2 (volumetric fraction). [98.236(c)(15)(i)(B)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MinimumMethaneConcentration | Conditionally Required: For the onshore natural gas processing industry segment only, for the range of concentration of CH4 in the complete surveys, the minimum concentration of CH4 (volumetric fraction). [98.236(c)(15)(i)(B)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| MaximumMethaneConcentration | Conditionally Required: For the onshore natural gas processing industry segment only, for the range of concentration of CH4 in the complete surveys, the maximum concentration of CH4 (volumetric fraction). [98.236(c)(15)(i)(B)] Set the units of measure to “volumetric fraction” in the attribute fractionUOM. |
| CarbonDioxideEmissions | Conditionally Required: For applicable industry segments other than natural gas distribution, report the total CO2 emissions for the specified leak source in metric tons. [98.236(c)(15)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| MethaneCarbonDioxideEquivalent | Conditionally Required: For applicable industry segments other than natural gas distribution, report the total CH4 emissions for the specified leak source in metric tons CO2e. [98.236(c)(15)(i)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Estimating Emissions Found in Leak Survey Details

<ghg:EstimatingEmissionsFoundInLeakSurveyDetails>

<ghg:EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ghg:ComponentType>Storage Station, Gas Service - Valve</ghg:ComponentType>

<ghg:FirstCompleteSurveyDate>2011-03-13</ghg:FirstCompleteSurveyDate>

<ghg:FirstSurveyLeaksFound>2</ghg:FirstSurveyLeaksFound>

<ghg:SecondCompleteSurveyDate>2011-06-13</ghg:SecondCompleteSurveyDate>

<ghg:SecondSurveyLeaksFound>1</ghg:SecondSurveyLeaksFound>

<ghg:ThirdCompleteSurveyDate>2011-09-13</ghg:ThirdCompleteSurveyDate>

<ghg:ThirdSurveyLeaksFound>0</ghg:ThirdSurveyLeaksFound>

<ghg:FourthCompleteSurveyDate>2011-12-13</ghg:FourthCompleteSurveyDate>

<ghg:FourthSurveyLeaksFound>1</ghg:FourthSurveyLeaksFound>

<ghg:MinimumCarbonDioxideConcentration fractionUOM="volumetric fraction">0.5</ghg:MinimumCarbonDioxideConcentration>

<ghg:MaximumCarbonDioxideConcentration fractionUOM="volumetric fraction">0.85</ghg:MaximumCarbonDioxideConcentration>

<ghg:MinimumMethaneConcentration fractionUOM="volumetric fraction">0.05</ghg:MinimumMethaneConcentration>

<ghg:MaximumMethaneConcentration fractionUOM="volumetric fraction">0.15</ghg:MaximumMethaneConcentration>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">0.0</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">7.2</ghg:MethaneCarbonDioxideEquivalent>

</ghg:EstimatingEmissionsFoundInLeakSurveyRowDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Estimating Emissions Using Population Counts Details Schema Diagram

Note: Data elements boxed in red are required.

For applicable industry segments other than natural gas distribution, if there were any equipment leaks calculated using population counts and factors (refer to 98.233(r)), report the following for each component type [98.236(c)(15)(ii)]:

- The component type (major equipment type for onshore production) for the equipment leak.  See list of allowable values. [98.236(c)(15)]

- Annual CO2 and CH4 emissions, in metric tons CO2e for each gas (refer to Equation W-31 of 98.233). [98.236(c)(15)(ii)(C)]

- Total count for each leak source type.

Estimating Emissions Using Population Counts Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| EstimatingEmissionsUsingPopulationCountsDetails | Parent Element (Conditionally Required): For applicable industry segments other than natural gas distribution, a collection of data elements to report if any component type (major equipment type for onshore production) used emission factors for estimating emissions for equipment leaks calculated using population counts and factors in the reporting year. |
| EstimatingEmissionsUsingPopulationCountsRowDetails | Parent Element: A collection of data elements to report for each component type (major equipment type for onshore production) that used emission factors for estimating emissions for equipment leaks calculated using population counts and factors. |
| ComponentType | Component type (major equipment type for onshore production) for the equipment leak. See list of allowable values. [98.236(c)(15)] Storage wellheads, Gas Service - Connector Storage wellheads, Gas Service - Valve Storage Wellheads, Gas Service - Pressure Relief Valve Storage Wellheads, Gas Service - Open Ended Line LNG Compressor - Vapor Recovery Compressor Onshore, gas service - valve Onshore, gas service - connector Onshore, gas service - open-ended line Onshore, gas service - pressure relief valve Onshore, light crude service - valve Onshore, light crude service - flange Onshore, light crude service - connector Onshore, light crude service - open-ended line Onshore, light crude service - other Onshore, heavy crude service - valve Onshore, heavy crude service - flange Onshore, heavy crude service - connector Onshore, heavy crude service - open-ended line Onshore, heavy crude service - other |
| CarbonDioxideEmissions | CO2 emissions for the specified component type (major equipment type for onshore production) in metric tons. [98.236(c)(15)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| MethaneCarbonDioxideEquivalent | CH4 emissions for the specified component type (major equipment type for onshore production) in metric tons CO2e. [98.236(c)(15)(ii)(C)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalCountForEachLeakSourceType | Total count for each type of leak source. [98.236(c)(15)(ii)(A)] |

Example for Estimating Emissions Using Population Counts Details

<ghg:EstimatingEmissionsUsingPopulationCountsDetails>

<ghg:EstimatingEmissionsUsingPopulationCountsRowDetails>

<ghg:ComponentType>Onshore, gas service - valve</ghg:ComponentType>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">15.9</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">12587.3</ghg:MethaneCarbonDioxideEquivalent>

<ghg:TotalCountForEachLeakSourceType>2</ghg:TotalCountForEachLeakSourceType>

</ghg:EstimatingEmissionsUsingPopulationCountsRowDetails>

</ghg:EstimatingEmissionsUsingPopulationCountsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Component Count Methodology One Details Schema Diagram

Note: Data elements boxed in red are required.

For an onshore production facility with any of the major equipment types listed in Table W-1B or Table W-1C for which equipment leak emissions are calculated using Component Count Methodology 1, report the following [98.236(c)(15)(ii)]:

- Major equipment type. [98.236(c)(15)(ii)(B)]

- Total count for each type of major equipment. [98.236(c)(15)(ii)(B)]

Component Count Methodology One Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ComponentCountMethodologyOneDetails | Parent Element (Conditionally Required): For onshore production, a collection of data elements describing the major equipment for which there were equipment leaks. |
| ComponentCountMethodologyOneRowDetails | Parent Element: A collection of data elements to report for each facility describing the major equipment for which there were equipment leaks. |
| MajorEquipmentType | For onshore production (refer to 98.230(a)(2)), report each major equipment type for which there were equipment leaks (see Table W-1B and Table W-1C) at the facility. Wellheads Separators Meters/piping Compressors In-line heaters Dehydrators Heater-treater Header |
| TotalCountForMajorEquipmentType | For onshore production (refer to 98.230(a)(2)), report the number of each major equipment type. |

Example for Component Count Methodology One Details

<ghg:ComponentCountMethodologyOneDetails>

<ghg:ComponentCountMethodologyOneRowDetails>

<ghg:MajorEquipmentType>Wellheads</ghg:MajorEquipmentType>

<ghg:TotalCountForMajorEquipmentType>3</ghg:TotalCountForMajorEquipmentType>

</ghg:ComponentCountMethodologyOneRowDetails>

<ghg:ComponentCountMethodologyOneRowDetails>

<ghg:MajorEquipmentType>Separators</ghg:MajorEquipmentType>

<ghg:TotalCountForMajorEquipmentType>3</ghg:TotalCountForMajorEquipmentType>

</ghg:ComponentCountMethodologyOneRowDetails>

</ghg:ComponentCountMethodologyOneDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

## Local Distribution Companies

This topic provides a step-by-step description of how to report information for a local distribution company.  This section is applicable to and required for the natural gas distribution industry segment only.

Local Distribution Companies Details (Part 1 of 3) Schema Diagram

Note: Data elements boxed in red are required.

Please see page 4 of this document for more information on conditionally required elements.

For local distribution companies, report the following:

- The CO2 and CH4 emissions totals and the total CO2e emissions for all above grade metering-regulating stations, all below grade metering-regulating stations, all distribution mains and all distribution services combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e and total CO2e emissions).  [98.236(c)]

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “AnnualCarbonDioxideAboveGradeMeteringStations”.

- “AnnualCarbonDioxideBelowGradeMeteringStations”.

- “AnnualCarbonDioxideDistributionMains”.

- “AnnualCarbonDioxideDistributionServices”.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “AnnualMethaneAboveGradeMeteringStations”.

- “AnnualMethaneBelowGradeMeteringStations”.

- “AnnualMethaneDistributionMains”.

- “AnnualMethaneDistributionServices”.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions” and “TotalMethaneCarbonDioxideEquivalent”.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

Local Distribution Companies Details (Part 1 of 3) Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| LocalDistributionCompaniesDetails | Parent Element: A collection of data elements to report for local distribution companies. [98.236(c)(16)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions for all above grade metering-regulating stations, all below grade metering-regulating stations, all distribution mains and all distribution services combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions for all above grade metering-regulating stations, all below grade metering-regulating stations, all distribution mains and all distribution services combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalCarbonDioxideEquivalent | Total combined CO2 and CH4 emissions for all above grade metering-regulating stations, all below grade metering-regulating stations, all distribution mains and all distribution services combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |

Example for Local Distribution Companies Details (Part 1 of 3)

<ghg:LocalDistributionCompaniesDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">806.0</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">561904.0</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">562710.0</ghg:TotalCarbonDioxideEquivalent>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

Local Distribution Companies Details (Part 2 of 3) Schema Diagram

Note: Data elements boxed in red are required.

For local distribution companies, report the following [98.236(c)(16)]:

- Total number of above grade T-D transfer stations in the facility. [98.236(c)(16)(i)]

- Number of years over which all T-D transfer stations will be monitored at least once. [98.236(c)(16)(ii)]

- Number of T-D stations monitored in reporting year. [98.236(c)(16)(iii)]

- Total number of below grade T-D transfer stations in the facility. [98.236(c)(16)(iv)]

- Total number of above grade metering-regulating stations (this count will include above grade T-D transfer stations) in the facility. [98.236(c)(16)(v)]

- Total number of below grade metering-regulating stations (this count will include below grade T-D transfer stations) in the facility. [98.236(c)(16)(vi)]

- Annual CO2 and CH4 emissions, in metric tons CO2e for each gas, from all above grade T-D transfer stations combined. [98.236(c)(16)(xvii)]

- Annual CO2and CH4 emissions, in metric tons CO2e for each gas, from all below grade T-D transfer stations combined. [98.236(c)(16)(xviii)]

- Annual CO2and CH4 emissions, in metric tons CO2e for each gas, from all above grade metering-regulating stations (including T-D transfer stations) combined. [98.236(c)(16)(xix)]

- Annual CO2and CH4 emissions, in metric tons CO2e for each gas, from all below grade metering-regulating stations (including T-D transfer stations) combined. [98.236(c)(16)(xx)]

- Annual CO2and CH4 emissions, in metric tons CO2e for each gas, from all distribution mains combined. [98.236(c)(16)(xxi)]

- Annual CO2and CH4 emissions, in metric tons CO2e for each gas, from all distribution services combined. [98.236(c)(16)(xxi)]

Note:  All data elements are required.  If your facility does not have any metering-regulating stations or transmission-distribution transfer stations, report “0” for the applicable data element(s).

Local Distribution Companies Details (Part 2 of 3) Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| AboveGradeTransferStations | Total number of above grade T-D transfer stations in the facility. [98.236(c)(16)(i)] |
| YearsTransferStationsMonitored | Number of years over which all T-D transfer stations will be monitored at least once. [98.236(c)(16)(ii)] |
| StationsMonitoredInCalendarYear | Number of T-D stations monitored in the reporting year. [98.236(c)(16)(iii)] |
| BelowGradeTransferStations | Total number of below grade T-D transfer stations in the facility. [98.236(c)(16)(iv)] |
| AboveGradeMeteringStations | Total number of above grade metering-regulating stations in the facility (this count will include above grade T-D transfer stations). [98.236(c)(16)(v)] |
| BelowGradeMeteringStations | Total number of below grade metering-regulating stations in the facility (this count will include below grade T-D transfer stations). [98.236(c)(16)(vi)] |
| AnnualCarbonDioxideAboveGradeTransferStations | Annual CO2 emissions from all above grade T-D transfer stations combined in metric tons. [98.236(c)(16)(xvii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneAboveGradeTransferStations | Annual CH4 emissions from all above grade T-D transfer stations combined in metric tons CO2e. [98.236(c)(16)(xvii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualCarbonDioxideBelowGradeTransferStations | Annual CO2 emissions from all below grade T-D transfer stations combined in metric tons. [98.236(c)(16)(xviii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneBelowGradeTransferStations | Annual CH4 emissions from all below grade T-D transfer stations combined in metric tons CO2e. [98.236(c)(16)(xviii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualCarbonDioxideAboveGradeMeteringStations | Annual CO2 emissions from all above grade metering-regulating stations (including T-D transfer stations) combined in metric tons. [98.236(c)(16)(xix)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneAboveGradeMeteringStations | Annual CH4 emissions from all above grade metering-regulating stations (including T-D transfer stations) combined in metric tons CO2e. [98.236(c)(16)(xix)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualCarbonDioxideBelowGradeMeteringStations | Annual CO2 emissions from all below grade metering-regulating stations (including T-D transfer stations) combined in metric tons. [98.236(c)(16)(xx)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneBelowGradeMeteringStations | Annual CH4 emissions from all below grade metering-regulating stations (including T-D transfer stations) combined in metric tons CO2e. [98.236(c)(16)(xx)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualCarbonDioxideDistributionMains | Annual CO2 emissions from all distribution mains combined in metric tons. [98.236(c)(16)(xxi)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneDistributionMains | Annual CH4 emissions from all distribution mains combined in metric tons CO2e. [98.236(c)(16)(xxi)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualCarbonDioxideDistributionServices | Annual CO2 emissions from all distribution services combined in metric tons. [98.236(c)(16)(xxii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| AnnualMethaneDistributionServices | Annual CH4 emissions from all distribution services combined in metric tons CO2e. [98.236(c)(16)(xxii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Local Distribution Companies Details (Part 2 of 3)

<AboveGradeTransferStations>300</AboveGradeTransferStations>

<YearsTransferStationsMonitored>5</YearsTransferStationsMonitored>

<StationsMonitoredInCalendarYear>300</StationsMonitoredInCalendarYear>

<BelowGradeTransferStations>97</BelowGradeTransferStations>

<AboveGradeMeteringStations>1141</AboveGradeMeteringStations>

<BelowGradeMeteringStations>498</BelowGradeMeteringStations>

<AnnualCarbonDioxideAboveGradeTransferStations massUOM="Metric Tons">14.0</AnnualCarbonDioxideAboveGradeTransferStations>

<AnnualMethaneAboveGradeTransferStations massUOM="Metric Tons">940875.0</AnnualMethaneAboveGradeTransferStations>

<AnnualCarbonDioxideBelowGradeTransferStations massUOM="Metric Tons">4.0</AnnualCarbonDioxideBelowGradeTransferStations>

<AnnualMethaneBelowGradeTransferStations massUOM="Metric Tons">247060.0</AnnualMethaneBelowGradeTransferStations>

<AnnualCarbonDioxideAboveGradeMeteringStations massUOM="Metric Tons">1.0</AnnualCarbonDioxideAboveGradeMeteringStations>

<AnnualMethaneAboveGradeMeteringStations massUOM="Metric Tons">444.0</AnnualMethaneAboveGradeMeteringStations>

<AnnualCarbonDioxideBelowGradeMeteringStations massUOM="Metric Tons">3.0</AnnualCarbonDioxideBelowGradeMeteringStations>

<AnnualMethaneBelowGradeMeteringStations massUOM="Metric Tons">2279.0</AnnualMethaneBelowGradeMeteringStations>

<AnnualCarbonDioxideDistributionMains massUOM="Metric Tons">599.0</AnnualCarbonDioxideDistributionMains>

<AnnualMethaneDistributionMains massUOM="Metric Tons">417614.0</AnnualMethaneDistributionMains>

<AnnualCarbonDioxideDistributionServices massUOM="Metric Tons">203.0</AnnualCarbonDioxideDistributionServices>

<AnnualMethaneDistributionServices massUOM="Metric Tons">141567.0</AnnualMethaneDistributionServices>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Local Distribution Companies Details (Part 3 of 3) Schema Diagram

Note: Data elements boxed in red are required.

- Leak factor for meter/regulator run developed in Equation W-32 of 98.233. [98.236(c)(16)(viii)]

- Number of miles of unprotected steel distribution mains. [98.236(c)(16)(ix)]

- Number of miles of protected steel distribution mains. [98.236(c)(16)(x)]

- Number of miles of plastic distribution mains. [98.236(c)(16)(xi)]

- Number of miles of cast iron distribution mains. [98.236(c)(16)(xii)]

- Number of unprotected steel distribution services. [98.236(c)(16)(xiii)]

- Number of protected steel distribution services. [98.236(c)(16)(xiv)]

- Number of plastic distribution services. [98.236(c)(16)(xv)]

- Number of copper distribution services. [98.236(c)(16)(xvi)]

Please note tht all of the listed data elements are required, and that a facility should report “0” for any type of distribution mains or distribution services that are not present at the facility.

Local Distribution Companies Details (Part 3 of 3) Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| MeterRegulatorLeakFactor | Leak factor for meter/regulator run developed in Equation W-32 of 98.233. [98.236(c)(16)(viii)] |
| MilesOfUnprotectedSteelDistributionMains | Number of miles of unprotected steel distribution mains. [98.236(c)(16)(ix)] Set the unit of measure to “Miles” in the attribute distanceUOM. |
| MilesOfProtectedSteelDistributionMains | Number of miles of protected steel distribution mains. [98.236(c)(16)(x)] Set the unit of measure to “Miles” in the attribute distanceUOM. |
| MilesOfPlasticDistributionMains | Number of miles of plastic distribution mains. [98.236(c)(16)(xi)] Set the unit of measure to “Miles” in the attribute distanceUOM. |
| MilesOfCastIronDistributionMains | Number of miles of cast iron distribution mains. [98.236(c)(16)(xii)] Set the unit of measure to “Miles” in the attribute distanceUOM. |
| UnprotectedSteelDistributionServices | Number of unprotected steel distribution services. [98.236(c)(16)(xiii)] |
| ProtectedSteelDistributionServices | Number of protected steel distribution services. [98.236(c)(16)(xiv)] |
| PlasticDistributionServices | Number of plastic distribution services. [98.236(c)(16)(xv)] |
| CopperDistributionServices | Number of copper distribution services. [98.236(c)(16)(xvi)] |

Example for Local Distribution Companies Details (Part 3 of 3)

<ghg:MeterRegulatorLeakFactor>25.34</ghg:MeterRegulatorLeakFactor>

<ghg:MilesOfUnprotectedSteelDistributionMains distanceUOM="Miles">50.56</ghg:MilesOfUnprotectedSteelDistributionMains>

<ghg:MilesOfProtectedSteelDistributionMains distanceUOM="Miles">75.78</ghg:MilesOfProtectedSteelDistributionMains>

<ghg:MilesOfPlasticDistributionMains distanceUOM="Miles">100.91</ghg:MilesOfPlasticDistributionMains>

<ghg:MilesOfCastIronDistributionMains distanceUOM="Miles">125.23</ghg:MilesOfCastIronDistributionMains>

<ghg:UnprotectedSteelDistributionServices>25</ghg:UnprotectedSteelDistributionServices>

<ghg:ProtectedSteelDistributionServices>105</ghg:ProtectedSteelDistributionServices>

<ghg:PlasticDistributionServices>113</ghg:PlasticDistributionServices>

<ghg:CopperDistributionServices>200</ghg:CopperDistributionServices>

</ghg:LocalDistributionCompaniesDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Enhanced Oil Recovery (EOR) Injection Pump Blowdown

This topic provides a step-by-step description of how to report EOR injection pump blowdown information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

EOR Injection Pump Blowdown Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

Report the following for EOR injection pump blowdown:

- The CO2 emissions totals and the total CO2e emissions for all EOR injection pump blowdowns combined in metric tons of CO2e (total CO2 emissions and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the data element “CarbonDioxideEmissions” for each EOR injection pump.

- The value to report for “TotalCarbonDioxideEquivalent” equals the value reported for “TotalCarbonDioxideEmissions”.

- Whether the facility had any enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

EOR Injection Pump Blowdown Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| EnhancedOilRecoveryInjectionPumpBlowdownDetails | Parent Element: A collection of data elements to report for enhanced oil recovery injection pump blowdown. [98.236(c)(17)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all EOR injection pumps combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year. |
| TotalCarbonDioxideEquivalent | Total CO2 emissions from all EOR injection pumps combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year. |
| DoesFacilityHaveEorInjectionPumpBlowdown | Indicate (Yes/No) if the facility had enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| UniqueInjectionPumpBlowdownsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year. |

Example for EOR Injection Pump Blowdown Details

<ghg:EnhancedOilRecoveryInjectionPumpBlowdownDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">32824.4</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">32824.4</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveEorInjectionPumpBlowdown>Yes</ghg:DoesFacilityHaveEorInjectionPumpBlowdown>

<ghg:BAMMIndicator>No</ghg:BAMMIndicator>

<ghg:SubstituteDataIndicator>No</ghg:SubstituteDataIndicator>

<ghg:UniqueInjectionPumpBlowdownsDetails>

<See example for Unique Injection Pump Blowdowns Details>

</ghg:UniqueInjectionPumpBlowdownsDetails>

</ghg:EnhancedOilRecoveryInjectionPumpBlowdownDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Unique Injection Pump Blowdowns Details Schema Diagram

Optional

Note: Data elements boxed in red are required.

For each EOR injection pump (refer to Equation W-37 of 98.233), report the following [98.236(c)(17)]:

- A sequential identification number for the EOR injection pump.  Note:  This number is used for reference only in the event that there is a validation message associated with the EOR injection pump data.

- An optional identifier for the EOR injection pump.

- Pump capacity, in barrels per day. [98.236(c)(17)(i)]

- Annual CO2 in metric tons. [98.236(c)(17)(v)]

- Volume of critical phase gas between isolation valves (cubic feet). [98.236(c)(17)(ii)]

- Number of blowdowns per year. [98.236(c)(17)(iii)]

- Critical phase EOR injection gas density (kg / ft3).  [98.236(c)(17)(iv)]

Unique Injection Pump Blowdowns Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| UniqueInjectionPumpBlowdownsDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had enhanced oil recovery injection pump blowdown subject to reporting under 98.232 in the reporting year. |
| UniqueInjectionPumpBlowdownsRowDetails | Parent Element: A collection of data elements to report for each EOR injection pump. |
| UniqueIdentifier | A unique ID for the EOR injection pump. Please number pumps sequentially. |
| EorInjectionPumpBlowdownIdentifier | Optional: Unique name or ID number for the EOR injection pump. |
| PumpCapacity | Pump capacity of the specified EOR injection pump in barrels per day. [98.236(c)(17)(i)] Set the units of measure to “barrels per day” in the attribute volUOM. |
| CarbonDioxideEmissions | Annual CO2 emissions from the specified EOR injection pump in metric tons. [98.236(c)(17)(v)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CriticalPhaseGasVolume | Volume of critical phase gas between isolation valves (cubic feet). [98.236(c)(17)(ii)] |
| NumberOfBlowdowns | Number of blowdowns per year. [98.236(c)(17)(iii)] |
| CriticalPhaseGasDensity | Critical phase EOR injection gas density (kg / ft3). [98.236(c)(17)(iv)] |

Example for Unique Injection Pump Blowdowns Details

<ghg:UniqueInjectionPumpBlowdownsDetails>

<ghg:UniqueInjectionPumpBlowdownsRowDetails>

<ghg:UniqueIdentifier>1</ghg:UniqueIdentifier>

<ghg:PumpCapacity volUOM="barrels per day">1058.3697</ghg:PumpCapacity>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">16462.2</ghg:CarbonDioxideEmissions>

<ghg:CriticalPhaseGasVolume>76</ghg:CriticalPhaseGasVolume>

<ghg:NumberOfBlowdowns>6</ghg:NumberOfBlowdowns>

<ghg:CriticalPhaseGasDensity>.004</ghg:CriticalPhaseGasDensity>

</ghg:UniqueInjectionPumpBlowdownsRowDetails>

<ghg:UniqueInjectionPumpBlowdownsRowDetails>

<ghg:UniqueIdentifier>2</ghg:UniqueIdentifier>

<ghg:PumpCapacity volUOM="barrels per day">1264.321</ghg:PumpCapacity>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">16362.2</ghg:CarbonDioxideEmissions>

<ghg:CriticalPhaseGasVolume>76</ghg:CriticalPhaseGasVolume>

<ghg:NumberOfBlowdowns>6</ghg:NumberOfBlowdowns>

<ghg:CriticalPhaseGasDensity>.004</ghg:CriticalPhaseGasDensity>

</ghg:UniqueInjectionPumpBlowdownsRowDetails>

</ghg:UniqueInjectionPumpBlowdownsDetails>

</ghg:EnhancedOilRecoveryInjectionPumpBlowdownDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Enhanced Oil Recovery (EOR) Hydrocarbon Liquids Dissolved CO2

This topic provides a step-by-step description of how to report EOR hydrocarbon liquids dissolved CO2 information for a facility.  This section is applicable to and required for the onshore petroleum and natural gas production industry segment only.

EOR Hydrocarbon Liquids Dissolved CO2 Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For EOR hydrocarbon liquids dissolved CO2, report the following:

- The CO2 emissions totals and the total CO2e emissions for all sub-basin categories combined in metric tons of CO2e (total CO2 emissions and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the data element “CarbonDioxideEmissions” for each sub-basin.

- The value to report for “TotalCarbonDioxideEquivalent” equals the value reported for “TotalCarbonDioxideEmissions”.

- Whether the facility had any enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

EOR Hydrocarbon Liquids Dissolved CO2 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails | Parent Element: A collection of data elements to report for enhanced oil recovery hydrocarbon liquids dissolved CO2. [98.236(c)(18)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all sub-basins reported combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 during the reporting year. |
| TotalCarbonDioxideEquivalent | Annual CO2 emissions from all sub-basins reported combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 during the reporting year. |
| DoesFacilityHaveEorLiquidsDissolved | Indicate (Yes/No) if the facility had enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| HydrocarbonLiquidsDissolvedDetails | Parent Element (Conditionally Required): A collection of data elements to report for sub-basins if the facility had enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 in the reporting year. |

Example for EOR Hydrocarbon Liquids Dissolved CO2 Details

<ghg:EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">338.7</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">338.7</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveEorLiquidsDissolved>Yes</ghg:DoesFacilityHaveEorLiquidsDissolved>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:HydrocarbonLiquidsDissolvedDetails>

<See example for EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details>

</ghg:HydrocarbonLiquidsDissolvedDetails>

</ghg:EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details Schema Diagram

Note: Data elements boxed in red are required.

For EOR hydrocarbon liquids dissolved CO2for each sub-basin category (refer to Equation W-38 of 98.233), report the following for each sub-basin category [98.236(c)(18)]:

- The unique identity of the sub-basin category (see Sub-Basin Identification for the naming convention).

- Annual CO2 emissions at the sub-basin level, expressed in metric tons CO2e. [98.236(c)(18)(iii)]

- Volume of crude oil produced (barrels per year). [98.236(c)(18)(i)]

- Amount of CO2 retained in hydrocarbon liquids (mt CO2 per barrel). [98.236(c)(18)(ii)]

EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| HydrocarbonLiquidsDissolvedDetails | Parent Element (Conditionally Required): A collection of data elements to report for sub-basins if the facility had enhanced oil recovery hydrocarbon liquids dissolved CO2 subject to reporting under 98.232 in the reporting year. |
| HydrocarbonLiquidsDissolvedSubbasinRowDetails | Parent Element: A collection of data elements to report for each sub-basin. |
| SubBasinIdentifier | Sub-basin ID. See Sub-Basin Identification for the required naming convention. |
| CarbonDioxideEmissions | Annual CO2 emissions in metric tons for the specified sub-basin. [98.236(c)(18)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| CrudeOilProduced | Volume of crude oil produced (barrels per year). [98.236(c)(18)(i)] |
| CarbonDioxideRetained | Amount of CO2 retained in hydrocarbon liquids (mt CO2 per barrel). [98.236(c)(18)(ii)] |

Example for EOR Hydrocarbon Liquids Dissolved Sub-Basin CO2 Details

<ghg:HydrocarbonLiquidsDissolvedDetails>

<ghg:HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - ALFALFA, OK (3) - Oil</ghg:SubBasinIdentifier>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">7.5</ghg:CarbonDioxideEmissions>

<ghg:CrudeOilProduced>54000</ghg:CrudeOilProduced>

<ghg:CarbonDioxideRetained>.5</ghg:CarbonDioxideRetained>

</ghg:HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<ghg:HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<ghg:SubBasinIdentifier>360 - BACA, CO (9) - Shale gas</ghg:SubBasinIdentifier>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">331.2</ghg:CarbonDioxideEmissions>

<ghg:CrudeOilProduced>55000</ghg:CrudeOilProduced>

<ghg:CarbonDioxideRetained>.6</ghg:CarbonDioxideRetained>

</ghg:HydrocarbonLiquidsDissolvedSubBasinRowDetails>

</ghg:HydrocarbonLiquidsDissolvedDetails>

</ghg:EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

## Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions

This topic provides a step-by-step description of how to report onshore petroleum and natural gas production and natural gas distribution combustion emissions information for a facility.  This section is applicable to and required for the following industry segments only:

- Onshore petroleum and natural gas production

- Natural gas distribution

Combustion Emissions Details Schema Diagram

Conditionally Required

Conditionally Required

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For combustion emissions, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions from all combustion emissions combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]  Note:  Report “0” if the facility did not have any combustion emissions subject to reporting under 98.232(c)(22) during the reporting year.

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the following data elements:

- “CarbonDioxideEmissions” for each external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr.

- “CarbonDioxideEmissions” for each internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “MethaneCarbonDioxideEquivalent” for each external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr.

- “MethaneCarbonDioxideEquivalent” for each internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the following data elements:

- “NitrousCarbonDioxideEquivalent” for each external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr.

- “NitrousCarbonDioxideEquivalent” for each internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether the facility had any combustion emissions subject to reporting under 98.232(c)(22) in the reporting year.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

Combustion Emissions Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| CombustionEmissionsDetails | Parent Element: A collection of data elements to report for onshore petroleum and natural gas production and natural gas distribution combustion emissions. [98.236(c)(22)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all units reported combined in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any combustion emissions subject to reporting under 98.232(c)(22) during the reporting year. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all units reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any combustion emissions subject to reporting under 98.232(c)(22) during the reporting year. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all units reported combined in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any combustion emissions subject to reporting under 98.232(c)(22) during the reporting year. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all units reported combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. Note: Report “0” if the facility did not have any combustion emissions subject to reporting under 98.232(c)(22) during the reporting year. |
| DoesFacilityHaveCombustionEmissions | Indicate (Yes/No) if the facility had combustion emissions subject to reporting under 98.232(c)(22) in the reporting year. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| ExternalFuelHeatCapacityLessThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity equal to or less than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity greater than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelHeatCapacityLessThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity equal to or less than 1 mmBtu/hr or 130 horsepower subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity greater than 1 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity greater than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity greater than 1 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |

Example for Combustion Emissions Details

<ghg:CombustionEmissionsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">35.7</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">102.3</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">16.7</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">154.7</ghg:TotalCarbonDioxideEquivalent>

<ghg:DoesFacilityHaveCombustionEmissions>Yes</ghg:DoesFacilityHaveCombustionEmissions>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<See example for External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details>

</ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<See example for External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details>

</ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<See example for Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details>

</ghg:InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<See example for Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details>

</ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<See example for External Fuel Type for Combustion Unit (greater than 5 mmBtu/hr) Details>

</ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<See example for Internal Fuel Type for Combustion Unit (greater than 1 mmBtu/hr) Details>

</ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

</ghg:CombustionEmissionsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)(22), report the following for each type of external fuel combustion unit with a heat capacity equal to or less than 5 mmBtu/hr [98.236(c)(22)]:

- The type of unit. [98.236(c)(19)(i)]

- The cumulative number of units of the type. [98.236(c)(19)(i)]

External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ExternalFuelHeatCapacityLessThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity equal to or less than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of external fuel combustion unit with a heat capacity equal to or less than 5 mmBtu/hr. |
| UnitType | Type of unit. See list of allowable values. [98.236(c)(19)(i)] Well drilling and completion equipment Workover equipment Natural gas dehydrators Steam boilers Process heaters |
| NumberOfUnits | Number of units of the type specified. [98.236(c)(19)(i)] |

Example for External Fuel Combustion Unit (less than or equal to 5 mmBtu/hr) Details

<ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:UnitType>Steam boilers</ghg:UnitType>

<ghg:NumberOfUnits>17</ghg:NumberOfUnits>

</ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:UnitType>Natural gas dehydrators</ghg:UnitType>

<ghg:NumberOfUnits>23</ghg:NumberOfUnits>

</ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

</ghg:ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)(22), report the following for each type of external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr [98.236(c)(19)]:

- The type of unit. [98.236(c)(19)(ii)]

- The cumulative number of units of the type. [98.236(c)(19)(ii)]

- The annual CO2, CH4 and N2O emissions from the type of unit, expressed in metric tons CO2e for each gas. [98.236(c)(19)(iii)]

External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity greater than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr. |
| UnitType | Type of unit. See list of allowable values. [98.236(c)(19)(ii)] Well drilling and completion equipment Workover equipment Natural gas dehydrators Steam boilers Process heaters |
| NumberOfUnits | Number of units of the type specified. [98.236(c)(19)(ii)] |
| CarbonDioxideEmissions | Annual CO2 emissions from the specified type of unit in metric tons. [98.236(c)(19)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| MethaneCarbonDioxideEquivalent | Annual CH4 emissions from the specified type of unit in metric tons CO2e. [98.236(c)(19)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NitrousCarbonDioxideEquivalent | Annual N2O emissions from the specified type of unit in metric tons CO2e. [98.236(c)(19)(iii)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for External Fuel Combustion Unit (greater than 5 mmBtu/hr) Details

<ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:UnitType>Process heaters</ghg:UnitType>

<ghg:NumberOfUnits>41</ghg:NumberOfUnits>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">6.1</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">15.2</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">3.9</ghg:NitrousCarbonDioxideEquivalent>

</ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:UnitType>Steam boilers </ghg:UnitType>

<ghg:NumberOfUnits>54</ghg:NumberOfUnits>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">6.2</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">30.4</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">3.9</ghg:NitrousCarbonDioxideEquivalent>

</ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ghg:ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)(22), report the following for each type of internal fuel combustion unit with a heat capacity equal to or less than 1 mmBtu/hr or 130 horsepower [98.236(c)(19)]:

- The type of unit. [98.236(c)(19)(v)]

- The cumulative number of units of the type. [98.236(c)(19)(v)]

Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| InternalFuelHeatCapacityLessThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity equal to or less than 1 mmBtu/hr or 130 horsepower subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of internal fuel combustion unit with a heat capacity equal to or less than 1 mmBtu/hr or 130 horsepower. |
| UnitType | Type of unit. See list of allowable values. [98.236(c)(19)(v)] Well drilling and completion equipment Workover equipment Electrical generators |
| NumberOfUnits | Number of units of the type specified. [98.236(c)(19)(v)] |

Example for Internal Fuel Combustion Unit (less than or equal to 1 mmBtu/hr or 130 horsepower) Details

<ghg:InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ghg:InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:UnitType>Workover equipment</ghg:UnitType>

<ghg:NumberOfUnits>22</ghg:NumberOfUnits>

</ghg:InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ghg:UnitType>Electrical generators</ghg:UnitType>

<ghg:NumberOfUnits>44</ghg:NumberOfUnits>

</ghg:InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

</ghg:InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)( 22), report the following for each type of internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr [98.236(c)(19)]:

- The type of unit. [98.236(c)(19)(vi)]

- The annual CO2, CH4 and N2O emissions from the type of unit, expressed in metric tons CO2e for each gas. [98.236(c)(19)(vi)]

Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity greater than 1 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr. |
| UnitType | Type of unit. See list of allowable values. [98.236(c)(19)(vi)] Well drilling and completion equipment Workover equipment Natural gas compressors Electrical generators |
| CarbonDioxideEmissions | Annual CO2 emissions from the specified type of unit in metric tons. [98.236(c)(19)(vi)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| MethaneCarbonDioxideEquivalent | Annual CH4 emissions from the specified type of unit in metric tons CO2e. [98.236(c)(19)(vi)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NitrousCarbonDioxideEquivalent | Annual N2O emissions from the specified type of unit in metric tons CO2e. [98.236(c)(19)(vi)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) Details

<ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:UnitType>Electrical generators</ghg:UnitType>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">11.5</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">28.0</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">4.1</ghg:NitrousCarbonDioxideEquivalent>

</ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:UnitType>Workover equipment</ghg:UnitType>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">11.9</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">28.7</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">4.8</ghg:NitrousCarbonDioxideEquivalent>

</ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ghg:InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

</ghg:CombustionEmissionsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Please see Required Sources to Report by Industry Segment for additional source types applicable to and required for each industry segment.

External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)(22), report the following for each type of external fuel combustion unit with a rated heat capacity greater than 5 mmBtu/hr, by fuel type [98.236(c)(19)]:

- The type of fuel combusted. [98.236(c)(19)(iv)]

- The cumulative volume of fuel combusted. [98.236(c)(19)(iv)]

- The fuel volume units. [98.236(c)(19)(iv)]

External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any external fuel combustion units with a heat capacity greater than 5 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of external fuel combustion unit with a heat capacity greater than 5 mmBtu/hr. |
| FuelType | Type of fuel combusted. See list of allowable values. [98.236(c)(19)(iv)] Pipeline gas Field gas Distillate Fuel Oil No. 2 Motor gasoline Propane Agricultural Byproducts Anthracite Asphalt and Road Oil Aviation Gasoline Biodiesel (100%) Biomass fuels—gaseous Biomass Fuels—Liquid Biomass fuels—solid Bituminous Blast Furnace Gas Butane Butylene Coal Coke Coke Oven Gas Crude Oil Distillate Fuel Oil No. 1 Distillate Fuel Oil No. 4 Ethane Ethanol Ethylene Fuel Gas Heavy Gas Oils Isobutane Isobutylene Kerosene Kerosene-Type Jet Fuel Landfill Gas Lignite Liquefied petroleum gases (LPG) Lubricants Mixed (Commercial sector) Mixed (Electric Power sector) Mixed (Industrial coking) Mixed (Industrial sector) Motor Gasoline Municipal Solid Waste Naphtha (<401 deg F) Natural Gasoline Other Biomass Gases Other Oil (>401 deg F) Peat Pentanes Plus Petrochemical Feedstocks Petroleum Coke Plastics Propane Gas Propylene Rendered Animal Fat Residual Fuel Oil No. 5 Residual Fuel Oil No. 6 Solid Byproducts Special Naphtha Subbituminous Tires Unfinished Oils Used Oil Vegetable Oil Wood and Wood Residuals (dry basis) |
| CumulativeFuelCombustedVolume | Cumulative volume of fuel combusted. [98.236(c)(19)(iv)] |
| FuelVolumeUnits | Fuel volume units. See list of allowable values. [98.236(c)(19)(iv)] mton kg lb cu ft cu meter gal bbl |

Example for External Fuel Combustion Unit (greater than 5 mmBtu/hr) by Fuel Type Details

<ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:FuelType>Vegetable Oil</ghg:FuelType>

<ghg:CumulativeFuelCombustedVolume>10000</ghg:CumulativeFuelCombustedVolume>

<ghg:FuelVolumeUnits>gal</ghg:FuelVolumeUnits>

</ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ghg:ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details Schema Diagram

Note: Data elements boxed in red are required.

For combustion emissions subject to reporting under 98.232(c)(22), report the following for each type of internal fuel combustion unit with a rated heat capacity greater than 1 mmBtu/hr, by fuel type [98.236(c)(19)]:

- The type of fuel combusted. [98.236(c)(19)(vii)]

- The cumulative volume of fuel combusted. [98.236(c)(19)(vii)]

- The fuel volume units. [98.236(c)(19)(vii)]

Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails | Parent Element (Conditionally Required): A collection of data elements to report if the facility had any internal fuel combustion units with a heat capacity greater than 1 mmBtu/hr subject to reporting under 98.232(c)(22) in the reporting year. |
| InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails | Parent Element: A collection of data elements to report for each type of internal fuel combustion unit with a heat capacity greater than 1 mmBtu/hr. |
| FuelType | Type of fuel combusted. See list of allowable values. [98.236(c)(19)(vii)] Pipeline gas Field gas Distillate Fuel Oil No. 2 Motor gasoline Propane Agricultural Byproducts Anthracite Asphalt and Road Oil Aviation Gasoline Biodiesel (100%) Biomass fuels—gaseous Biomass Fuels—Liquid Biomass fuels—solid Bituminous Blast Furnace Gas Butane Butylene Coal Coke Coke Oven Gas Crude Oil Distillate Fuel Oil No. 1 Distillate Fuel Oil No. 4 Ethane Ethanol Ethylene Fuel Gas Heavy Gas Oils Isobutane Isobutylene Kerosene Kerosene-Type Jet Fuel Landfill Gas Lignite Liquefied petroleum gases (LPG) Lubricants Mixed (Commercial sector) Mixed (Electric Power sector) Mixed (Industrial coking) Mixed (Industrial sector) Motor Gasoline Municipal Solid Waste Naphtha (<401 deg F) Natural Gasoline Other Biomass Gases Other Oil (>401 deg F) Peat Pentanes Plus Petrochemical Feedstocks Petroleum Coke Plastics Propane Gas Propylene Rendered Animal Fat Residual Fuel Oil No. 5 Residual Fuel Oil No. 6 Solid Byproducts Special Naphtha Subbituminous Tires Unfinished Oils Used Oil Vegetable Oil Wood and Wood Residuals (dry basis) |
| CumulativeFuelCombustedVolume | Cumulative volume of fuel combusted. [98.236(c)(19)(vii)] |
| FuelVolumeUnits | Fuel volume units. See list of allowable values. [98.236(c)(19)(vii)] mton kg lb cu ft cu meter gal bbl |

Example for Internal Fuel Combustion Unit (greater than 1 mmBtu/hr) by Fuel Type Details

<ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ghg:FuelType>Vegetable Oil</ghg:FuelType>

<ghg:CumulativeFuelCombustedVolume>10000</ghg:CumulativeFuelCombustedVolume>

<ghg:FuelVolumeUnits>gal</ghg:FuelVolumeUnits>

</ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ghg:InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

## Offshore Sources [98.236(b)]

This topic provides a step-by-step description of how to report offshore sources information for a facility.  This section is applicable to and required for the offshore petroleum and natural gas production industry segment only.

Offshore Emissions Details Schema Diagram

Note: Data elements boxed in red are required. Please see page 4 of this document 
for more information on conditionally required elements.

For offshore emissions, report the following:

- The CO2, CH4 and N2O emissions totals and the total CO2e emissions for all offshore emissions sources combined in metric tons of CO2e (total CO2 emissions, total CH4 emissions in CO2e, total N2O emissions in CO2e and total CO2e emissions).  [98.236(c)]

- The value to report for “TotalCarbonDioxideEmissions” equals the sum of the values reported for the data element “CarbonDioxideEmissions” for each emission source.

- The value to report for “TotalMethaneCarbonDioxideEquivalent” equals the sum of the values reported for the data element “MethaneCarbonDioxideEquivalent” for each emission source.

- The value to report for “TotalNitrousCarbonDioxideEquivalent” equals the sum of the values reported for the data element “NitrousCarbonDioxideEquivalent” for each emission source.

- The value to report for “TotalCarbonDioxideEquivalent” equals the sum of the values reported for “TotalCarbonDioxideEmissions”, “TotalMethaneCarbonDioxideEquivalent” and “TotalNitrousCarbonDioxideEquivalent”.

- Whether BAMM were used for any parameters to calculate GHG emissions.  [98.3(c)(7)]

- If BAMM were used, a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used.  [98.3(c)(7)]

- Whether missing data procedures were used for any parameters to calculate GHG emissions.  [98.235]

Offshore Emissions Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| OffshoreEmissionsDetails | Parent Element: A collection of data elements to report for Total Emissions from Offshore Source Emissions. [98.236(b)] |
| TotalCarbonDioxideEmissions | Annual CO2 emissions from all sources combined in metric tons. [98.236(a)(2)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalMethaneCarbonDioxideEquivalent | Annual CH4 emissions from all sources combined in metric tons CO2e. [98.236(a)(2)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalNitrousCarbonDioxideEquivalent | Annual N2O emissions from all sources combined in metric tons CO2e. [98.236(a)(2)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions from all sources combined in metric tons CO2e. Set the units of measure to “Metric Tons” in the attribute massUOM. |
| BAMMIndicator | Indicate (Yes/No) if BAMM were used for any parameters to calculate GHG emissions. [98.3(c)(7)] |
| BAMMDescription | Conditionally Required: If BAMM were used, provide a brief description of the BAMM used, parameters measured by BAMM and time period BAMM was used. [98.3(c)(7)] |
| SubstituteDataIndicator | Indicate (Yes/No) if missing data procedures were used for any parameters to calculate GHG emissions. [98.235] |
| OffshoreEmissionsSourcesDetails | Parent Element: A collection of data elements to report for offshore petroleum and natural gas production facilities. [98.236(b)] |

Example for Subpart W Source Reporting Details

<ghg:OffshoreEmissionsDetails>

<ghg:TotalCarbonDioxideEmissions massUOM="Metric Tons">5454.6</ghg:TotalCarbonDioxideEmissions>

<ghg:TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">3232.4</ghg:TotalMethaneCarbonDioxideEquivalent>

<ghg:TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons >1010.9</ghg:TotalNitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">9697.9</ghg:TotalCarbonDioxideEquivalent>

<ghg:BAMMIndicator>Yes</ghg:BAMMIndicator>

<ghg:BAMMDescription>BAMM Description</ghg:BAMMDescription>

<ghg:SubstituteDataIndicator>Yes</ghg:SubstituteDataIndicator>

<ghg:OffshoreEmissionsSourcesDetails>

<See example for Offshore Emissions Sources Details>

</ghg:OffshoreEmissionsSourcesDetails>

</ghg:OffshoreEmissionsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data

Offshore Emissions Sources Details Schema Diagram

Note: Data elements boxed in red are required.

For offshore petroleum and natural gas production, report under Subpart W the emissions of CH4, CO2 and N2O as applicable to the source type (in metric tons CO2e per year at standard conditions) individually for all of the emissions source types listed in the most recent BOEMRE study except for combustion emission source types [98.236(b)]:

Offshore production facilities under BOEMRE jurisdiction shall report the same annual emissions as calculated and reported by BOEMRE in data collection and emissions estimation study published by BOEMRE referenced in 30 CFR 250.302 through 304 (GOADS).  For any calendar year that does not overlap with the most recent BOEMRE emissions study publication year, report the most recent BOEMRE reported emissions data published by BOEMRE referenced in 30 CFR 250.302 through 304 (GOADS).  Adjust emissions based on the operating time for the facility relative to the operating time in the most recent BOEMRE published study.  [98.233(s)(1)(i), 98.233(s)(2)(i)]

Offshore production facilities that are not under BOEMRE jurisdiction shall use monitoring methods and calculation methodologies published by BOEMRE referenced in 30 CFR 250.302 through 304 to calculate and report emissions (GOADS).  For any calendar year that does not overlap with the most recent BOEMRE emissions study publication, report the most recent reported emissions data with emissions adjusted based on the operating time for the facility relative to operating time in the previous reporting period.  [98.233(s)(1)(i), 98.233(s)(2)(i)]

Note:  Combustion emissions other than flaring must be reported under Subpart C.

Offshore Emissions Sources Details Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| OffshoreEmissionsSourcesDetails | Parent Element: A collection of data elements to report for offshore petroleum and natural gas production facilities. [98.236(b)] |
| OffshoreEmissionsSourcesRowDetails | Parent Element: A collection of data elements to report for each applicable offshore emissions source. Report data for each source separately. |
| EmissionSource | Emission source. See the list of allowable values. [98.236(b)] Amine Unit. Combustion Flares - Light Smoke - No Pilot Fuel-flaring. Combustion Flares - Light Smoke - Pilot Fuel - pilot. Combustion Flares - Light Smoke - Pilot Fuel-flaring. Combustion Flares - Medium Smoke - No Pilot Fuel - flaring. Combustion Flares - Medium Smoke - Pilot Fuel - flaring. Combustion Flares - Medium Smoke - Pilot Fuel - pilot. Combustion Flares - No Smoke - No Pilot Fuel - flaring. Combustion Flares - No Smoke - Pilot Fuel - flaring. Combustion Flares - No Smoke - Pilot Fuel - pilot. Fugitives - Compressor centrifugal dry - gas. Fugitives - Compressor centrifugal dry - NG liq. Fugitives - Compressor centrifugal dry - heavy oil. Fugitives - Compressor centrifugal dry - light oil. Fugitives - Compressor centrifugal dry - oil/water. Fugitives - Compressor centrifugal dry - oil/water/gas. Fugitives - Compressor centrifugal wet - gas. Fugitives - Compressor centrifugal wet - NG liq. Fugitives - Compressor centrifugal wet - heavy oil. Fugitives - Compressor centrifugal wet - light oil. Fugitives - Compressor centrifugal wet - oil/water. Fugitives - Compressor centrifugal wet - oil/water/gas. Fugitives - Compressor reciprocating - gas. Fugitives - Compressor reciprocating - NG liq. Fugitives - Compressor reciprocating - heavy oil. Fugitives - Compressor reciprocating - light oil. Fugitives - Compressor reciprocating - oil/water. Fugitives - Compressor reciprocating - oil/water/gas. Fugitives - Connectors - gas. Fugitives - Connectors - NG liq. Fugitives - Connectors - heavy oil. Fugitives - Connectors - light oil. Fugitives - Connectors - oil/water. Fugitives - Connectors - oil/water/gas. Fugitives - Flanges - gas. Fugitives - Flanges - NG liq. Fugitives - Flanges - heavy oil. Fugitives - Flanges - light oil. Fugitives - Flanges - oil/water. Fugitives - Flanges - oil/water/gas. Fugitives - Open-Ended Lines - gas. Fugitives - Open-Ended Lines - NG liq. Fugitives - Open-Ended Lines - heavy oil. Fugitives - Open-Ended Lines - light oil. Fugitives - Open-Ended Lines - oil/water. Fugitives - Open-Ended Lines - oil/water/gas. Fugitives - Other Equipment - gas. Fugitives - Other Equipment - NG liq. Fugitives - Other Equipment - heavy oil. Fugitives - Other Equipment - light oil. Fugitives - Other Equipment - oil/water. Fugitives - Other Equipment - oil/water/gas. Fugitives - Pumps - gas. Fugitives - Pumps - NG liq. Fugitives - Pumps - heavy oil. Fugitives - Pumps - light oil. Fugitives - Pumps - oil/water. Fugitives - Pumps - oil/water/gas. Fugitives - Valves - gas. Fugitives - Valves - NG liq. Fugitives - Valves - heavy oil. Fugitives - Valves - light oil. Fugitives - Valves - oil/water. Fugitives - Valves - oil/water/gas. Glycol Dehydrator Unit. Losses from Flashing. Mud Degassing - oil-based muds. Mud Degassing - water-based muds. Mud Degassing - synthetic-based muds. Pneumatic Pump. Pressure/Level Controllers. Storage Tank Operations - crude oil. Storage Tank Operations - condensate. Cold Vent. |
| CarbonDioxideEmissions | Annual CO2 emissions from the specified source at standard conditions in metric tons. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| MethaneCarbonDioxideEquivalent | Annual CH4 emissions from the specified source at standard conditions in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| NitrousCarbonDioxideEquivalent | Annual N2O emissions from the specified source at standard conditions in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |
| TotalCarbonDioxideEquivalent | Total combined CO2, CH4 and N2O emissions for the specified source at standard conditions in metric tons CO2e. [98.236(c)] Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Offshore Emissions Sources Details

<ghg:OffshoreEmissionsSourcesDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Amine gas sweetening unit</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Minor source, caisson</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Flare: light smoke, no continuous pilot, flare</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Flare: light smoke, with continuous pilot, flare</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Flare: light smoke, with continuous pilot, pilot</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:OffshoreEmissionsSourcesRowDetails>

<ghg:EmissionSource>Flare: medium smoke, with continuous pilot, flare</ghg:EmissionSource>

<ghg:CarbonDioxideEmissions massUOM="Metric Tons">98.7</ghg:CarbonDioxideEmissions>

<ghg:MethaneCarbonDioxideEquivalent massUOM="Metric Tons">87.6</ghg:MethaneCarbonDioxideEquivalent>

<ghg:NitrousCarbonDioxideEquivalent massUOM="Metric Tons">76.5</ghg:NitrousCarbonDioxideEquivalent>

<ghg:TotalCarbonDioxideEquivalent massUOM="Metric Tons">262.8</ghg:TotalCarbonDioxideEquivalent>

</ghg:OffshoreEmissionsSourcesRowDetails>

</ghg:OffshoreEmissionsSourcesDetails>

</ghg:OffshoreEmissionsDetails>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

## Facility-Level Roll-up Emissions

Each facility must report the following facility-level emission totals:

- Total CO2 equivalent (CO2e) emissions (excluding biogenic CO2) aggregated across all direct emitter source categories (Subparts C-HH) associated with the facility.

- Total biogenic CO2 emissions aggregated across all direct emitter source categories (Subparts C-HH) associated with the facility.

Each supplier must report the following supplier totals:

- Total CO2e associated with products supplied aggregated across Subparts NN, OO and PP (as applicable).  Note:  Do not include Subpart LL and MM totals in this data element as these values are not being collected in e-GGRT.

Facility-Level Roll-up Emissions Schema Diagram

Note: Data elements boxed in red are required.

For Subpart W, report total emissions for CO2e (excluding biogenic CO2) by adding the value reported for the data element “TotalReportedCarbonDioxideEquivalent” to the total CO2e emissions (excluding biogenic CO2) aggregated across all source category Subparts associated with the facility.

Note:  You must follow the rounding rules found in Table 1.

Facility-Level Roll-up Emissions Data Element Definitions

| Data Element Name | Description |
| --- | --- |
| TotalNonBiogenicCO2eFacilitySubpartsCtoJJ | Add the total CO2e value for Subpart W in metric tons to the total CO2e emissions (excluding biogenic CO2) aggregated across all source category Subparts associated with the facility according to the guidelines above. Set the units of measure to “Metric Tons” in the attribute massUOM. |

Example for Facility-Level Roll-up Emissions

<ghg:TotalNonBiogenicCO2eFacilitySubpartsCtoJJ massUOM="Metric Tons">19223587.1</ghg:TotalNonBiogenicCO2eFacilitySubpartsCtoJJ>

Note:  The code excerpt above is presented here to demonstrate the concept of reporting greenhouse gas emissions data.

.

# Appendix A - Sample XML Document for Subpart W

Sample XML Document for Subpart W - Onshore Petroleum and Natural Gas Production industry segment

(Note: Data values do not reflect an actual facility's emissions.  Additional sample XML files for Subpart W are posted on the e-GGRT help site. http://www.ccdsupport.com/confluence/display/help/Additional+XML+Examples)

<?xml version="1.0"?><GHG xmlns="http://www.ccdsupport.com/schema/ghg">

<FacilitySiteInformation>

<CertificationStatement>The designated representative or alternate designated representative must sign (i.e., agree to) this certification statement. If you are an agent and you click on "SUBMIT", you are not agreeing to the certification statement, but are submitting the certification statement on behalf of the designated representative or alternate designated representative who is agreeing to the certification statement. An agent is only authorized to make the electronic submission on behalf of the designated representative, not to sign (i.e., agree to) the certification statement.</CertificationStatement>

<ReportingYear>2014</ReportingYear>

<FacilitySiteDetails>

<FacilitySite>

<FacilitySiteIdentifier>515789</FacilitySiteIdentifier>

<FacilitySiteName>Angkor</FacilitySiteName>

</FacilitySite>

<LocationAddress>

<LocationAddressText>4518 40th St.t.</LocationAddressText>

<LocalityName>Sunnyside</LocalityName>

<StateIdentity>

<StateCode>NY</StateCode>

</StateIdentity>

<AddressPostalCode>10004</AddressPostalCode>

</LocationAddress>

<CogenerationUnitEmissionsIndicator>N</CogenerationUnitEmissionsIndicator>

<PrimaryNAICSCode>221122</PrimaryNAICSCode>

<ParentCompanyDetails>

<ParentCompany>

<ParentCompanyLegalName>MLJ</ParentCompanyLegalName>

<StreetAddress>223 Ma St</StreetAddress>

<City>Arlington</City>

<State>VA</State>

<Zip>22207</Zip>

<PercentOwnershipInterest>100</PercentOwnershipInterest>

</ParentCompany>

</ParentCompanyDetails>

<TotalNonBiogenicCO2eFacilitySubpartsCtoJJ massUOM="Metric Tons">5150928

</TotalNonBiogenicCO2eFacilitySubpartsCtoJJ>

<TotalBiogenicCO2FacilitySubpartsCtoJJ massUOM="Metric Tons">0</TotalBiogenicCO2FacilitySubpartsCtoJJ>

<TotalCO2eSupplierSubpartsKKtoPP massUOM="Metric Tons">0</TotalCO2eSupplierSubpartsKKtoPP>

<PlantCodeIndicator>N</PlantCodeIndicator>

<SubPartInformation>

<SubPartW>

<GHGasInfoDetails>

<GHGasName>Methane</GHGasName>

<GHGasQuantity massUOM="Metric Tons">

<CalculatedValue>149185.92</CalculatedValue>

</GHGasQuantity>

</GHGasInfoDetails>

<GHGasInfoDetails>

<GHGasName>Nitrous Oxide</GHGasName>

<GHGasQuantity massUOM="Metric Tons">

<CalculatedValue>4.475</CalculatedValue>

</GHGasQuantity>

</GHGasInfoDetails>

<GHGasInfoDetails>

<GHGasName>Carbon Dioxide</GHGasName>

<GHGasQuantity massUOM="Metric Tons">

<CalculatedValue>1419946.4</CalculatedValue>

</GHGasQuantity>

</GHGasInfoDetails>

<SubPartWReportingFormsDetails>

<SubpartWSummaryDetails>

<SelectedIndustrySegmentNumber>2</SelectedIndustrySegmentNumber>

<SelectedIndustrySegmentName>Onshore petroleum and natural gas production [98.230(a)(2)]

</SelectedIndustrySegmentName>

<AnnualGaseousThroughput volUOM="MMscf">2452.534</AnnualGaseousThroughput>

<AnnualLiquidThroughput volUOM="thousand barrels">4345.1</AnnualLiquidThroughput>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">1419946.4</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric Tons">3729648.0

</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric Tons">1333.5

</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">5150927.9</TotalReportedCarbonDioxideEquivalent>

<SubpartWSourceReportingFormDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Sub-Basin Selection</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Natural Gas Pneumatic Devices [98.236(c)(1)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">1433.7</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">16652.1</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">18085.8</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Natural Gas Driven Pneumatic Pumps [98.236(c)(2)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">13.3</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">154.5</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">167.8</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Acid Gas Removal Units [98.236(c)(3)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">2206.4</TotalReportedCarbonDioxideEmissions>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">2206.4</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails><SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Dehydrators [98.236(c)(4)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">7.6</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">9109.6</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">38.3</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">9155.5</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Well Venting for Liquids Unloading [98.236(c)(5)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">9.6</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">8965.8</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">8975.4</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Gas Well Completions and Workovers [98.236(c)(6)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">376576.5</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">2262420.3</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">200.1</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">2639196.9</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Blowdown Vent Stacks [98.236(c)(7)]</ReportingCategory>

<RequiredForIndustrySegment>No</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Gas from Produced Oil Sent to Atmospheric Tanks [98.236(c)(8)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">963180.1</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">1401058.2</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">300.7</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">2364539.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Transmission Tanks [98.236(c)(9)]</ReportingCategory>

<RequiredForIndustrySegment>No</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Well Testing Venting and Flaring [98.236(c)(10)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">200.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">200.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">100.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">500.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Associated Gas Venting and Flaring [98.236(c)(11)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Flare Stacks [98.236(c)(12)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">16411.1</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">6048.6</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">159.8</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">22619.5</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Centrifugal Compressors [98.236(c)(13)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">20000.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">21000.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">41000.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Reciprocating Compressors [98.236(c)(14)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">11.1</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">19.1</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">30.2</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Other Emissions from Equipment Leaks Estimated Using Emission Factors [98.236(c)(15)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">9048.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">4000.7</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">13048.7</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Local Distribution Companies [98.236(c)(16)]</ReportingCategory>

<RequiredForIndustrySegment>No</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Enhanced Oil Recovery Injection Pump Blowdown [98.236(c)(17)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">4.2</TotalReportedCarbonDioxideEmissions>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">4.2</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Enhanced Oil Recovery Hydrocarbon Liquids Dissolved CO2 [98.236(c)(18)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">9.6</TotalReportedCarbonDioxideEmissions>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">9.6</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Onshore Petroleum and Natural Gas Production and Natural Gas Distribution Combustion Emissions

[98.236(c)(19)]</ReportingCategory>

<RequiredForIndustrySegment>Yes</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">30835.2</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">19.1</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">534.6</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">31388.9</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

<SubpartWSourceReportingFormRowDetails>

<ReportingCategory>Offshore Sources [98.236(b)]</ReportingCategory>

<RequiredForIndustrySegment>No</RequiredForIndustrySegment>

<TotalReportedCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEmissions>

<TotalReportedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedMethaneCarbonDioxideEquivalent>

<TotalReportedNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</TotalReportedNitrousCarbonDioxideEquivalent>

<TotalReportedCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalReportedCarbonDioxideEquivalent>

</SubpartWSourceReportingFormRowDetails>

</SubpartWSourceReportingFormDetails>

</SubpartWSummaryDetails>

<OnshoreRequirementsDetails>

<SelectedBasin>535 - Green River Basin</SelectedBasin>

<OnshoreProductionRequirementsDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>ALBANY, WY (1)</SubBasinCounty>

<SubBasinFormationType>Oil</SubBasinFormationType>

<SubBasinIdentifier>535 - ALBANY, WY (1) - Oil</SubBasinIdentifier>

<ApiGravity densityUOM="degrees">95</ApiGravity>

<GasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">100</GasToOilRatio>

<AveragePressure pressureUOM="psia">105</AveragePressure>

</OnshoreProductionRequirementsSubBasinRowDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>CARBON, WY (7)</SubBasinCounty>

<SubBasinFormationType>High permeability gas</SubBasinFormationType>

<SubBasinIdentifier>535 - CARBON, WY (7) - High permeability gas</SubBasinIdentifier>

</OnshoreProductionRequirementsSubBasinRowDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>MOFFAT, CO (81)</SubBasinCounty>

<SubBasinFormationType>Shale gas</SubBasinFormationType>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

</OnshoreProductionRequirementsSubBasinRowDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>ROUTT, CO (107)</SubBasinCounty>

<SubBasinFormationType>Coal seam</SubBasinFormationType>

<SubBasinIdentifier>535 - ROUTT, CO (107) - Coal seam</SubBasinIdentifier>

</OnshoreProductionRequirementsSubBasinRowDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>SUBLETTE, WY (35)</SubBasinCounty>

<SubBasinFormationType>Other tight reservoir rock</SubBasinFormationType>

<SubBasinIdentifier>535 - SUBLETTE, WY (35) - Other tight reservoir rock</SubBasinIdentifier>

</OnshoreProductionRequirementsSubBasinRowDetails>

<OnshoreProductionRequirementsSubBasinRowDetails>

<SubBasinCounty>SWEETWATER, WY (37)</SubBasinCounty>

<SubBasinFormationType>Other tight reservoir rock</SubBasinFormationType>

<SubBasinIdentifier>535 - SWEETWATER, WY (37) - Other tight reservoir rock</SubBasinIdentifier>

</OnshoreProductionRequirementsSubBasinRowDetails>

</OnshoreProductionRequirementsDetails>

</OnshoreRequirementsDetails>

<PneumaticDeviceVentingDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">1433.7</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">16652.1</TotalMethaneCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">18085.8</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveHighBleedDevices>Yes</DoesFacilityHaveHighBleedDevices>

<DoesFacilityHaveIntermittentBleedDevices>Yes</DoesFacilityHaveIntermittentBleedDevices>

<DoesFacilityHaveLowBleedDevices>Yes</DoesFacilityHaveLowBleedDevices>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<PneumaticDeviceTypesDetails>

<PneumaticDeviceTypesRowDetails>

<PneumaticDeviceType>High-bleed Pneumatic Devices</PneumaticDeviceType>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">975</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">11323.5</TotalMethaneCarbonDioxideEquivalent>

<ActualCount>20</ActualCount>

<EstimatedCount>20</EstimatedCount>

</PneumaticDeviceTypesRowDetails>

<PneumaticDeviceTypesRowDetails>

<PneumaticDeviceType>Intermittent Bleed Pneumatic Devices</PneumaticDeviceType>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">430.1</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">4995.6</TotalMethaneCarbonDioxideEquivalent>

<ActualCount>20</ActualCount>

<EstimatedCount>18</EstimatedCount>

</PneumaticDeviceTypesRowDetails>

<PneumaticDeviceTypesRowDetails>

<PneumaticDeviceType>Low-Bleed Pneumatic Devices</PneumaticDeviceType>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">28.6</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">333</TotalMethaneCarbonDioxideEquivalent>

<ActualCount>20</ActualCount>

<EstimatedCount>20</EstimatedCount>

</PneumaticDeviceTypesRowDetails>

</PneumaticDeviceTypesDetails>

</PneumaticDeviceVentingDetails>

<NaturalGasDrivenPneumaticPumpsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">13.3</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">154.5</TotalMethaneCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">167.8</TotalCarbonDioxideEquivalent>

<DoesFacilityHavePneumaticPumps>Yes</DoesFacilityHavePneumaticPumps>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<PneumaticPumpTypesDetails>

<PneumaticPumpTypesRowDetails>

<PneumaticPumpType>Natural Gas Driven Pneumatic Pumps</PneumaticPumpType>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">13.3</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">154.5</TotalMethaneCarbonDioxideEquivalent>

<TotalPneumaticPumpCount>120</TotalPneumaticPumpCount>

</PneumaticPumpTypesRowDetails>

</PneumaticPumpTypesDetails>

</NaturalGasDrivenPneumaticPumpsDetails>

<AcidGasRemovalUnitsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">2206.4</TotalCarbonDioxideEmissions>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">2206.4</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveAcidGasRemovalUnits>Yes</DoesFacilityHaveAcidGasRemovalUnits>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<AcidGasRemovalNumberOfUnitsDetails>

<AcidGasRemovalNumberOfUnitsRowDetails>

<UniqueIdentifier>1</UniqueIdentifier>

<UnitIdentifier>Acid Gas Unit 1</UnitIdentifier>

<CalculationMethodology>Calculation Methodology 1 (98.233(d)(1))</CalculationMethodology>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">551.6</TotalCarbonDioxideEmissions>

<AverageVentCarbonDioxideFraction fractionUOM="volumetric fraction">0.2</AverageVentCarbonDioxideFraction>

<AverageNaturalGasCarbonDioxideFractionOut fractionUOM="volumetric

fraction">100</AverageNaturalGasCarbonDioxideFractionOut>

<TotalThroughput volUOM="million actual cubic feet per year">10</TotalThroughput>

</AcidGasRemovalNumberOfUnitsRowDetails>

<AcidGasRemovalNumberOfUnitsRowDetails>

<UniqueIdentifier>2</UniqueIdentifier>

<UnitIdentifier>Acid Gas Unit 2</UnitIdentifier>

<CalculationMethodology>Calculation Methodology 2 (98.233(d)(2))</CalculationMethodology>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">551.6</TotalCarbonDioxideEmissions>

<AverageVentCarbonDioxideFractionMethod2 fractionUOM="volumetric

fraction">0.2</AverageVentCarbonDioxideFractionMethod2>

<AverageNaturalGasCarbonDioxideFractionOut fractionUOM="volumetric

fraction">100</AverageNaturalGasCarbonDioxideFractionOut>

<TotalThroughput volUOM="million actual cubic feet per year">10</TotalThroughput>

</AcidGasRemovalNumberOfUnitsRowDetails>

<AcidGasRemovalNumberOfUnitsRowDetails>

<UniqueIdentifier>3</UniqueIdentifier>

<UnitIdentifier>Acid Gas Unit 3</UnitIdentifier>

<CalculationMethodology>Calculation Methodology 3 (98.233(d)(3))</CalculationMethodology>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">551.6</TotalCarbonDioxideEmissions>

<AverageNaturalGasCarbonDioxideFractionIn fractionUOM="volumetric

fraction">0.2</AverageNaturalGasCarbonDioxideFractionIn>

<AverageNaturalGasCarbonDioxideFractionOut fractionUOM="volumetric

fraction">100</AverageNaturalGasCarbonDioxideFractionOut>

<TotalThroughput volUOM="million actual cubic feet per year">10</TotalThroughput>

</AcidGasRemovalNumberOfUnitsRowDetails>

<AcidGasRemovalNumberOfUnitsRowDetails>

<UniqueIdentifier>4</UniqueIdentifier>

<UnitIdentifier>Acid Gas Unit 4</UnitIdentifier>

<CalculationMethodology>Calculation Methodology 4 (98.233(d)(4))</CalculationMethodology>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">551.6</TotalCarbonDioxideEmissions>

<AverageNaturalGasCarbonDioxideFractionOut fractionUOM="volumetric

fraction">100</AverageNaturalGasCarbonDioxideFractionOut>

<TotalThroughput volUOM="million actual cubic feet per year">10</TotalThroughput>

</AcidGasRemovalNumberOfUnitsRowDetails>

</AcidGasRemovalNumberOfUnitsDetails>

</AcidGasRemovalUnitsDetails>

<DehydratorsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">7.6</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">9109.6</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">38.3</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">9155.5</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveDehydrators>Yes</DoesFacilityHaveDehydrators>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<GlycolDehydratorsLessThanSpecifiedValueDetails>

<GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<VentGasControlsUsed>Vapor Recovery</VentGasControlsUsed>

</GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<VentGasControlsUsed>Dehydrator Vents to Flares</VentGasControlsUsed>

</GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<VentGasControlsUsed>Regenerator fire-box/fire tubes</VentGasControlsUsed>

</GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<VentGasControlsUsed>No Vent Controls</VentGasControlsUsed>

</GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<GlycolDehydratorsLessThanSpecifiedValueRowDetails>

<VentGasControlsUsed>Other / Multiple Vent Gas Controls</VentGasControlsUsed>

<VentGasControlsOtherDescription>This is just a description </VentGasControlsOtherDescription>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">0.1</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">28.7</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">157.9</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.8</FlaringNitrousCarbonDioxideEquivalent>

<GlycolDehydratorCount>10</GlycolDehydratorCount>

</GlycolDehydratorsLessThanSpecifiedValueRowDetails>

</GlycolDehydratorsLessThanSpecifiedValueDetails>

<AbsorbentDesiccantDehydratorsDetails>

<AbsorbentDesiccantDehydratorsRowDetails>

<DeviceType>Absorbent Desiccant dehydrators</DeviceType>

</AbsorbentDesiccantDehydratorsRowDetails>

</AbsorbentDesiccantDehydratorsDetails>

<GlycolDehydratorsMoreThanSpecifiedValueDetails>

<GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<UniqueIdentifier>1</UniqueIdentifier>

<VentGasControlsUsed>Vapor Recovery</VentGasControlsUsed>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">1.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">275.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1509.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">7.5</FlaringNitrousCarbonDioxideEquivalent>

<NaturalGasFlowRate flowUOM="MMscfd">4.3</NaturalGasFlowRate>

<AbsorbentCirculationPumpType>Natural gas pneumatic</AbsorbentCirculationPumpType>

<IsStripperGasUsed>Yes</IsStripperGasUsed>

<IsFlashTankSeparatorUsed>No</IsFlashTankSeparatorUsed>

<AbsorbentType>Triethylene glycol (TEG)</AbsorbentType>

<GlycolDehydratorOperatingHours timeUOM="Hours">4</GlycolDehydratorOperatingHours>

<WetNaturalGasTemperature tempUOM="Fahrenheit">45</WetNaturalGasTemperature>

<WetNaturalGasPressure pressureUOM="psig">0.3</WetNaturalGasPressure>

<WetNaturalGasCarbonDioxideConcentration fractionUOM="fraction (number between 0 and

1)">0.24</WetNaturalGasCarbonDioxideConcentration>

<WetNaturalGasMethaneConcentration fractionUOM="fraction (number between 0 and

1)">0.5</WetNaturalGasMethaneConcentration>

</GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<UniqueIdentifier>2</UniqueIdentifier>

<VentGasControlsUsed>Dehydrator Vents to Flares</VentGasControlsUsed>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">1.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">275.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1509.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">7.5</FlaringNitrousCarbonDioxideEquivalent>

<NaturalGasFlowRate flowUOM="MMscfd">4.3</NaturalGasFlowRate>

<AbsorbentCirculationPumpType>Air pneumatic</AbsorbentCirculationPumpType>

<IsStripperGasUsed>No</IsStripperGasUsed>

<IsFlashTankSeparatorUsed>Yes</IsFlashTankSeparatorUsed>

<AbsorbentType>Diethylene glycol (DEG) </AbsorbentType>

<GlycolDehydratorOperatingHours timeUOM="Hours">4</GlycolDehydratorOperatingHours>

<WetNaturalGasTemperature tempUOM="Fahrenheit">45</WetNaturalGasTemperature>

<WetNaturalGasPressure pressureUOM="psig">0.3</WetNaturalGasPressure>

<WetNaturalGasCarbonDioxideConcentration fractionUOM="fraction (number between 0 and

1)">0.25</WetNaturalGasCarbonDioxideConcentration>

<WetNaturalGasMethaneConcentration fractionUOM="fraction (number between 0 and

1)">0.5</WetNaturalGasMethaneConcentration>

</GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<UniqueIdentifier>3</UniqueIdentifier>

<VentGasControlsUsed>Regenerator fire-box/fire tubes</VentGasControlsUsed>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">1.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">275.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1509.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">7.5</FlaringNitrousCarbonDioxideEquivalent>

<NaturalGasFlowRate flowUOM="MMscfd">4.3</NaturalGasFlowRate>

<AbsorbentCirculationPumpType>Electric</AbsorbentCirculationPumpType>

<IsStripperGasUsed>Yes</IsStripperGasUsed>

<IsFlashTankSeparatorUsed>No</IsFlashTankSeparatorUsed>

<AbsorbentType>Triethylene glycol (TEG)</AbsorbentType>

<GlycolDehydratorOperatingHours timeUOM="Hours">4</GlycolDehydratorOperatingHours>

<WetNaturalGasTemperature tempUOM="Fahrenheit">45</WetNaturalGasTemperature>

<WetNaturalGasPressure pressureUOM="psig">0.3</WetNaturalGasPressure>

<WetNaturalGasCarbonDioxideConcentration fractionUOM="fraction (number between 0 and

1)">0.28</WetNaturalGasCarbonDioxideConcentration>

<WetNaturalGasMethaneConcentration fractionUOM="fraction (number between 0 and

1)">0.5</WetNaturalGasMethaneConcentration>

</GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<UniqueIdentifier>4</UniqueIdentifier>

<VentGasControlsUsed>Other / Multiple Vent Gas Controls</VentGasControlsUsed>

<VentGasControlsOtherDescription>Other</VentGasControlsOtherDescription>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">1.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">275.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1509.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">7.5</FlaringNitrousCarbonDioxideEquivalent>

<NaturalGasFlowRate flowUOM="MMscfd">4.3</NaturalGasFlowRate>

<AbsorbentCirculationPumpType>Natural gas pneumatic</AbsorbentCirculationPumpType>

<IsStripperGasUsed>No</IsStripperGasUsed>

<IsFlashTankSeparatorUsed>Yes</IsFlashTankSeparatorUsed>

<AbsorbentType>Ethylene glycol (EG)</AbsorbentType>

<GlycolDehydratorOperatingHours timeUOM="Hours">4</GlycolDehydratorOperatingHours>

<WetNaturalGasTemperature tempUOM="Fahrenheit">45</WetNaturalGasTemperature>

<WetNaturalGasPressure pressureUOM="psig">0.3</WetNaturalGasPressure>

<WetNaturalGasCarbonDioxideConcentration fractionUOM="fraction (number between 0 and

1)">0.3</WetNaturalGasCarbonDioxideConcentration>

<WetNaturalGasMethaneConcentration fractionUOM="fraction (number between 0 and

1)">0.5</WetNaturalGasMethaneConcentration>

</GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

<UniqueIdentifier>5</UniqueIdentifier>

<VentGasControlsUsed>No Vent Controls</VentGasControlsUsed>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">1.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric Tons">275.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1509.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">7.5</FlaringNitrousCarbonDioxideEquivalent>

<NaturalGasFlowRate flowUOM="MMscfd">4.3</NaturalGasFlowRate>

<AbsorbentCirculationPumpType>Air pneumatic</AbsorbentCirculationPumpType>

<IsStripperGasUsed>Yes</IsStripperGasUsed>

<IsFlashTankSeparatorUsed>No</IsFlashTankSeparatorUsed>

<AbsorbentType>Ethylene glycol (EG)</AbsorbentType

><GlycolDehydratorOperatingHours timeUOM="Hours">4</GlycolDehydratorOperatingHours>

<WetNaturalGasTemperature tempUOM="Fahrenheit">45</WetNaturalGasTemperature>

<WetNaturalGasPressure pressureUOM="psig">0.3</WetNaturalGasPressure>

<WetNaturalGasCarbonDioxideConcentration fractionUOM="fraction (number between 0 and

1)">0.2</WetNaturalGasCarbonDioxideConcentration>

<WetNaturalGasMethaneConcentration fractionUOM="fraction (number between 0 and

1)">0.5</WetNaturalGasMethaneConcentration>

</GlycolDehydratorsMoreThanSpecifiedValueRowDetails>

</GlycolDehydratorsMoreThanSpecifiedValueDetails>

</DehydratorsDetails>

<WellVentingDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">9.6</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">8965.8</TotalMethaneCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">8975.4</TotalCarbonDioxideEquivalent>

<DidFacilityHaveWellVenting>Yes</DidFacilityHaveWellVenting>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<WellVentingCalculationMethodOneDetails>

<WellVentingCalculationMethodOneRowDetails>

<SubBasinIdentifier>535 - ALBANY, WY (1) - Oil</SubBasinIdentifier>

<TubingDiameterAndPressure><1 inch, <25 psig</TubingDiameterAndPressure>

<NumberOfWells>1</NumberOfWells>

<NumberOfPlungerLifts>1</NumberOfPlungerLifts>

<NumberOfUnloadingsVented>9</NumberOfUnloadingsVented>

<DidWellHavePlungerLift>No</DidWellHavePlungerLift>

<CasingPressure pressureUOM="psia">100</CasingPressure>

<InternalCasingDiameter heightUOM="Inches">4</InternalCasingDiameter>

<WellDepth heightUOM="Feet">500</WellDepth>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">0.2</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">203.7</TotalMethaneCarbonDioxideEquivalent>

<AverageWellVentingFlowRate massUOM="Metric Tons">8.3</AverageWellVentingFlowRate>

</WellVentingCalculationMethodOneRowDetails>

<WellVentingCalculationMethodOneRowDetails>

<SubBasinIdentifier>535 - CARBON, WY (7) - High permeability gas</SubBasinIdentifier>

<TubingDiameterAndPressure><1 inch, >25 psig and <60 psig</TubingDiameterAndPressure>

<NumberOfWells>2</NumberOfWells>

<NumberOfPlungerLifts>2</NumberOfPlungerLifts>

<NumberOfUnloadingsVented>10</NumberOfUnloadingsVented>

<DidWellHavePlungerLift>Yes</DidWellHavePlungerLift>

<TubingPressure pressureUOM="psia">50</TubingPressure>

<InternalTubingDiameter heightUOM="Inches">1</InternalTubingDiameter>

<WellDepth heightUOM="Feet">500</WellDepth>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">4.4</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">4075.5</TotalMethaneCarbonDioxideEquivalent>

<AverageWellVentingFlowRate massUOM="Metric Tons">8.3</AverageWellVentingFlowRate>

</WellVentingCalculationMethodOneRowDetails>

<WellVentingCalculationMethodOneRowDetails>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

<TubingDiameterAndPressure><1 inch, >60 psig and <110 psig</TubingDiameterAndPressure>

<NumberOfWells>1</NumberOfWells>

<NumberOfPlungerLifts>1</NumberOfPlungerLifts>

<NumberOfUnloadingsVented>11</NumberOfUnloadingsVented>

<DidWellHavePlungerLift>No</DidWellHavePlungerLift>

<CasingPressure pressureUOM="psia">100</CasingPressure>

<InternalCasingDiameter heightUOM="Inches">4</InternalCasingDiameter>

<WellDepth heightUOM="Feet">500</WellDepth>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">0.2</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">203.7</TotalMethaneCarbonDioxideEquivalent>

<AverageWellVentingFlowRate massUOM="Metric Tons">8.3</AverageWellVentingFlowRate>

</WellVentingCalculationMethodOneRowDetails>

</WellVentingCalculationMethodOneDetails>

<WellVentingCalculationMethodTwoDetails>

<WellVentingCalculationMethodTwoRowDetails>

<SubBasinIdentifier>535 - ROUTT, CO (107) - Coal seam</SubBasinIdentifier>

<NumberOfWellsVentedWithoutPlungerLifts>1</NumberOfWellsVentedWithoutPlungerLifts>

<AverageInternalCasingDiameter heightUOM="Inches">4</AverageInternalCasingDiameter>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">4.4</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">4075.5</TotalMethaneCarbonDioxideEquivalent>

<CumulativeNumberOfUnloadingsVented>20</CumulativeNumberOfUnloadingsVented>

</WellVentingCalculationMethodTwoRowDetails>

</WellVentingCalculationMethodTwoDetails>

<WellVentingCalculationMethodThreeDetails>

<WellVentingCalculationMethodThreeRowDetails>

<SubBasinIdentifier>535 - SUBLETTE, WY (35) - Other tight reservoir rock</SubBasinIdentifier>

<NumberOfWellsVentedWithPlungerLifts>1</NumberOfWellsVentedWithPlungerLifts>

<NumberOfPlungerLifts>1</NumberOfPlungerLifts>

<AverageInternalTubingDiameter heightUOM="Inches">4</AverageInternalTubingDiameter>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">0.2</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">203.7</TotalMethaneCarbonDioxideEquivalent>

<CumulativeNumberOfUnloadingsVented>30</CumulativeNumberOfUnloadingsVented>

</WellVentingCalculationMethodThreeRowDetails>

<WellVentingCalculationMethodThreeRowDetails>

<SubBasinIdentifier>535 - SWEETWATER, WY (37) - Other tight reservoir rock</SubBasinIdentifier>

<NumberOfWellsVentedWithPlungerLifts>1</NumberOfWellsVentedWithPlungerLifts>

<NumberOfPlungerLifts>1</NumberOfPlungerLifts>

<AverageInternalTubingDiameter heightUOM="Inches">4</AverageInternalTubingDiameter>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">0.2</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">203.7</TotalMethaneCarbonDioxideEquivalent>

<CumulativeNumberOfUnloadingsVented>30</CumulativeNumberOfUnloadingsVented>

</WellVentingCalculationMethodThreeRowDetails>

</WellVentingCalculationMethodThreeDetails>

</WellVentingDetails>

<WellCompletionsWorkoversDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">376576.5</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">2262420.3</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">200.1</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">2639196.9</TotalCarbonDioxideEquivalent>

<DidFacilityHaveCompletionsWithHydraulic>Yes</DidFacilityHaveCompletionsWithHydraulic>

<DidFacilityHaveCompletionsWithoutHydraulic>Yes</DidFacilityHaveCompletionsWithoutHydraulic>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<WellCompletionsWorkoversWithHydraulicFracturingDetails>

<WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - ALBANY, WY (1) - Oil</SubBasinIdentifier>

<WellType>Vertical</WellType>

<CompletionsEmissionsEquationUsed>Eq. W-10A and Estimated Flow Rates (based Eq. W-11A or Eq. W-

11B)</CompletionsEmissionsEquationUsed>

<WorkoversEmissionsEquationUsed>Eq. W-10A and Estimated Flow Rates (based Eq. W-11A or Eq. W-

11B)</WorkoversEmissionsEquationUsed>

<TotalCompletions>1</TotalCompletions>

<TotalFlareGasWorkovers>1</TotalFlareGasWorkovers

><TotalVentGasWorkovers>1</TotalVentGasWorkovers>

<CompletionsVentingCarbonDioxideEmissions massUOM="Metric

Tons">710.6</CompletionsVentingCarbonDioxideEmissions>

<CompletionsVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">655005.7</CompletionsVentingMethaneCarbonDioxideEquivalent>

<CompletionsFlaringCarbonDioxideEmissions massUOM="Metric

Tons">110583.2</CompletionsFlaringCarbonDioxideEmissions>

<CompletionsFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">13634.9</CompletionsFlaringMethaneCarbonDioxideEquivalent>

<CompletionsFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">58.9</CompletionsFlaringNitrousCarbonDioxideEquivalent>

<WorkoversVentingCarbonDioxideEmissions massUOM="Metric Tons">90.9</WorkoversVentingCarbonDioxideEmissions>

<WorkoversVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">83746.3</WorkoversVentingMethaneCarbonDioxideEquivalent>

<WorkoversFlaringCarbonDioxideEmissions massUOM="Metric

Tons">14138.8</WorkoversFlaringCarbonDioxideEmissions>

<WorkoversFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">1743.3</WorkoversFlaringMethaneCarbonDioxideEquivalent>

<WorkoversFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">7.6</WorkoversFlaringNitrousCarbonDioxideEquivalent>

<SeparatingCompletions>86</SeparatingCompletions>

<SeparatingWorkovers>75</SeparatingWorkovers>

<TwoYearDelayIndicator>Yes</TwoYearDelayIndicator>

<CompletionsGasRecoveredToSales>85</CompletionsGasRecoveredToSales>

<WorkoversGasRecoveredToSales>123.5</WorkoversGasRecoveredToSales>

</WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - CARBON, WY (7) - High permeability gas</SubBasinIdentifier>

<WellType>Vertical</WellType>

<CompletionsEmissionsEquationUsed>Eq. W-10A and Measured Flow Rates</CompletionsEmissionsEquationUsed>

<WorkoversEmissionsEquationUsed>Eq. W-10A and Measured Flow Rates</WorkoversEmissionsEquationUsed>

<TotalCompletions>1</TotalCompletions>

<TotalFlareGasWorkovers>1</TotalFlareGasWorkovers>

<TotalVentGasWorkovers>1</TotalVentGasWorkovers>

<CompletionsVentingCarbonDioxideEmissions massUOM="Metric

Tons">710.6</CompletionsVentingCarbonDioxideEmissions>

<CompletionsVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">655005.7</CompletionsVentingMethaneCarbonDioxideEquivalent>

<CompletionsFlaringCarbonDioxideEmissions massUOM="Metric

Tons">110583.2</CompletionsFlaringCarbonDioxideEmissions>

<CompletionsFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">13634.9</CompletionsFlaringMethaneCarbonDioxideEquivalent>

<CompletionsFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">58.9</CompletionsFlaringNitrousCarbonDioxideEquivalent>

<WorkoversVentingCarbonDioxideEmissions massUOM="Metric Tons">90.9</WorkoversVentingCarbonDioxideEmissions>

<WorkoversVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">83746.3</WorkoversVentingMethaneCarbonDioxideEquivalent>

<WorkoversFlaringCarbonDioxideEmissions massUOM="Metric

Tons">14138.8</WorkoversFlaringCarbonDioxideEmissions>

<WorkoversFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">1743.3</WorkoversFlaringMethaneCarbonDioxideEquivalent>

<WorkoversFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">7.6</WorkoversFlaringNitrousCarbonDioxideEquivalent>

<SeparatingCompletions>23</SeparatingCompletions>

<SeparatingWorkovers>230</SeparatingWorkovers>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>1</APIWellNumbers>

<CompletionBackflowFlowRate flowUOM="cubic feet per hour">3.3</CompletionBackflowFlowRate>

<WorkoverBackflowFlowRate flowUOM="cubic feet per hour">11.2</WorkoverBackflowFlowRate>

<CompletionsNumberOfBackflowDays timeUOM="Days">45</CompletionsNumberOfBackflowDays>

<WorkoversNumberOfBackflowDays timeUOM="Days">24</WorkoversNumberOfBackflowDays>

<CompletionsGasRecoveredToSales>23</CompletionsGasRecoveredToSales>

<WorkoversGasRecoveredToSales>456.3</WorkoversGasRecoveredToSales>

</WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

<WellType>Horizontal</WellType>

<CompletionsEmissionsEquationUsed>Equation W-10B</CompletionsEmissionsEquationUsed>

<WorkoversEmissionsEquationUsed>Equation W-10B</WorkoversEmissionsEquationUsed>

<TotalCompletions>1</TotalCompletions>

<TotalFlareGasWorkovers>1</TotalFlareGasWorkovers>

<TotalVentGasWorkovers>1</TotalVentGasWorkovers>

<CompletionsVentingCarbonDioxideEmissions massUOM="Metric

Tons">710.6</CompletionsVentingCarbonDioxideEmissions>

<CompletionsVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">655005.7</CompletionsVentingMethaneCarbonDioxideEquivalent>

<CompletionsFlaringCarbonDioxideEmissions massUOM="Metric

Tons">110583.2</CompletionsFlaringCarbonDioxideEmissions>

<CompletionsFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">13634.9</CompletionsFlaringMethaneCarbonDioxideEquivalent>

<CompletionsFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">58.9</CompletionsFlaringNitrousCarbonDioxideEquivalent>

<WorkoversVentingCarbonDioxideEmissions massUOM="Metric Tons">90.9</WorkoversVentingCarbonDioxideEmissions>

<WorkoversVentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">83746.3</WorkoversVentingMethaneCarbonDioxideEquivalent>

<WorkoversFlaringCarbonDioxideEmissions massUOM="Metric

Tons">14138.8</WorkoversFlaringCarbonDioxideEmissions>

<WorkoversFlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">1743.3</WorkoversFlaringMethaneCarbonDioxideEquivalent>

<WorkoversFlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">7.6</WorkoversFlaringNitrousCarbonDioxideEquivalent>

<SeparatingCompletions>50</SeparatingCompletions>

<SeparatingWorkovers>145</SeparatingWorkovers>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>1</APIWellNumbers>

<CompletionBackflowFlowRate flowUOM="cubic feet per hour">3.3</CompletionBackflowFlowRate>

<WorkoverBackflowFlowRate flowUOM="cubic feet per hour">13.5</WorkoverBackflowFlowRate>

<CompletionsNumberOfBackflowDays timeUOM="Days">67</CompletionsNumberOfBackflowDays>

<WorkoversNumberOfBackflowDays timeUOM="Days">99</WorkoversNumberOfBackflowDays>

<CompletionsGasRecoveredToSales>100</CompletionsGasRecoveredToSales>

<WorkoversGasRecoveredToSales>200.8</WorkoversGasRecoveredToSales>

</WellCompletionsWorkoversWithHydraulicFracturingRowDetails>

</WellCompletionsWorkoversWithHydraulicFracturingDetails>

<WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

<WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - ROUTT, CO (107) - Coal seam</SubBasinIdentifier>

<TotalGasVentingDays>1</TotalGasVentingDays>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">0.2</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">9.6</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">1.8</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.3</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.2</FlaringNitrousCarbonDioxideEquivalent>

<NumberOfCompletions>198</NumberOfCompletions>

<NumberOfWorkoversThatFlareOrVentGas>123</NumberOfWorkoversThatFlareOrVentGas>

</WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - SUBLETTE, WY (35) - Other tight reservoir rock</SubBasinIdentifier>

<TotalGasVentingDays>1</TotalGasVentingDays>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">0.2</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">9.6</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">1.8</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.3</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.2</FlaringNitrousCarbonDioxideEquivalent>

<NumberOfCompletions>200</NumberOfCompletions>

<NumberOfWorkoversThatFlareOrVentGas>200</NumberOfWorkoversThatFlareOrVentGas>

</WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

<SubBasinIdentifier>535 - SWEETWATER, WY (37) - Other tight reservoir rock</SubBasinIdentifier>

<TotalGasVentingDays>1</TotalGasVentingDays>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">0.2</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">9.6</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">1.8</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.3</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.2</FlaringNitrousCarbonDioxideEquivalent>

<NumberOfCompletions>175</NumberOfCompletions>

<NumberOfWorkoversThatFlareOrVentGas>145</NumberOfWorkoversThatFlareOrVentGas>

</WellCompletionsWorkoversWithoutHydraulicFracturingRowDetails>

</WellCompletionsWorkoversWithoutHydraulicFracturingDetails>

</WellCompletionsWorkoversDetails>

<AtmosphericTanksDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">963180.1</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">1401058.2</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">300.7</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">2364539.0</TotalCarbonDioxideEquivalent>

<DidFacilityHaveGasFromProducedOil>Yes</DidFacilityHaveGasFromProducedOil>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<CalculationMethodFiveTotalVolumeOfOilProduction volUOM="barrels per

year">1737874</CalculationMethodFiveTotalVolumeOfOilProduction>

<CalculationMethodFiveProductionFractionWithVaporRecovery fractionUOM="fraction (number between 0 and

1)">0.5</CalculationMethodFiveProductionFractionWithVaporRecovery>

<CalculationMethodFiveProductionFractionWithFlaring fractionUOM="fraction (number between 0 and

1)">0.5</CalculationMethodFiveProductionFractionWithFlaring>

<CalculationMethodFiveTanksOnWellpads>10</CalculationMethodFiveTanksOnWellpads>

<ImproperlyFunctioningDumpValueWellheadSeparatorCount>1</ImproperlyFunctioningDumpValueWellheadSeparatorCount>

<AtmosphericTanksCalculationMethodOneOrTwoDetails>

<AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

<SubBasinIdentifier>535 - ALBANY, WY (1) - Oil</SubBasinIdentifier>

<CalculationMethodology>Calculation Methodology 1</CalculationMethodology>

<NumberOfWellHeadSeparators>10</NumberOfWellHeadSeparators>

<AverageSeparatorTemperature tempUOM="Fahrenheit">40</AverageSeparatorTemperature>

<AveragePressure pressureUOM="psi">4</AveragePressure>

<AverageAPIGravity densityUOM="degrees">15</AverageAPIGravity>

<HydrocarbonTanks>10</HydrocarbonTanks>

<StockTanks>10</StockTanks>

<TanksWithVaporRecovery>5</TanksWithVaporRecovery>

<TanksWithFlaring>5</TanksWithFlaring>

<TanksWithEmissionControl>10</TanksWithEmissionControl>

<MinimumFlashMethaneConcentration fractionUOM="volumetric

fraction">0</MinimumFlashMethaneConcentration>

<MaximumFlashMethaneConcentration fractionUOM="volumetric

fraction">0.82</MaximumFlashMethaneConcentration>

<MinimumFlashGasCarbonDioxideConcentration fractionUOM="volumetric

fraction">0</MinimumFlashGasCarbonDioxideConcentration>

<MaximumFlashGasCarbonDioxideConcentration fractionUOM="volumetric

fraction">0.93</MaximumFlashGasCarbonDioxideConcentration>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">8116.3</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">271301.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">234351.9</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">5426.1</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">74.8</FlaringNitrousCarbonDioxideEquivalent>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>45</APIWellNumbers>

<TotalVolumeOfOil volUOM="barrels per year">305.3</TotalVolumeOfOil>

<AnnualCarbonDioxideRecovered massUOM="Metric Tons">600.2</AnnualCarbonDioxideRecovered>

<AnnualMethaneRecovered massUOM="Metric Tons">254.02</AnnualMethaneRecovered>

</AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

<AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

<SubBasinIdentifier>535 - CARBON, WY (7) - High permeability gas</SubBasinIdentifier>

<CalculationMethodology>Calculation Methodology 2</CalculationMethodology>

<NumberOfWellHeadSeparators>10</NumberOfWellHeadSeparators>

<AverageSeparatorTemperature tempUOM="Fahrenheit">40</AverageSeparatorTemperature>

<AveragePressure pressureUOM="psi">4</AveragePressure>

<AverageAPIGravity densityUOM="degrees">15</AverageAPIGravity>

<HydrocarbonTanks>10</HydrocarbonTanks>

<StockTanks>10</StockTanks>

<TanksWithVaporRecovery>5</TanksWithVaporRecovery>

<TanksWithFlaring>5</TanksWithFlaring>

<TanksWithEmissionControl>10</TanksWithEmissionControl>

<MinimumFlashMethaneConcentration fractionUOM="volumetric

fraction">0</MinimumFlashMethaneConcentration>

<MaximumFlashMethaneConcentration fractionUOM="volumetric

fraction">0.82</MaximumFlashMethaneConcentration>

<MinimumFlashGasCarbonDioxideConcentration fractionUOM="volumetric

fraction">0</MinimumFlashGasCarbonDioxideConcentration>

<MaximumFlashGasCarbonDioxideConcentration fractionUOM="volumetric

fraction">0.93</MaximumFlashGasCarbonDioxideConcentration>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">8116.3</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">271301.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">234351.9</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">5426.1</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">74.8</FlaringNitrousCarbonDioxideEquivalent>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>45</APIWellNumbers>

<TotalVolumeOfOil volUOM="barrels per year">800.6</TotalVolumeOfOil>

<AnnualCarbonDioxideRecovered massUOM="Metric Tons">457.89</AnnualCarbonDioxideRecovered>

<AnnualMethaneRecovered massUOM="Metric Tons">896.2</AnnualMethaneRecovered>

</AtmosphericTanksCalculationMethodOneOrTwoSubBasinRowDetails>

</AtmosphericTanksCalculationMethodOneOrTwoDetails>

<AtmosphericTanksCalculationMethodThreeOrFourDetails>

<AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

<CalculationMethodology>Calculation Methodology 3</CalculationMethodology>

<WellsSendingOilToTanks>10</WellsSendingOilToTanks>

<WellsSendingOilToSeparators>10</WellsSendingOilToSeparators>

<MinimumAPIGravity densityUOM="degrees">15</MinimumAPIGravity>

<MaximumAPIGravity densityUOM="degrees">68</MaximumAPIGravity>

<TanksOnWellpads>10</TanksOnWellpads>

<TanksOnWellpadsWithVaporRecovery>1</TanksOnWellpadsWithVaporRecovery>

<TanksOnWellpadsWithFlaring>1</TanksOnWellpadsWithFlaring>

<TanksOffWellpadsWithVaporRecovery>1</TanksOffWellpadsWithVaporRecovery>

<TanksOffWellpadsWithFlaring>7</TanksOffWellpadsWithFlaring>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">8116.3</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">271301.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">226235.6</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">5426.1</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">74.8</FlaringNitrousCarbonDioxideEquivalent>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>345</APIWellNumbers>

<TotalVolumeOfSalesOil volUOM="barrels per year">452.3</TotalVolumeOfSalesOil>

<AnnualCarbonDioxideRecovered massUOM="Metric Tons">785.2</AnnualCarbonDioxideRecovered>

<AnnualMethaneRecovered massUOM="Metric Tons">1002.3</AnnualMethaneRecovered>

</AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

<AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

<CalculationMethodology>Calculation Methodology 4</CalculationMethodology>

<WellsSendingOilToTanks>10</WellsSendingOilToTanks>

<WellsSendingOilToSeparators>10</WellsSendingOilToSeparators>

<MinimumAPIGravity densityUOM="degrees">15</MinimumAPIGravity>

<MaximumAPIGravity densityUOM="degrees">68</MaximumAPIGravity>

<TanksOnWellpads>10</TanksOnWellpads>

<TanksOnWellpadsWithVaporRecovery>1</TanksOnWellpadsWithVaporRecovery>

<TanksOnWellpadsWithFlaring>1</TanksOnWellpadsWithFlaring>

<TanksOffWellpadsWithVaporRecovery>1</TanksOffWellpadsWithVaporRecovery>

<TanksOffWellpadsWithFlaring>7</TanksOffWellpadsWithFlaring>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">8116.3</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">271301.1</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">226235.6</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">5426.1</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">74.8</FlaringNitrousCarbonDioxideEquivalent>

<TwoYearDelayIndicator>No</TwoYearDelayIndicator>

<APIWellNumbers>2234</APIWellNumbers>

<TotalVolumeOfSalesOil volUOM="barrels per year">452.3</TotalVolumeOfSalesOil>

<AnnualCarbonDioxideRecovered massUOM="Metric Tons">785.2</AnnualCarbonDioxideRecovered>

<AnnualMethaneRecovered massUOM="Metric Tons">1002.3</AnnualMethaneRecovered>

</AtmosphericTanksCalculationMethodThreeOrFourSubBasinRowDetails>

</AtmosphericTanksCalculationMethodThreeOrFourDetails>

<AtmosphericTanksCalculationMethodFiveDetails>

<AtmosphericTanksCalculationMethodFiveSubBasinRowDetails>

<SubBasinIdentifier>535 - SUBLETTE, WY (35) - Other tight reservoir rock</SubBasinIdentifier>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">266.5</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">12837.3</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">865.6</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">266.5</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">1.5</FlaringNitrousCarbonDioxideEquivalent>

<NumberOfWellheadSeparators>203</NumberOfWellheadSeparators>

<NumberOfWellsWithoutWellheadSeparators>153</NumberOfWellsWithoutWellheadSeparators>

<AnnualCarbonDioxideRecovered massUOM="Metric Tons">789.63</AnnualCarbonDioxideRecovered>

<AnnualMethaneRecovered massUOM="Metric Tons">1002.96</AnnualMethaneRecovered>

</AtmosphericTanksCalculationMethodFiveSubBasinRowDetails>

</AtmosphericTanksCalculationMethodFiveDetails>

<AtmosphericTanksWellheadSeparatorDumpValveDetails>

<AtmosphericTanksWellheadSeparatorDumpValveSubBasinRowDetails>

<SubBasinIdentifier>535 - SWEETWATER, WY (37) - Other tight reservoir rock</SubBasinIdentifier>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">8407.8</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">281045.6</VentingMethaneCarbonDioxideEquivalent>

</AtmosphericTanksWellheadSeparatorDumpValveSubBasinRowDetails>

</AtmosphericTanksWellheadSeparatorDumpValveDetails>

</AtmosphericTanksDetails>

<WellTestingDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">200.0</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">200.0</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">100.0</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">500.0</TotalCarbonDioxideEquivalent>

<DidFacilityPerformWellTesting>Yes</DidFacilityPerformWellTesting>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<WellTestingBasinsDetails>

<WellTestingBasinsRowDetails>

<BasinIdentifier>535 - Green River Basin</BasinIdentifier>

<NumberOfWellsTested>10</NumberOfWellsTested>

<AverageGasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">1.5</AverageGasToOilRatio>

<AverageNumberOfDaysTested>40</AverageNumberOfDaysTested>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">100.0</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">100.0</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">100.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">100.0</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric

Tons">100.0</FlaringNitrousCarbonDioxideEquivalent>

</WellTestingBasinsRowDetails>

</WellTestingBasinsDetails>

</WellTestingDetails>

<AssociatedGasVentingFlaringDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">0.0</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalCarbonDioxideEquivalent>

<DidFacilityHaveGasVenting>Yes</DidFacilityHaveGasVenting>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<AssociatedGasVentingFlaringBasinsDetails>

<AssociatedGasVentingFlaringBasinsRowDetails>

<BasinIdentifier>535 - Green River Basin</BasinIdentifier>

<NumberOfWellsVenting>1</NumberOfWellsVenting>

<NumberOfWellsFlaring>0</NumberOfWellsFlaring>

<AverageGasToOilRatio fractionUOM="cubic feet of gas per barrel of oil">0</AverageGasToOilRatio>

<VentingCarbonDioxideEmissions massUOM="Metric Tons">0.0</VentingCarbonDioxideEmissions>

<VentingMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</VentingMethaneCarbonDioxideEquivalent>

<FlaringCarbonDioxideEmissions massUOM="Metric Tons">0.0</FlaringCarbonDioxideEmissions>

<FlaringMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">0.0</FlaringMethaneCarbonDioxideEquivalent>

<FlaringNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</FlaringNitrousCarbonDioxideEquivalent>

</AssociatedGasVentingFlaringBasinsRowDetails>

</AssociatedGasVentingFlaringBasinsDetails>

</AssociatedGasVentingFlaringDetails>

<FlareStacksDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">16411.1</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">6048.6</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">159.8</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">22619.5</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveFlareStacks>Yes</DoesFacilityHaveFlareStacks>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<UniqueFlareStacksDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>1</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>Yes</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>Yes</WereCemsUsed>

<CombustedAndUncombustedCarbonDioxide massUOM="Metric

Tons">0.5</CombustedAndUncombustedCarbonDioxide>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>2</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>Yes</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>Yes</WereCemsUsed>

<CombustedAndUncombustedCarbonDioxide massUOM="Metric

Tons">0.5</CombustedAndUncombustedCarbonDioxide>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>3</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>4</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>5</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>6</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>7</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>8</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>9</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>10</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>11</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>12</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>13</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>14</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>15</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>16</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>No</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

<UniqueFlareStacksRowDetails>

<UniqueIdentifier>17</UniqueIdentifier>

<DoesFlareHaveContinuousFlowMonitor>Yes</DoesFlareHaveContinuousFlowMonitor>

<PercentOfGasSent percentUOM="Number (between 0 and 100)">10</PercentOfGasSent>

<DoesFlareHaveContinuousGasAnalyzer>No</DoesFlareHaveContinuousGasAnalyzer>

<WereCemsUsed>No</WereCemsUsed>

<UncombustedCarbonDioxideEmissions massUOM="Metric Tons">964.8</UncombustedCarbonDioxideEmissions>

<UncombustedMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">355.8</UncombustedMethaneCarbonDioxideEquivalent>

<CombustedCarbonDioxideEmissions massUOM="Metric Tons">0.5</CombustedCarbonDioxideEmissions>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">9.4</NitrousCarbonDioxideEquivalent>

<GasSentToFlare volUOM="Cubic Feet">8.6</GasSentToFlare>

<FlareCombustionEfficiency>0.75</FlareCombustionEfficiency>

</UniqueFlareStacksRowDetails>

</UniqueFlareStacksDetails>

</FlareStacksDetails>

<CentrifugalCompressorsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">20000.0</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">21000.0</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">41000.0</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveCentrifugalCompressors>Yes</DoesFacilityHaveCentrifugalCompressors>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<TotalAnnualCompressorCarbonDioxideEmissions massUOM="Metric

Tons">20000.0</TotalAnnualCompressorCarbonDioxideEmissions>

<TotalAnnualCompressorMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">21000.0</TotalAnnualCompressorMethaneCarbonDioxideEquivalent>

<CompressorCount>323</CompressorCount>

</CentrifugalCompressorsDetails>

<ReciprocatingCompressorsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">11.1</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">19.1</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">30.2</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveReciprocatingCompressors>Yes</DoesFacilityHaveReciprocatingCompressors>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<TotalAnnualCompressorCarbonDioxideEmissions massUOM="Metric

Tons">11.1</TotalAnnualCompressorCarbonDioxideEmissions>

<TotalAnnualCompressorMethaneCarbonDioxideEquivalent massUOM="Metric

Tons">19.1</TotalAnnualCompressorMethaneCarbonDioxideEquivalent>

<CompressorCount>456</CompressorCount>

</ReciprocatingCompressorsDetails>

<OtherEmissionsFromEquipmentLeaksDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">9048.0</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">4000.7</TotalMethaneCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">13048.7</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveEquipmentLeaks>Yes</DoesFacilityHaveEquipmentLeaks>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<EstimatingEmissionsFoundInLeakSurveyDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Compressor Components, Gas Service - Valve</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Compressor Components, Gas Service - Connector</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Compressor Components, Gas Service - Open-Ended Line</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Compressor Components, Gas Service - Pressure Relief Valve</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Non-compressor Components, Gas Service - Valve</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Non-compressor Components, Gas Service - Connector</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Non-compressor Components, Gas Service - Open-Ended Line</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

<EstimatingEmissionsFoundInLeakSurveyRowDetails>

<ComponentType>Non-compressor Components, Gas Service - Pressure Relief Valve</ComponentType>

<FirstCompleteSurveyDate>2010-01-01</FirstCompleteSurveyDate>

<FirstSurveyLeaksFound>1</FirstSurveyLeaksFound>

<SecondCompleteSurveyDate>2010-03-01</SecondCompleteSurveyDate>

<SecondSurveyLeaksFound>2</SecondSurveyLeaksFound>

<ThirdCompleteSurveyDate>2010-06-01</ThirdCompleteSurveyDate>

<ThirdSurveyLeaksFound>3</ThirdSurveyLeaksFound>

<FourthCompleteSurveyDate>2010-10-01</FourthCompleteSurveyDate>

<FourthSurveyLeaksFound>4</FourthSurveyLeaksFound>

<CarbonDioxideEmissions massUOM="Metric Tons">6.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">500.0</MethaneCarbonDioxideEquivalent>

</EstimatingEmissionsFoundInLeakSurveyRowDetails>

</EstimatingEmissionsFoundInLeakSurveyDetails>

<EstimatingEmissionsUsingPopulationCountsDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, light crude service - flange</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.2</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>7</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails><EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, light crude service - connector</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.5</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>8</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, light crude service - open-ended line</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>30</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, light crude service - other</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>21</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, heavy crude service - valve</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>24</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, heavy crude service - flange</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>77</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, heavy crude service - connector</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>56</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, heavy crude service - open-ended line</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>30</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

<EstimatingEmissionsUsingPopulationCountsRowDetails>

<ComponentType>Onshore, heavy crude service - other</ComponentType>

<CarbonDioxideEmissions massUOM="Metric Tons">1000.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<TotalCountForEachLeakSourceType>15</TotalCountForEachLeakSourceType>

</EstimatingEmissionsUsingPopulationCountsRowDetails>

</EstimatingEmissionsUsingPopulationCountsDetails>

<ComponentCountMethodologyOneDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Wellheads</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Separators</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Meters/piping</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Compressors</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>In-line heaters</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Dehydrators</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Heater-treater</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

<ComponentCountMethodologyOneRowDetails>

<MajorEquipmentType>Header</MajorEquipmentType>

<TotalCountForMajorEquipmentType>200</TotalCountForMajorEquipmentType>

</ComponentCountMethodologyOneRowDetails>

</ComponentCountMethodologyOneDetails>

</OtherEmissionsFromEquipmentLeaksDetails>

<EnhancedOilRecoveryInjectionPumpBlowdownDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">4.2</TotalCarbonDioxideEmissions>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">4.2</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveEorInjectionPumpBlowdown>Yes</DoesFacilityHaveEorInjectionPumpBlowdown>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>Yes</SubstituteDataIndicator>

<UniqueInjectionPumpBlowdownsDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>1</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.23</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>2</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>3</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.2</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>4</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>5</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.2</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>6</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>230</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>7</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>75</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.2</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>8</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>75</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>9</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>75</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">45</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>10</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>75</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">45</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>11</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>75</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">23</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>12</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>15</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">102</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>13</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>15</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.33</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>14</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>15</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">0.5</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>15</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>89</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>16</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>89</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">200</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>17</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume

><NumberOfBlowdowns>200</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">78</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>18</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>231</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">89</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>19</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>520</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">123</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>20</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>111</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

<UniqueInjectionPumpBlowdownsRowDetails>

<UniqueIdentifier>21</UniqueIdentifier>

<PumpCapacity volUOM="barrels per day">10</PumpCapacity>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CriticalPhaseGasVolume volUOM="Cubic Meters">20.3</CriticalPhaseGasVolume>

<NumberOfBlowdowns>111</NumberOfBlowdowns>

<CriticalPhaseGasDensity densityUOM="kg/ft3">100</CriticalPhaseGasDensity>

</UniqueInjectionPumpBlowdownsRowDetails>

</UniqueInjectionPumpBlowdownsDetails>

</EnhancedOilRecoveryInjectionPumpBlowdownDetails>

<EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">9.6</TotalCarbonDioxideEmissions>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">9.6</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveEorLiquidsDissolved>Yes</DoesFacilityHaveEorLiquidsDissolved>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>No</SubstituteDataIndicator>

<HydrocarbonLiquidsDissolvedDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - ALBANY, WY (1) - Oil</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">500</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - CARBON, WY (7) - High permeability gas</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">4.4</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">500</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - MOFFAT, CO (81) - Shale gas</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">300</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - ROUTT, CO (107) - Coal seam</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">4.4</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">500</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - SUBLETTE, WY (35) - Other tight reservoir rock</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">300</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<HydrocarbonLiquidsDissolvedSubBasinRowDetails>

<SubBasinIdentifier>535 - SWEETWATER, WY (37) - Other tight reservoir rock</SubBasinIdentifier>

<CarbonDioxideEmissions massUOM="Metric Tons">0.2</CarbonDioxideEmissions>

<CrudeOilProduced volUOM="barrels per year">300</CrudeOilProduced>

<CarbonDioxideRetained fractionUOM="metric tons per barrel">56</CarbonDioxideRetained>

</HydrocarbonLiquidsDissolvedSubBasinRowDetails>

</HydrocarbonLiquidsDissolvedDetails>

</EnhancedOilRecoveryHydrocarbonLiquidsDissolvedDetails>

<CombustionEmissionsDetails>

<TotalCarbonDioxideEmissions massUOM="Metric Tons">30835.2</TotalCarbonDioxideEmissions>

<TotalMethaneCarbonDioxideEquivalent massUOM="Metric Tons">19.1</TotalMethaneCarbonDioxideEquivalent>

<TotalNitrousCarbonDioxideEquivalent massUOM="Metric Tons">534.6</TotalNitrousCarbonDioxideEquivalent>

<TotalCarbonDioxideEquivalent massUOM="Metric Tons">31388.9</TotalCarbonDioxideEquivalent>

<DoesFacilityHaveCombustionEmissions>Yes</DoesFacilityHaveCombustionEmissions>

<BAMMIndicator>Yes</BAMMIndicator>

<BAMMDescription>description</BAMMDescription>

<SubstituteDataIndicator>No</SubstituteDataIndicator>

<ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Well drilling and completion equipment</UnitType>

<NumberOfUnits>1</NumberOfUnits>

</ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Workover equipment</UnitType>

<NumberOfUnits>2</NumberOfUnits>

</ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Natural gas dehydrators</UnitType>

<NumberOfUnits>1</NumberOfUnits>

</ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Steam boilers</UnitType>

<NumberOfUnits>2</NumberOfUnits>

</ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Process heaters</UnitType>

<NumberOfUnits>1</NumberOfUnits>

</ExternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

</ExternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Well drilling and completion equipment</UnitType>

<NumberOfUnits>1</NumberOfUnits>

<CarbonDioxideEmissions massUOM="Metric Tons">184.3</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.1</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</NitrousCarbonDioxideEquivalent>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Workover equipment</UnitType>

<NumberOfUnits>2</NumberOfUnits>

<CarbonDioxideEmissions massUOM="Metric Tons">2885.7</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">2.3</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">6.8</NitrousCarbonDioxideEquivalent>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Natural gas dehydrators</UnitType>

<NumberOfUnits>1</NumberOfUnits>

<CarbonDioxideEmissions massUOM="Metric Tons">4444.8</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">4.0</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">11.8</NitrousCarbonDioxideEquivalent>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Steam boilers</UnitType>

<NumberOfUnits>2</NumberOfUnits>

<CarbonDioxideEmissions massUOM="Metric Tons">7902.8</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">3.1</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">248.8</NitrousCarbonDioxideEquivalent>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Process heaters</UnitType>

<NumberOfUnits>1</NumberOfUnits>

<CarbonDioxideEmissions massUOM="Metric Tons">0.0</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.0</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</NitrousCarbonDioxideEquivalent>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ExternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Well drilling and completion equipment</UnitType>

<NumberOfUnits>1</NumberOfUnits>

</InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Workover equipment</UnitType>

<NumberOfUnits>2</NumberOfUnits>

</InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

<UnitType>Electrical generators</UnitType>

<NumberOfUnits>1</NumberOfUnits>

</InternalFuelHeatCapacityLessThanSpecifiedValueRowDetails>

</InternalFuelHeatCapacityLessThanSpecifiedValueDetails>

<InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Well drilling and completion equipment</UnitType>

<CarbonDioxideEmissions massUOM="Metric Tons">184.3</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">0.1</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">0.0</NitrousCarbonDioxideEquivalent>

</InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Workover equipment</UnitType>

<CarbonDioxideEmissions massUOM="Metric Tons">2885.7</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">2.3</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">6.8</NitrousCarbonDioxideEquivalent>

</InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Electrical generators</UnitType>

<CarbonDioxideEmissions massUOM="Metric Tons">4444.8</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">4.0</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">11.8</NitrousCarbonDioxideEquivalent>

</InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

<UnitType>Natural gas compressors</UnitType>

<CarbonDioxideEmissions massUOM="Metric Tons">7902.8</CarbonDioxideEmissions>

<MethaneCarbonDioxideEquivalent massUOM="Metric Tons">3.1</MethaneCarbonDioxideEquivalent>

<NitrousCarbonDioxideEquivalent massUOM="Metric Tons">248.8</NitrousCarbonDioxideEquivalent>

</InternalFuelHeatCapacityGreaterThanSpecifiedValueRowDetails>

</InternalFuelHeatCapacityGreaterThanSpecifiedValueDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Field gas</FuelType>

<CumulativeFuelCombustedVolume>23</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>mton</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Distillate Fuel Oil No. 2</FuelType>

<CumulativeFuelCombustedVolume>23</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>lb</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Propane</FuelType>

<CumulativeFuelCombustedVolume>100</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>gal</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Other Oil (>401 deg F)</FuelType>

<CumulativeFuelCombustedVolume>100</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>bbl</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Anthracite</FuelType>

<CumulativeFuelCombustedVolume>400</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>kg</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Peat</FuelType>

<CumulativeFuelCombustedVolume>700</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>bbl</FuelVolumeUnits>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

</ExternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

<InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Distillate Fuel Oil No. 2</FuelType>

<CumulativeFuelCombustedVolume>78</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>bbl</FuelVolumeUnits>

</InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

<FuelType>Asphalt and Road Oil</FuelType>

<CumulativeFuelCombustedVolume>200</CumulativeFuelCombustedVolume>

<FuelVolumeUnits>gal</FuelVolumeUnits>

</InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueRowDetails>

</InternalFuelTypeHeatCapacityGreaterThanSpecifiedValueDetails>

</CombustionEmissionsDetails>

</SubPartWReportingFormsDetails>

</SubPartW>

</SubPartInformation>

</FacilitySiteDetails>

<StartDate>2014-01-01</StartDate>

<EndDate>2014-12-31</EndDate>

<DateTimeReportGenerated>2015-01-20T18:53:52</DateTimeReportGenerated>

<SubpartBAMMDetails>

<SubpartName>AR4</SubpartName>

<WasBAMMUsed>N</WasBAMMUsed>

</SubpartBAMMDetails>

</FacilitySiteInformation>

</GHG>